IT Programming

Year 2025 - 2026

Panasiuk, Jesse

NSCC  V0.0.10

Contents

Introduction to Class Notes	17

PROG1700 Logic and Programming	18

Python	18

Variables	18

If Statements	18

While Loops	19

Functions	20

Lists	23

For Loops	26

In	27

Dictionaries	27

NETW1027 Introduction to Networking and Security	30

Network Components	30

Host Rules	30

Peer-to-peer	31

End Devices	32

Intermediary Devices	33

Network Media	34

Network Representations and Topologies	35

Network Representations	35

Topology Diagrams	37

Common Types of Networks	38

Networks of Many Sizes	38

LANs and WANs	39

The Internet	42

Intranet and Extranets	43

Internet Connections	44

Internet Access Technologies	44

Home and Small Office Internet Connections	44

Business Internet Connections	46

The Converging Network	47

Reliable Networks	48

Network Architecture	48

Fault Tolerance	49

Scalability	49

Quality of Service	50

Network Security	51

Network Trends	52

Recent Trends	52

Bring Your Own Device (BYOD)	53

Online Collaboration	53

Video Communications	53

Cloud Computing	54

Technology Trends in the Home	55

Powerline Networking	55

Wireless Broadband	56

Network Security	57

Security Threats	57

Security Solutions	58

Basic Switch and End Device Configuration	59

Cisco IOS Access	59

Operating Systems	59

GUI	59

Purpose of an OS	60

Access Methods	60

Terminal Emulation Program	61

IOS Navigation	61

Primary Command Modes	61

Configuration Mode and Subconfiguration Modes	62

Navigation Between IOS Modes	63

Note About Syntax Checker Activities	64

The Command Structure	64

Basic IOS Command Structure	64

IOS Command Syntax Check	65

IOS Help Features	66

Hot Keys and Shortcuts	66

Basic Device Configuration	67

Device Name	67

Password Guidelines	69

Configure Passwords	70

Encrypt Passwords	71

Banner Messages	72

Save Configurations	73

Configuration Files	73

Alter the Running Configuration	74

Capture Configuration to a Text File	74

Ports and Address	76

IP Addresses	76

Interfaces and Ports	78

Configure IP Addressing	80

Manual IP Address Configuration for End Devices	80

Automatic IP address Configuration for End Devices	81

Switch Virtual Interface Configuration	82

Protocols and Models	83

The Rules	83

Communications Fundamentals	83

Communication Protocols	83

Rule Establishment	83

Network Protocol Requirements	84

Message Encoding	84

Message Formatting and Encapsulation	85

Message Size	85

Message Timing	86

Message Delivery Options	86

Protocols	87

Network Protocols Overview	87

Network Protocol Functions	87

Protocol Interaction	88

Protocol Suites	89

Network Protocol Suites	89

Evolution of Protocol Suites	90

TCP/IP Protocol Example	91

TCP/IP Protocol Suite	92

Standard Organizations	95

Open Standards	95

Internet Standards	96

Electronic And Communications Standards	97

Reference Model	97

The Benefits of Using a Layered Model	97

The OSI Reference Model	98

The TCP/IP Protocol Model	99

OSI and TCP/IP Comparison	100

Data Encapsulation	101

Segmenting Messages	101

Sequencing	101

Protocol Data Unites	102

Encapsulation Example	103

De-encapsulation Example	103

Data Access	103

Addresses	103

Layer 3 Logical Address	103

Devices on the Same Network	104

Role of the Data Link Layer Adress – Same IP Network	105

Role of the Network Layer Addresses	106

Role of the Data Link Layer Addresses – Different IP Networks	107

Data Link Addresses	108

Purpose of the Physical Layer	110

The Physical Connection	110

The Physical Layer	112

Physical Layer Characteristics	112

Physical Layer Standards	112

Physical Components	113

Encoding	113

Signaling	114

Encoding	114

Bandwidth Terminology	115

Copper Cabling	116

Characteristics of Copper Cabling	116

Types of Copper Cabling	118

Unshielded Twisted-pair (UTP)	118

Shielded Twisted-pair (STP)	119

Coaxial Cable	120

UTP Cabling	121

Properties of UTP Cabling	121

UTP Cabling Standards and Connectors	122

Straight-through and Crossover UTP	125

Fiber-Optic Cabling	126

Properties of Fiber-Optic Cabling	126

Types of Fiber Media	126

Fiber-Optic Cabling Usage	127

Fiber-Optic Connectors	128

Fiber Patch Cords	130

Fiber versus Copper	130

Wireless Media	131

Properties of Wireless Media	131

Types of Wireless Media	132

Wireless LAN	133

Binary Number Systems	133

Binary and IPv4 Addresses	133

Binary Positional Notation	135

Convert Binary to Decimal	137

Decimal to Binary Conversion	139

IPv4 Addresses	140

Hexadecimal Number Systems	142

Hexadecimal and IPv6 Addresses	142

Decimal to Hexadecimal Conversions	144

Hexadecimal to Decimal Conversions	144

Data Link Layer	144

Purpose of the Data Link Layer	144

Topologies	146

Half-duplex Communications	147

Full-duplex Communications	148

Controlled access	148

Data Link Frame	148

Frame Field	148

Layer 2 Addresses	149

Ethernet Switching	149

Ethernet Frames	149

Ethernet Encapsulation	149

Data Link Sublayers	151

MAC Sublayer	152

Ethernet Frame Fields	153

Ethernet MAC Address	154

MAC Address and Hexadecimal	154

Ethernet MAC Address	156

Frame Processing	157

Unicast MAC address	157

Broadcast MAC Address	158

Multicast MAC address	158

The MAC Address Table	160

Switch Fundamentals	160

Switch Learning and Forwarding	160

Filtering Frames	162

Switch Speeds and Forwarding	164

Frame Forwarding Methods on Cisco Switches	164

Cut-Through Switching	164

Memory Buffering on Switches	165

Duplex and Speed Settings	166

Auto-MDIX	167

Network Layer	168

Network Layer Characteristics	168

The Network Layer	168

IP Encapsulation	170

Characteristics of IP	171

Connectionless	171

Best Effort	172

Media Independent	173

IPv4 Packet	174

IPv4 Packet Header	174

IPv4 Packet Header Fields	175

IPv6 Packet	176

Limitations of IPv4	176

IPv6 Overview	177

IPv4 Packet Header Fields in the IPv6 Packet Header	177

IPv6 Packet Header	179

How a Host Routes	181

Host Forwarding Decision	181

Default Gateway	181

A Host Routes to the Default Gateway	182

Host Routing Tables	182

Introduction to Routing	183

Router Packet Forwarding Decision	183

IP Router Routing Table	185

Static Routing	186

Dynamic Routing	187

Introduction to an IPv4 Routing Table	189

Address Resolution	191

MAC and IP	191

Destination on Same Network	191

Destination on Remote Network	192

Address Resolution Protocol (ARP)	194

ARP Overview	194

ARP Functions	195

ARP Request	195

ARP Operation – ARP Reply	196

ARP Role in Remote Communications	196

Removing Entries from an ARP Tables	197

ARP Tables on Networking Devices	197

ARP Issues – ARP Broadcasts and ARP Spoofing	198

IPv6 Neighbor Discovery	199

IPv6 Neighbor Discovery Messages	199

IPv6 Neighbor Discovery – Address Resolution	200

Basic Router Configuration	201

Configure Initial Router Settings	201

Basic Router Configuration Steps	201

Basic Router Configuration Example	202

Configure Interfaces	204

Configure Router Interfaces	204

Configure Router Interfaces Example	204

Verify Interface Configuration	206

Configuration Verification Commands	206

Configure the Default Gateway	207

Default Gateway on a Host	207

Default Gateway on a Switch	209

IPv4 Addressing	211

IPv4 Address Structure	211

Network and Host Portions	211

The Subnet Mask	211

The Prefix Length	213

Determining the Network – Logical AND	214

Network, Host and Broadcast Addresses	215

IPv4 Unicast, Broadcast and Multicast	218

Unicast	218

Broadcast	218

Multicast	219

Types of IPv4 Address	220

Public and Private IPv4 Addresses	220

Routing to the Internet	221

Special Use IPV4 Addresses	222

Legacy Classful Address	223

Assignment of IP Addresses	224

Network Segmentation	225

Broadcast Domains and Segmentation	225

Problems with Large Broadcast Domains	226

Reasons for Segmenting Networks	227

Subnet an IPv4 Network	228

Subnet on an Octet Boundary	228

Subnet within an Octet Boundary	230

Subnet a Slash 16 and a Slash 8 Prefix	232

Create Subnet with a Slash 16 prefix	232

Create 100 Subnets with a Slash 16 prefix	234

Create 1000 Subnets with a Slash 8 prefix	236

Subnet to Meet Requirements	239

Subnet Private versus Public IPv4 Address Space	239

Minimize Unused Host IPv4 Addresses and Maximize Subnets	242

Efficient IPv4 Subnetting	243

VLSM	246

IPV4 Address Conservation	246

VLSM	248

VLSM Topology Address Assignment	251

Structured Design	252

IPv4 Network Address Planning	252

Device Address Assignment	253

IPv6 Addressing	254

IPv4 Issues	254

Need for IPv6	254

IPv4 and IPv6 Coexistence	256

IPv6 Address Representation	256

IPv6 Addressing Formats	256

Rule 1 – Omit Leading Zeros	258

Rule 2 – Double Colon	259

IPv6 Address Types	260

Unicast, Multicast, Anycast	260

IPv6 Prefix Length	260

Types of IPv6 Unicast Addresses	261

A Note About the Unique Local Addresses	261

IPv6 GUA	262

IPv6 GUA Structure	263

IPv6 LLA	264

GUA and LLA Static Configuration	265

Static GUA Configuration on a Router	265

Static GUA Configuration on a Windows Host	266

Static Configuration of a Link-Local Unicast Address	268

Dynamic Addressing for IPv6 GUAs	269

RS and RA Messages	269

Method 1 – SLAAC	270

Method 2 – SLAAC and Stateless DHCPv6	271

Method 3 – Stateful DHCPv6	271

EUI-64 Process Vs. Randomly Generated	272

EUI-64 Process	273

Randomly Generated Interface IDs	275

Dynamic Addressing for IPv6 LLAs	275

Dynamic LLAs	275

Dynamic LLAs on Windows	275

Dynamic LLAs on Cisco Routers	275

Verify IPv6 Address Configuration	275

OSYS1200 Introduction to Windows Administration	276

The Evolving Role of System Administrators in the Future of Computing	276

Introduction	276

The Impact of Cloud Computing	276

Automation and DevOps	276

Security and Risk Management	276

Emerging Technologies and Skill Requirements	276

Conclusion	277

Windows: Using System and Utilities	277

Settings Overview	277

Administrative Tools	277

Microsoft Management Console	278

Services	279

Command-Line Administration Tools	280

Installing and Configuring Apps	281

The Windows App Landscape	281

Managing Apps and Processes with Task Manager	281

Managing Startup Apps	281

Managing Local and Cloud Storage/Managing Disk and File Types	281

Disk Technology	281

Disk Partition Styles	282

Disk Types	282

Disk Management Tools	283

Adding a New Drive	283

Configuring Storage Spaces Fault Tolerance	283

User Accounts, Passwords and Credentials and User Management	284

User Profiles	284

The Default Profile	285

Mandatory Profile	285

Roaming Profile	285

Public Profile	286

Windows Security and Privacy	286

Managing Hardware Devices/Application Support	286

Application Environments	286

The Registry	286

Registry Structure: Registry Hives	287

Registry Editing Tools	287

Performance Power	288

Troubleshooting Windows Errors	288

Steps Recorder	288

Reliability Monitor	288

Event Viewer	289

System Recovery	290

Reset This PC	290

Windows Recovery Environment (WinRE)	291

Windows Terminal/PowerShell	291

Managing Windows PC in Enterprise	291

Active Directory	291

Active Directory Structure	291

Active Directory Partitions	292

Active Directory Sites and Replication	292

Active Directory and DNS	292

Joining a Domain	292

Time Synchronization	292

Offline Domain Join	292

Group Policy	293

Group Policy Inheritance	293

Group Policy Preferences	293

Multiple Local Policies	293

Controlling Device Installation	293

DBAS1007 Data Fundamentals	293

Databases	293

ACID Properties	294

Best Practices	294

Normalization	295

Normal Forms	295

Normalization Steps	295

ERDs and ERMs	298

Entity Relationship Modeling (ERM)	299

Entity Relationship Diagrams (ERD)	300

Key Differences	300

Components	300

Chen Notation	300

Three Types of Data Models	301

Conceptual Data Model	301

Logical Data Model	301

Physical Data Model	301

SQL	302

SQL Statements & Execution	302

SQL DDL	302

Logical Data Model	303

WEBD1000 Website Development	303

Web Development History	303

Introduction to Web Development	303

The Birth of the Web (1989 – 1991)	303

The Rise of Web Browsers (1993 – 1995)	303

The Browser Wars and Standards (1995 – 1998)	304

The Dot-Com Boom and Bust (1997 – 2001)	304

The Modern Web (2002 – Present)	304

Introduction to Usability	304

Principles of Good Usability	305

Principle of Contrast	305

Applying Contrast Effectively	305

Principle of Repetition	305

Applying Repetition Effectively	306

Principle of Alignment	306

Applying Alignment Effectively	306

Principle of Proximity	306

Applying Proximity Effectively	306

What is Web Accessibility?	307

Importance of Accessibility	307

Understanding WCAG (Web Content Accessibility Guidelines)	307

Accessible Design Components: Perceivable Content	308

Accessible Design Components: Operable Interface	308

Accessible Design Components: Understandable Content	308

Accessible Design Components: Robust Content	308

Tools and Resources for Testing Accessibility	309

Planning and Designing Websites	309

Understanding Project Requirements	309

Gathering Project Requirements	309

Analyzing Content Requirements	310

Designing Based on Criteria	310

Translating Requirements into Design	310

Designing for Content First	311

Balancing Aesthetics and Functionality	311

Importance of Wireframes in Web Design	311

Wireframing Process	312

Key Components of a Wireframe	312

Tools for Wireframing	312

Creating Web Structure Diagram	313

Steps to Create a Web Structure Diagram	313

Best Practices or Wireframing and Structure Diagrams	313

HTML	314

Understanding HTML Tags an Attributes	314

Understanding URLS	314

Creating Hyperlinks with HTML	315

Absolute Vs. Relative URLS	315

Adding Images to Web Pages	315

Using Special Characters in HTML	315

Introduction to CSS (Cascading Style Sheets)	316

Responsive Web Design	316

What is Responsive Web Design?	316

Why Responsive Design is Important.	316

Introduction to Media Queries	317

What are Media Queries?	317

Creating Fluid Layouts	317

Using Media Queries for Responsive Design	318

Responsive Images and Media	319

Fluid Typography	320

Responsive Font Sizing	320

Practical Example: Building a Responsive Layout	320

Introduction to Flexbox	321

Creating Layout with Flexbox	322

Using Float for Layouts	322

Frameworks	322

Summaries	323

PROG1700 Logic and Programming	323

NETW1027 Introduction to Networking and Security	323

Networking Summary 1.0	323

Networking Summary 2.0	323

Networking Summary 3.0	324

Networking Summary 4.0	325

Number System Summary	325

Data Link Layer Summary	326

Ethernet Switching Summary	326

Network Layer Summary	330

Address Resolution Summary	331

Basic Router Configuration Summary	332

OSYS1200 Introduction to Windows Administration	333

DBAS1007 Data Fundamentals	333

WEBD1000 Website Development	333

Glossary	334

PROG1700 Logic and Programming	334

NETW1027 Introduction to Networking and Security	334

OSYS1200 Introduction to Windows Administration	334

DBAS1007 Data Fundamentals	334

WEBD1000 Website Development	334

# Introduction to Class Notes

# PROG1700 Logic and Programming

## Python

### Variables

Python, by convention uses snake_case when naming variables as opposed to camelCase.

Python does not require a “;” at the end of each line, unlike other languages (GML, C++, etc)

Variables are used to store information.  When this variable is later called, whatever is stored within it, will be used in its place. Variables can take any name you give with (which the exception of some words used specifically for Python). Variables can be used to store Ints, floats, doubles, Booleans and even strings.

You can obtain the type of a variable by using “type(variable_name)” in a “print()” function.

You can also convert a variable by using “type(variable_name)”.

### If Statements

If statements are used as a conditional branch to make programs more “interesting” and interactable by having different outcomes based on different variables.

The proper formatting for an if statement would be:

If variable_name (==, >, <, <= or >=) value:

Note: An if statement is checking to see “if something is true or false” and that the statements are always followed by “:”

If statements always check against a Boolean value (True or False) and can have different components to it.  An elif (else if) statement may follow an if statement’s first condition, if another condition is to be checked.  An else statement may follow an if statement’s first condition, or any subsequent elif statements if we want to have something happen, if the original condition, or any of the previous conditions, are not met.

If statements can also check multiple conditionals by using “and” and “or” in their conditonals.

Unlike C++, if statements in Python do not use “{ }” (curly braces) to contain their results.  Instead, Python uses indentations.

i.e. if variable_name > 10 and variable_name < 25:

print(“See how it is indented? This means it is part of the if statement.”)

### While Loops

While loops are similar to If statements, in that they perform a block of code based on a Boolean conditional.  However, the difference is that while loops don’t have an elif or else component.  Instead, when a while loop terminates, if the Boolean conditional is not met, the program will loop back to the beginning of the while loop.

In addition to satisfying the Boolean conditional, you can also use the “break” command.  The “break” code will “break” you out of the loop, regardless of if the Boolean conditional of the while loop has been met.

### Functions

Functions are like little mini blocks of code.  They are good for separating chunks of code out of the “main body” of your script.  They are also a great way to DRY (Don’t Repeat Yourself) your code, as anything that must be repeated, can be placed in its own function, so that you only need to call/invoke the function, rather than the entire block of code.

If you look at the above figure, the get_user_input() is responsible for getting our input from the user.  Within it, it is asking the user to enter an integer and checking if it meets our requirements (being greater than 0).  Now, if we need to ask the user for multiple integers, or if we need to ask for it, again, later down the road, we can simply invoke it by typing get_user_input() instead of re-typing out, everything that is within the function. What’s more, is if we later decide to modify the code to ask for the input an additional time, we just add that “get_user_input()” line.

When creating a function, we use the keyword “def” followed by <function name> an end it with “():”.  The “()” is where we are going to pass in arguments, but we’ll discuss that later.

Just like If statements and while loops, everything indented below the “def <function name>()” is what will happen when we invoke the function.

Think of functions like elaborate variables:

If typing “x = 12” replaces x with 12 whenever you call x, then typing:

def call_this():

print(“This is a function”)

replaces “call_this()” with “print(“this is a function”) whenever you call, call_this().

Functions can be called by variables.

Consider the below:

def get_user_input():

print(“Please enter an integer”)

user_input = input()

return user_input

x = get_user_input()

See the “return user_input”?  That is “returning” whatever value is in “user_input” when we exit the function.  It is then being stored in the variable that invoked the function, which is x.  So now, x = user_input.

Now, if you wanted to get another input, you could simply add “y = get_user_input()” beneath “x = get_user_input()

Functions can also be passed “arguments” or “parameters”.

Consider the below:

def add(x, y):

sum = x + y:

print(“The sum is “ + str(sum))

See how add has an “x” and a “y” value?  Those are arguments that have been passed to it and will be used as part of it’s “function”.  How do we pass them?  By adding them to the “()” when we call it, like so:

add(4, 5)

This will call the function “add()” and pass it “4” and “5” as arguments.  It will store 4 in x and 5 in y.  When the function then goes through it’s code, it’s going to turn sum from “sum = x + y” into “sum = 4 + 5” and this, the print command will print “9”

So, how does this all come together?

Well, consider this:

def get_user_input():

print(“Please enter an integer:”)

user_input = input()

return user_input()

def add(a, b):

sum = a + b

return sum

x = get_user_input()

y = get_user_input()

z = add(x, y)

print(“The sum is “ + str(y))

Okay.  So, lets break this down.  First, we are defining “get_user_input()” and “add()”, just like we did previous examples.  Then, in the main body of our code, we are declaring two variables, “x” and “y”.  We are then using those variables to call “get_user_input” for each variable and storing the function’s return value into them.  Imagine we entered 4 and 5.  X will now equal 4 and Y will now equal 5.

Then we call add(a, b).

Now, this part is going to get confusing.  The reason I used “a” and “b” when defining the function “add” is because I wanted to demonstrate that these can be anything. We can call them “Will” and “Gary”.  Just because we are passing the values of “x” and “y” into add(a, b) doesn’t mean it has to use “x” and “y”.  So, when we call add(), we call it with “add(x, y)” (“x” and “y” being the values we want to pass it) the function will then do “a = x” and “b = y” in the background.

So, when we call add(x, y) it will turn “x” and “y” into “a” and “b”, add the values together (4 and 5, respectively, in our imagination) and then return the value of “sum” and store it into “y”.

When we then get to “print(“The sum is “ +str(y))” it will print “The sum is 9”

### Lists

Lists are variables that contain multiple entries of data.

Imagine if you wanted to gather student names and store those in variables.  You could have a variable for each and every student (student1 = mike, student2 = Alice, student3 = Todd) but that would be cumbersome.  That I where a List come in handy.

Lists are denoted by the Square Bracket ([]).

See the figure below for a look at using a variable for each student, against storing all names in a single variable:

So, if we’re storing the names in a list, like the above… how do we access the data within that list?  What if we wanted to get “Alice” from the list?

In our “A variable for every student” example, we could use student2 in reference of “Alice” but that wont work with our list. For this, we would reference the index of the list.

In Python (and most programming languages) lists are built on a 0-index.  This means that the first object within that list is in the “0 index” not the “1 index”.

With that in mind, if we wanted to get “Alice” from the list of student name, we would use the following:

student_names[1]

	Knowing that, we can think of the list as being this:

student_names[0] = “Mike”

student_names[1] = “Alice”

student_names[2] = “Todd”

What about modifying a list, once it has been created? You can do that using some built in commands.  You can also use assignment to alter a list object.

For assignment, we can simply use from the above.  Let’s say we wanted to change Alice’s name to Abigal.

student_name[1] = “Abigail

Regardless of what was in Index-1, it will now be “Abigail.”

The above will print the following to the terminal:

Regarding the built in commands you can use to alter a list, this would be “.pop” and “.append”

These are called Methods.  Essentially, functions that work on a particular variable.

temp1 = student_names.pop(1)

This will “pop out” Abigail from the list and store it into a variable named “temp1”

“.append” will “append,” or add, data to the end of the list.

student_names.append(temp1)

This will add/append Abigail to the end of the list.

Consider the figures below:

You can even have a “list of lists”
	

	These are called “2D Lists” (You can even go further and have a “list of lists of lists” and so on)

References the positions of these are just like references a traditional list.

Consider the following:

### For Loops

Like While Loops, For Loops are a way to iterate through a list (or dictionary, which we’ll get to) and “do something for each object in that loop.”  In most cases, a While Loop can be used, in place of a For Loop, so choosing which one to use will usually be up to you to decide.  But, because there are some cases where you will need to use a For Loop over a While Loop and vice versa (and that For Loops are common in the industry) it is good to have a solid understanding of how both loops work.  You can even use them together!

A For Loop comes in the following form:

For this to make a little more sense, I am going to use the list of student names we made and altered in the previous chapter.

While you can probably already guess that this will run the print function, “for each name in student names,” printing out each name, but let’s breakdown the why and the how it is doing this.

The “name” portion in “for name in student_names:” could have been anything.  It is just a temporary variable used to reference the specific place of the object we are iterating through, at an given time.  So, while it is helpful (and recommended) to give the variable a name that makes, somewhat, logical sense to make it easier to understand and follow, we could put anything here.  We could put “for x in student_names:”, or we could use “for volcanoes in student_names:” or even “for jsdhfskdjhfs in student_names:”

The “student_names” portion of “for name in student_names” is the object we want to iterate through.  It doesn’t have to be a variable.  As an example, if we wanted to do the following:

By turning “student_name” into a string, this is now going to iterate through the string.  For each letter in that string, it will then print that letter.

Notice how, even though the “name” portion of “for name in “student names” is still “name” yet it printed the letters.  This is, again, because the actual variable name doesn’t matter.  It is simply a way to reference what index of the object we are iterating through we are at, at any given time.

The “in” in “for name in “student names” is simply saying “for each index in this object.”

It is different from the “in” operand that is used to check if some value is found “inside” another object.

### In

“In” as an operand (not as part of a For Loop) is not unlike other operands we are already familiar with.  I.e., “+”, “-“, “and”, “or”.

In takes a value or object on the left an checks to see if it is “in” a value or object on the right.

This is going to print “True” because the value “1” is inside the list-object “[1, 2, 3]”

### Dictionaries

Dictionaries are like lists, but with a little more versatility.  Where the position of each value in a list is automatically given a numerical value to reference (student_names[0] = “Mike” is Index-0, student_names[1] = “Abigail” is Index-1 and student_names[2] = “Todd” is Index-2) with dictionaries, we can name those indexes.  These are call “keys.”

These “keys” must be unique.

Dictionaries are denoted with Curly Braces ({})

This is basically a list.  It is a list containing a string-“Mike” and an Integer-24, with the “indexes” of “Name” and “Age” instead of “0” and “1”, respectively.

We can reference these in a similar way to how we reference positions in a list.  The difference is, is that we will the “key” rather than the index.

We can also have a “list of dictionaries”.

This may seem a little overwhelming, but we just need to break it down to its component parts if we want to be able to reference the data inside of this structure.

If we wanted to get Abigail’s age, how would we do this? Let’s build the command, piece by piece.

Well, Abigail is within the second set of curly braces ( {} ), this means that her information is in the “second index of the outer-list” which would put her at Index-1

So, we start with:

However, this is going to reference the whole dictionary.  If put in a print() command, it will print the following:

And we only want her “Age.” How do we get this?

Well, remember that with dictionaries, the “key” is really just an “index” that we were able to name, inside of automatically assigning it a numerical value.  And just like in the 2D-list example, we can reference data “2 lists deep” by using double square brackets ( [][] ):

We can do the same with the list of dictionaries, by using double square brackets ( [][] ) however, we will pass it the index of the outer-list and the key of the inner-dictionary:
	

	So, you get Abigail’s age, we’d use the following:

Plugging this into a print() will display “26” in the terminal.

# NETW1027 Introduction to Networking and Security

## Network Components

### Host Rules

All computers that are connected to a network and participate, directly in network communication are classified as hosts.  Hots can be called “end devices.” Some hosts are called “clients.” However, the term hosts specifically refer to devices on the network that are assigned a number for communication purposes.  This number identifies the host within a particular network. This number is called the ”Internet Protocol (IP)” address.  An IP address identifies the host and the network to which the host is attached.

Servers are computers with software that allow them to provide information, like email or web pages, to other end devices on the network.  Each service requires separate server software.  For example, a server requires web server software to provide web services to the network.  A computer with server software can provide services simultaneously to many different clients.

As mentioned before, clients are a type of host.  Clients have software for requesting and displaying the information obtained from the server, as shown in the figure.

An example of client software is a web browser, like Chrome or Firefox.  A single computer can also run multiple types of client software.  For example, a user can check email and view a web page while instant messaging and listening to an audio stream.  The table lists three common types of software.

| Type | Description |
| --- | --- |
| Email | The email server runs email software. Clients use mail client software, such as Microsoft Outlook, to acces email on the server. |
| Web | The web server runs web server software. Clients use browser software, such as Windows Internet Explorer, to access web pages on the server. |
| File | The file server stores corporate and user files in a central locatin. The client devices access these files with client software such as the Windows File Explorer. |

### Peer-to-peer

Client and server software usually run on separate computers, but it is also possible for one computer to be used for both roles at the same time.  In small businesses and homes, many computers function as the servers and clients on the network.  This type of network is called a peer-to-peer network.

In the figure, the print sharing PC has a Universal Serial Bus (USB) connection to the printer and a network connection, using a network interface card (NIC), to the file share PC.

### End Devices

The network devices that people are most familiar with are end devices. To distinguish one end device from another, each end device on a network has an address.  When an end device initiates communication, it uses the address of the destination end device to specify where to deliver the message.

An end device is either the source or destination of a message transmitted over the network.

### Intermediary Devices

Intermediary devices connect the individual end devices to the network.  They can connect multiple individual networks to form an internetwork.  These intermediary devices provide connectivity and ensure that data flow across the network.

Intermediary devices use the destination end device address, in conjunction with information about the network interconnections, to determine the path the messages should take through the network.  Examples of the more common intermediary devices and a list of functions are shown in the figure.

Intermediary network devices perform some or all of these functions:

- Regenerate and retransmit communication signals.

- Maintain information about what pathways exist through the network and internetwork

- Notify other devices of errors and communication failures.

- Direct data along alternate pathways when there is a link failure.

- Classify and direct messages according to priorities.

- Permit or deny the flow of data, based on security settings.

Note:  Not shown above is a legacy ethernet hub.  An ethernet hub is also known as a multiport repeater.  Repeaters regenerate and retransmit communication signals.  Notice that all intermediary devices perform the function of a repeater.

### Network Media

Communication transmits across a network on media.  The media provides the channel over which the message travels from source to destination.

Modern networks primarily use three types of media to interconnect devices, as shown in the figure:

- Metal wires within cables

  - Data is encoded into electrical impulses.

- Glass or plastic fibers within cables (fiber-optic cable)

  - Data is encoded into pulses of light.

- Wireless transmission

  - Bata is encoded via modulation of specific frequencies of electromagnetic waves.

The four main criteria for choosing network media are these:

- What is the maximum distance that the media can successfully carry a signal?

- What is the environment in which the media will be installed?

- What is the amount of data and at what speed must it be transmitted?

- What is the cost of media and installation?

## Network Representations and Topologies

### Network Representations

Network architects an administrators must be able to show what their networks will look like  They need to be able to easily see which components connect to other components, where they will be located, and how they will be connected.  Diagrams of networks often use symbols, like those show in the figure, to represent the different devices and connections that make up a network.

A diagram provides an easy way to understand how devices connect in a large network.  This type of “picture” of a network is known as a topology diagram. The ability to recognize the logical representations of the physical networking components is critical to being able to visualize the organization and operation of a network.

In addition to these representations, specialized terminology is used to describe how each of these devices and media connect to each other:

- Network Interface Card (NIC)

  - A NIC physically connects the end device to the network.

- Physical Port

  - A connector or outlet on a networking device where the media connects to an end device on another networking device.

- Interface

  - Specialized ports on a networking device that connect to individual networks.  Because routers connect networks, the ports on a router are referred to as network interfaces.

Note: The terms port ad interface are often used interchangeably.

### Topology Diagrams

Topology diagrams are mandatory documentation for anyone working with a network.  They provide a visual map of how the network is connected.  There are two types of topology diagrams: physical and logical.

Physical Topology Diagrams

Physical topology diagrams illustrate the physical location of intermediary devices and cable installation, as shown in the figure.  You can see that the rooms in which these devices are locate are labeled in this physical topology.

Logical Topology Diagrams

Logical topology diagrams illustrate devices, ports and the addressing scheme of the network, as shown in the figure. You can see which end devices are connected to which intermediary devices and what media is being used.

## Common Types of Networks

### Networks of Many Sizes

Now that you are familiar with the components that make up networks and their representations in physical and logical topologies, you are ready to learn about the many different types of networks.

Networks come in all sizes.  They range from simple networks consisting of two computers, to networks connecting millions of devices.

Simple home networks let you share resources, such as printers, documents pictures, and music, among a few local end devices.

Small office and home office (SOHO) networks allow people to work from home, or a remote office.  Many self-employed workers use these types of networks to advertise and sell products, order supplies, and communicate with customers.

Businesses and large organizations use networks to provide consolidation, storage, and access to information on network servers. Networks provide email, instant messaging, and collaboration among employees.  Many organizations use their network’s connection to the internet to provide products and services to customers.

The internet is the largest network in existence.  In fact, the term internet means a “network of networks”.  It is a collection of interconnected private and public networks.

In small businesses and homes, many computers function as both servers and clients on the network.  This type of network is called a peer-to-peer network.

| Small Home Networks | Connect a few computers to each other and to the internet |
| --- | --- |
| Small Office and Home Office Networks | The SOHO network allows computers in a home office or a remote office to connect to a corporate network, or access centralized, shared resources. |
| Medium to Large Networks | Medium to large networks, such as those used by corporations and schools, can have many locations with hundreds or thousands of interconnected hosts. |
| World Wide Networks | The internet is a network of networks that connects hundreds of millions of computers world=wide. |

### LANs and WANs

Network infrastructures vary greatly in terms of:

  - Size of the area covered

  - Number of users connected

  - Number and types of service available

  - Area of responsibility

The two most common types of network infrastructures are Local Area Network (LANs) and Wide Area Networks(WANs).

A LAN is a network infrastructure the provides access to users and end devices in a small geographical area. A LAN is typically used in a department within an enterprise, a home or a small business network.

A WAN is a network infrastructure that provides access to other networks over a wide geographical area, which is typically owned and managed by a larger corporation or a telecommunications service provider.  The figure shows LANs connected to WANs.

LANS

A LAN is a network infrastructure that spans a small geographical area. LANs have specific characteristics.

  - LANs interconnect end devices in a limited area such as a home, school, office building or campus.

  - A LAN is usually administered by a single organization or individual. Administrative control is enforced at the network level and governs the security and access control policies.

  - LANs provide high-speed bandwidth to internal end devices and intermediary devices, as shown in the figure.

WANS

The figure shows a WAN which interconnects to LANs. A WAN is a network infrastructure that spans a wide geographical area. WANs are typically managed by a Service Provider (SPs) or Internet Service Provider (ISPs)

WANs have specific characteristics:

  - WANs interconnect LANs over wide geographical areas such as between cities, states, provinces, countries or continents.

  - WANs are usually administered by multiple service providers.

  - WANs typically provide lower speed links between LANs

### The Internet

The internet is a worldwide collection of interconnected networks (internetwork, or internet for short). The figure shows one way to view the internet as a collection of interconnected LANs and WANs.

Some of the LAN examples are connected to each other through a WAN connection.  WANs are then connected to each other.  WANs can connect through copper wires, fiber-optic cables and wireless transmissions (not shown).

The internet is not owned by an individual or group.  Ensuring effective communication across this diverse infrastructure requires the application of consistent and commonly recognized technologies and standards as well as the cooperation of many network administration agencies.  There are organization that were developed to help maintain the structure and standardization of internet protocol and processes.  These organizations include the Internet Engineering Task Force (ETF), Internet Corporation for Assigned Names and Numbers (ICANN), and the Internet Architecture Board (IAB) plus many others.

### Intranet and Extranets

There are two other terms which are like the term internet: intranet and extranet.

Intranet is a term often use to refer to a private connection of LANs and WANs that belong to an organization. An intranet is designed to be accessible only to the organization’s members, employees, or others with authorization.

An organization may use an extranet to provide secure and safe access to individuals who work for a different organization but require access to the organization’s data. Here are some examples of extranets:

  - A company this is providing access to outside suppliers and contractors.

  - A hospital that is providing a booking system to doctors so they can make appointments for their patients.

  - A local office of education that is providing budget and personnel information to the schools in its district.

The figure illustrates the levels of access that different groups have to a company intranet, a company extranet, and the internet.

## Internet Connections

### Internet Access Technologies

So, now you have a basic understanding of what makes up a network and the different types of networks.  But how do you actually connect users and organization to the internet?  As you may have guessed, there are many different ways to do this.

Home users, remote workers and small offices typically require a connection to an ISP to access the internet.  Connection options vary greatly between ISPs and geographical locations.  However, popular choices include broadband cable, broadband digital subscriber line (DSL), wireless WANs, and mobile services.

Organizations usually need access to other corporate sites as well as the internet.  Fast connections are required to support business services including IP phones, video conferencing and data center storage.  SPs offer business-class connection.  Popular business-class services include business DSL, leased lines, and Metro Ethernet.

### Home and Small Office Internet Connections

The figure below illustrates common connection options for small office and home office user.

Cable – Typically offered by cable television service providers, the internet data signal transmits on the same cable that delivers cable television.  It provides a high bandwidth, high availability and an always-o connection to the internet.

DSL – Digital Subscriber Lines also provide high bandwidth, high availability and an always-on connection to the internet.  DSL runs over the telephone line.  In general, small office and home office users connect using Asymmetrical DSL (ADSL) which means that the download speed is faster than the upload speed.

Cellular – Cellular internet access uses a cell phone network to connect.  Wherever you can get a cellular signal, you can get cellular internet access.  Performance is limited by the capabilities of the phone and the cell tower to which it is connected.

Satellite – The availability of satellite internet access is a benefit in those areas that would otherwise have no internet connectivity at all.  Satellite dishes require a clear line of sight to the satellite.

Dial-up Telephone – An inexpensive option that uses any phone line and a modem.  The low bandwidth provided by a dial-up modern connection is not sufficient for large data transfer, although it is useful for mobile access while traveling.

The choice of connection varies depending on geographical location and service provider availability.

### Business Internet Connections

Corporate connection option differ from home user options.  Businesses may require higher bandwidth, dedicated bandwidth and managed services.  Connection options that are available differ depending on the type of service providers located nearby.

The figure below illustrates common connection options for businesses.

Dedicate Leased Line – Leased lines are reserved circuits within the service provider’s network that connect geographically separated offices for private voice and/or data networking.  The circuits are rented at a monthly or yearly rate.

Metro Ethernet – This is sometimes known as Ethernet WAN.  In this module, we will refer to it as Metro Ethernet.  Metro ethernets extend LAN access technology into the WAN.

Business DSL – Business DSL is available in various formats.  A popular choice is Symmetric Digital Subscriber Lines (SDSL) which is similar to the consumer version of DSL but provides uploads and downloads at the same high speeds.

Satellite – Satellite service can provide a connection when a wired solution is not available.

The choice of connection varies depending on geographical location and service provider availability.

### The Converging Network

Traditional Separate Networks

Consider a school built thirty years ago  Back then, some classrooms were cabled for the data network, telephone network and video network for televisions.  These separate networks could not communicate with each other.  Each network used different technologies to carry the communication signal.  Each network had its own set of rules and standards to ensure successful communication.  Multiple services ran on multiple networks.

Converged Network

Toay, the separate data, telephone, and video networks converge.  Unlike dedicated networks, converged networks are capable of delivering data, voice, and video between many different types of devices over the same network infrastructure.  This network infrastructure uses the same set of rules, agreements and implementation standards.  Converged data networks can carry multiple services on one network.

## Reliable Networks

### Network Architecture

Have you ever been busy working online, only to have “the internet go down”? as you know by now, the internet did not go down, you just lost your connection to it.  It is very frustrating.  With so many people in the world relying on network access to work and learn, it is imperative that networks are reliable.  In this context, reliability means more than your connection to the internet.  This topic focuses on the four aspects of network reliability.

The role of the network has changed from a data-only network to a system that enables the connections of people, devices, and information in a media-rich, converged network environment.  For networks to function efficiently and grow in this type of environment, the network must be built upon a standard network architecture.

Networks also support a wide range of application and services.  They must operate over many different types of cables and devices, which make up the physical infrastructure.  The term network architecture, in this context, refers to the technologies that support the infrastructure and the programmed services an rules, or protocols, the move data across the network.

As networks evolve, we have learned that there are four basic characteristics that network architects must address to meet user expectations:

- Fault Tolerance

- Scalability

- Quality of Service (QoS)

- Security

### Fault Tolerance

A fault tolerant network is one that limits the umber of affected devices during a failure.  It is built to allow quick recovery when such a failure occurs.  These networks depend on multiple paths between the source and destination of a message.  If one path fails, the messages are instantly sent over a different link.  Having multiple paths to a destination is known as redundancy.

Implementing a packet-switched network is one way that reliable networks provide redundancy. Packet switching splits traffic into packets that are routed over a shared network.  A single message, such as an email or a video stream, is broken into multiple message blocks, called packets.  Each packet has the necessary addressing information of the source and destination of the message.  The routers within the network switch the packets based on the condition of the network at that moment.  This means that all the packets in a single message could take very different paths to the same destination.  In the figure, the user is unaware and unaffected by the router that is dynamically changing the route when a link fails.

### Scalability

A scalable network expands quickly to support new users and applications.  It does this without degrading the performance of services that are being accessed by existing users.  The figure shows how a new network is easily added to an existing network.  These networks are scalable because the designers follow accepted standards and protocols.  This let software and hardware vendor focus on improving products and services without having to design a new set of rules for operating within the network.

### Quality of Service

Quality of Service (QoS) is an increasing requirement of networks today.  New applications available to users over networks, such as voice and live video transmissions, create higher expectation for the quality of the delivered services.  Have you ever tried to watch a video with constant breaks and pauses?  As data, voice and video content continue to converge onto the same network, QoS becomes a primary mechanism for managing congestion and ensuring reliable delivery of content to all users.

Congestion occurs when the demand for bandwidth exceeds the amount available.  Network bandwidth is measured in the umber of bits that can be transmitted in a single second, or bits per second (bps).  When simultaneous communications are attempted across the network, the demand for network bandwidth can exceed its availability, creating network congestion

When the volume of traffic is greater than what can be transported across the network, devices will hold the packets in memory until resources become available to transmit them.  In the figure, one user us requesting a web page, and another is on a phone call. With a QoS policy in place, the router can manage the flow of data and voice traffic, giving priority to voice communications if the network experiences congestion.  The focus of QoS is to prioritize time-sensitive traffic.  The type of traffic, not the content of traffic, is what is important.

### Network Security

The network infrastructure, service, and the data contained on network-attached devices are crucial personal and business assets.  Network administrators must address two types of network security concerns: network infrastructure security and information security.

Securing the network infrastructure includes physically securing devices that provide network connectivity and preventing unauthorized access to the management software that resides on them, as shown in the figure.

Network administrators must also protect the information contained within the packets being transmitted over the network, and the information stored on network attached devices.  In order to achieve the goals of network security, here are three primary requirements.

  - Confidentiality – Data confidentiality means that only the intended and authorized recipients can access and read data

  - Integrity – Data integrity assures users that the information has not been altered in transmission, rom origin to destination.

  - Availability – Data availability assures users of timely and reliable access to data services for authorized users.

## Network Trends

### Recent Trends

You know a lot about networks now, what they are made of, how they connect us and what is needed to keep them reliable.  But networks, like everything else, continue to change.  There are a few trends in networking that you, as a NetAcad student, should know about.

As new technologies and end-user devices come to market, business and consumers must continue to adjust to this ever-changing environment.  There are several networking trends that affect organizations and consumers:

- Bring Your Own Device (BYOD)

- Online collaboration

- Video Communications

- Cloud Computing

### Bring Your Own Device (BYOD)

The concept of any device, for any content, in any manner, is a major global trend that requires significant changes to the way we use devices and safely connect them to networks.  This is called Bring Your Own Device (BYOD).

BYOD enables end users with the freedom to use personal tools to access information and communicate across a business or campus network.  With the growth of consumer devices, and the related drop in cost, employees and students may have advanced computing and networking devices for personal use.  These include laptops, notebooks, tablets, smart phones and e-readers.  These may be purchased by the company or school, purchased by the individual, or both.

BYOD mean any device, with any ownership, used anywhere.

### Online Collaboration

Individuals want to connect to the network, not only for access to data applications, but also to collaborate with one another.  Collaboration is defined as “the act of working with another or others on a join project.” Collaboration tools, like Cisco WebEx, give employees, students, teachers, customers and partners a way to instantly connect, interact and achieve their objectives.

Collaboration is a critical and strategic priority in education.  Students need to collaborate to assist each other in learning, to develop the team skills used in the workforce, and to work together on team-based projects.

Cisco WebEx Teams is a multifunctional collaboration tool that lets you send instant messages to one or more people, post images, and post videos and links.  Each team ‘space’ maintains a history of everything that is posted there.

### Video Communications

Another facet of networking that is critical to the communication and collaboration effort is video.  Video is used for communications, collaboration, and entertainment.  Video calls are made to and from anyone with an internet connection, regardless of where they are located.

Video conferencing is a powerful tool for communicating with others, both locally and globally.  Video is becoming a critical requirement for effective collaboration as organization extend across geographic and cultural boundaries.

### Cloud Computing

Cloud computing is one of the ways that we access and store data.  Cloud computing allows us to store personal files, even backup an entire drive on servers over the internet.  Applications such as word processing and photo editing can be accessed using the cloud.

For businesses, Cloud computing extends the capabilities of IT without requiring investment in now infrastructure, training new personnel, or licensing new software.  These services are available on-demand and delivered economically to any device that is anywhere in the world without compromising security or function.

Cloud computing is possible because of data centers.  Data centers are facilities used to house computer systems and associated components.  A data center can occupy one room of a building, one or more floors, or an entire warehouse-sized building.  Data centers are typically very expensive to build and maintain.  For this reason, only large organization use privately built data centers to house their data and provide services to users.  Smaller organization that cannot afford to maintain their own private data center can reduce the overall cost of ownership by leasing server and storage services from a larger data center organization in the cloud.

For security, reliability, and fault tolerance, cloud providers often store data in distributed data centers.  Instead of storing all the data of a person or organization in one data center, it is stored in multiple data enters in different locations.

There are four primary types of clouds: Public clouds, Private clouds, Hybrid clouds, and Community clouds.

| Public clouds | Cloud-based applications and services offered in a public cloud are made available to the general population. Services may be free or are offered on a pay-per-use model, such as paying for online storage. The public cloud uses the internet to provide services. |
| --- | --- |
| Private clouds | Cloud-base application and services offered in a private cloud are intended for a specific organization or entity, such as a government. A private cloud can be set up using the organization’s private network, through this can be expensive to build and maintain. A private cloud can also be managed by an outside organization with strict access security. |
| Hybrid clouds | A hybrid cloud is made up of two or more clouds (example: part private, part public), where each part remains a distinct object, but both are connected using a single architecture. Individuals on a hybrid cloud would be able to have degrees of access to various services based on user access rights. |
| Community clouds | A community cloud is created for exclusive use by specific entities or organizations. The differences between public clouds and community clouds are the functional needs that have been customize for the community. Or example, healthcare organizations must remain compliant with policies and laws (e.g HIPAA) that require special authentication and confidentiality. Community clouds are use by multiple organizations that have similar needs and concerns. Community clouds are similar to a public cloud environment, but with set levels of security, privacy and even regulatory compliance of a private cloud. |

### Technology Trends in the Home

Networking trends are not only affecting the way we communicate at work and at school, but also changing many aspects of the home.  The newest home trends include “smart home technology.”

Smart home technology integrates into every-day appliances, which can then connect with other devices to make the appliances more ‘smart’ or automated.  For example, you could prepare food and place it in the over for cooking prior to leaving the house for the day.  You program your smart over for the food you want it to cook.  It would also be connected to your “calendar of events” so that it would determine what time you should be available to eat and adjust start times and length of cooking accordingly.  It could even adjust cooking times and temperatures based on changes in schedule.  Additionally, a smart phone or tablet connection lets you connect to the over directly, to make any desired adjustments.  When the food is ready, the oven sends an alert message to you (or someone you specify) that the food is done and warming.

Smart home technology is currently being developed for all rooms with a house.  Smart home technology will become more common as home networking and high-speed internet technology expands.

### Powerline Networking

Powerline networking for home networks uses existing electrical wiring to connect devices, as shown in the figure.

Using a standard powerline adaptor, devices can connect to the LAN wherever there is an electrical outlet.  No data cables need to be installed, and there is little to no additional electricity used.  Using the same wiring that delivers electricity, powerline networking sends information by sending data on certain frequencies.

Powerline networking is especially useful when wireless access points cannot reach all the devices in the home. Powerline networking is not a substitute for dedicate cabling in data networks.  However, it is an alternative when data network cables or wireless communications are not possible or effective.

### Wireless Broadband

In many areas where cable an DSL are not available, wireless may be used to connect to the internet.

Wireless Internet Service Provider

A Wireless Internet Service Provider (WISP) is an ISP that connects subscribers to a designated access point or hot spot using similar wireless technologies found in home wireless local area networks (WLANs).  WISPs are more commonly found in rural environments where DSL or cable services are not available.

Although a separate transmission tower may be installed for the antenna, typically the antenna is attached to an existing elevated structure, such as a water tower or radio tower.  A small dish or antenna is installed on the subscriber’s roof in range of the WISP transmitter.  The subscriber’s access unit is connected to the wire network inside the home.  From the perspective of the home user, the setup is not much different than DSL or cable service.  The main difference is that the connection from the home to the ISP is wireless instead of a physical cable.

Wireless Broadband Service

This solution uses the same cellular technology as a smart phone.  An antenna is installed outside the house providing either wireless or wired connectivity for devices in the home.  In many areas, home wireless broadband is competing directly with DSL and cable services.

## Network Security

### Security Threats

You have, no doubt, heard or read news stories about a company network being breached, giving threat actors access to the personal information of thousands of customers.  For this reason, network security is always going to be a top priority of administrators.

Network security is an integral part of computer networking, regardless of whether the network is in a home with a single connection to the internet or is a corporation with thousands of users.  Network security must consider the environment, as well as the tools and requirements of the network.  It must be able to secure data while allowing for the quality of service that users expect o the network.

Securing a network involves protocols, technologies, devices, tools and techniques in order to protect data and mitigate threats.  Threat vectors may be external or internal.  Many external network security threats today originate from the internet.

There are several common external threats to networks:

  - Viruses, worms, and Trojan horses – These contain malicious software or code running on a user device.

  - Spyware and adware – These are types of software which are installed on a user’s device.  The software then secretly collects information about the user.

  - Zero-day attacks – Also called zero-hour attacks, these occur on the first day that a vulnerability become known.

  - Threat actor attacks – A malicious person attacks user devices or network resources.

  - Denial of server attacks – These attacks slow or crash applications and processes on a network device.

  - Data interception and theft – This attack captures private information from an organization’s network.

  - Identity theft – This attack steals the login credentials of a user in order to access private data.

It is equally important to consider internal threats.  There have been many studies that show that the most common data breaches happen because of internal users of the network.  This can be attributed to lost or stolen devices, accidental misuse by employees, and in the business environment, even malicious employees.  With the evolving BYOD strategies, corporate data is much more vulnerable.  Therefore, when developing a security policy, it is important to address both external and internal security threats.

### Security Solutions

No single solution can protect the network from the variety of threats that exist.  For this reason, security should be implemented in multiple layers, using more than one security solution.  If one security component fails to identify and protect the network, other may succeed.

A home network security implementation is usually rather basic.  Typically, you implement it on the end devices, as well as at the point of connection to the internet, and can even rly on contracted services from the ISP.

These are the basic security components for a home or small office network:

  - Antivirus and antispyware – Hese applications help to protect end devices from becoming infected with malicious software.

  - Firewall filtering  - Firewall filtering blocks unauthorized access into and out of the network.  This may include a host-based firewall system that prevents unauthorized access to the end device, or a basic filtering service on the home router to prevent unauthorized access from the outside world into the network.

In contrast, the network security implementation for a corporate network usually consists of many components built into the network to monitor and filter traffic.  Ideally, al components work together, which minimized maintenance and improves security  Larger networks and corporate networks use antivirus, antispyware, and firewall filtering, but they also have other security requirements:

  - Dedicated firewall systems – These provide more advanced firewall capabilities that can filter large amounts of traffic with more granularity.

  - Access control lists (ACL)  - These further filter access and traffic forwarding based on IP addresses and application.

  - Intrusion prevention system (IPS) – These identify fast-spreading threats, such as zero-day or zero-hour attacks.

  - Virtual private networks (VPN) – These provide secure access into an organization for remote workers.

Network security requirements must consider the environment, as well as the various application and computing requirements. Both home and business environments must be able to secure their data while still allowing for the quality of service that users expect of each technology.  Additionally, the security solution implemented must be adaptable to the growing and changing trends of the network.

The study of network security threats and mitigation techniques starts with a clear understanding of the underlying witching and routing infrastructure used to organize network services.

# Basic Switch and End Device Configuration

## Cisco IOS Access

### Operating Systems

All end devices and network devices require an operating system (OS). The portion of the OS that interacts directly with the computer hardware is known as the kernel.  The portion that interfaces with applications and the user is known as the shell.  The user can interact with the shell using a command-line interface (CLI) or a graphical user interface (GUI).

- Shell  - The user interface that allows users to request specific tasks from the computer.  These requests can be made either through CLI or GUI interfaces.

- Kernal – Communicates between the hardware and software of a computer and manages how hardware resources are used to meet software requirements.

- Hardware – The physical part of a computer including underlying electronics.

When using a CLI, the user interacts directly with the system in a text-based environment by entering commands on the keyboard at a command prompt.  The system executes the command, often provided textual output.  The CLI requires very little overhead to operate.  However, it does require that the user have knowledge of the underlying command structure that controls the system.

### GUI

A GUI such as Windows, macOS, Linux KDE, Apple iOS, or Android allows the user to interact with the systems using an environment of graphical icons, menus and windows.  The GUI is ore user-friendly and requires less knowledge of the underlying command structure that controls the system.  For this reason, most users rely on GUI environments.

However, GUIs may not always be able to provide all the features available with the CLI.  GUIs can also fail, crash or simply not operate as specified.  For these reasons, network devices are typically accessed through CLI.  The CLI is less resource intensive and very stable when compared to a GUI.

The family of network operating systems used on many Cisco devices is called the Cisco Internetwork Operating System (IOS). Cisco IOS is used on many Cisco routers and switches regardless of the type or size of the device.  Each device router or switch type uses a different version of Cisco IOS. Other Cisco operating systems include IOS XE, IOS XR, and NX-OS.

Note: The operating system on home routers is usually called firmware. The most common method for configuring a home router is by using a web browser-based GUI.

### Purpose of an OS

Network operating systems are similar to PC operating systems. Through GUI, a PC operating system enable a user to do the following:

- Use a mouse to make selections and run programs

- Enter text and text-based commands

- View output on a monitor

A CLI-based network operating system (e.g. the Cisco IOS on a switch or router) enables a network technician to do the following:

- Use a keyboard to run CLI-based network programs.

- Use a keyboard to enter text and text-based commands.

- View output on a monitor.

Cisco networking devices run particular versions of the Cisco IOS. The IOS version is dependent on the type of device being used and the required features. While all devices come with a default IOS and feature set, it is possible to upgrade the IOS version or feature set to obtain additional capabilities.

### Access Methods

A switch will forward traffic by default and does not need to be explicitly configured to operate.  For example, two configured hosts connected to the same new witch would be able to communicate.

Regardless of the default behaviour of a new switch, all switches should be configured and secured.

| Console | This is a physical management port that provides out-of-band access to a Cisco device. Out-of-band access refers to access via a dedicated management channel that is use for device maintenance purposes only. The advantage of using a console port is that the device is accessible even if no networking services are configured, such as performing the initial configuration. A computer running terminal emulation software and a special console cable to connect to the device are required for a console connection. |
| --- | --- |
| Secure Shell (SSH) | SSH is an in-band and recommended method for remotely establishing a secure CLI connection, through a virtual interface, over a network. Unlike a console connection, SSH connections require active networking services on the device, including an active interface configured with an address. Most versions of Cisco IOS include an SSH server and an SSH client that can be used to establish SSH sessions with other devices. |
| Telnet | Telnet is an insecure, in-band method of remotely establishing a CLI session, through a virtual interface, over a network. Unlike SSH, Telnet does not provide a secure, encrypted connection and should only be use in a lab environment. User authentication, passwords and the commands are sent over the network in plaintext. The best practice is to use the SSH instead of Telnet. Cisco IOS include both a Telnet server and a Telnet client |

Note: Some devices, such as routers, may also support a legacy auxiliary port that was used to establish a CLI session remotely over a telephone connection using a modern.  Similar to a console connection, the AUX port is out-of-band and does not require networking services to be configured or available.

### Terminal Emulation Program

There are several terminal emulation programs you can use to connect to a networking device either by a serial connection over a console port, or by an SSH/Telnet connection.  These program allow you to enhance your productivity by adjusting window sizes, changing font sizes and changing color schemes.

## IOS Navigation

### Primary Command Modes

In the previous topic, you learned that all network devices require an OS and that they can be configured using the CLI or a GUI.  Using the CLI may provide the network administrator with more precise control and flexibility than using the GUI.  This topic discuses using CLI to navigate the Cisco IOS

As a security feature, the Cisco IOS software separates management access into the follow two command modes:

- User EXEC Mode – This mode has limited capabilities but is useful for basic operations.  It allows only a limited number of basic monitoring commands but does not allow the execution of any commands that might change the configuration of the device.  The user EXEC mode is identified by the CLI prompt that ends with the > symbol

- Privileged EXEC Mode – To execute configuration commands, a network administrator must access privileged EXEC mode.  Higher configuration modes, like global configuration mode, can only be reached from privilege EXEC mode. The privileged EXEC mode can be identified by the prompt ending with the # symbol.

| User ExXEC Mode | - Mode allows access to only a limited umber of basic monitoring commands - It is often referred to as “view-only” mode. | Switch> Router> |
| --- | --- | --- |
| Privilege EXEC Mode | - Mode allows access to all commands and features. - The user can use any monitoring commands and execute configuration and management commands. | Switch# Router# |

### Configuration Mode and Subconfiguration Modes

To configure the device, the user must enter global configuration mode, which is commonly called global config mode.

From global config mode, CLI configuration changes are made that affect the operation of the device.  Global configuration mode is identified by a prompt that ends with (config)# after the device name, such as Switch(config)#.

Global configuration mode is accessed before other specific configuration modes.  From global config mode, the user can enter different subconfiguration modes.  Each of these modes allows the configuration of a particular part of function of the IOS device.  Two common subconfiguration modes include:

- Line Configuration Mode – Used to configure console, SSH, Telnet or AUX access.

- Interface Configuration Mode – Used to configure a switch port or router network interface.

When the CLI is used, the mode is identified by the command-line prompt that is unique to that mode.  By default, every prompt begins with the device name.  Following the name, the remainder of the prompt indicates the mode.  For example, the default prompt for line configuration mode is Switch(config-line)# and the default prompt for interface configuration mode is Switch(config-if)#.

### Navigation Between IOS Modes

Various commands are used to move in and out of command prompts. To move from user EXEC mode to privileged EXEC mode, use the enable command.  Use the disable privileged EXEC mode command to return to user EXEC mode.

Note: Privilege EXEC mode is sometimes called enable mode.

To move in and out of global configuration mode, use the configure terminal privilege EXEC mode command.  To return to the privileged EXEC mode, enter the exit global config mode command.

There are many different subconfiguration modes.  For example, to enter line subconfiguration mode, you use the line command followed by the management line type and number you wish to access.  Use the exit command to exit the subconfiguration mode and return to global configuration mode.

To move from any subconfiguration mode of the global configuration mode to the mode one step above it in the hierarchy of modes, enter the exit command.

To move from any subconfiguration mode to the privilege EXEC mode, enter the end command or enter the key combination Ctrl+Z.

You can also move directly from one subconfiguration mode to another. Notice how after selecting an interface, the command prompt changes from (config-line)# to (config-if)#.

### Note About Syntax Checker Activities

When you are learning how to modify device configurations, you might want to start in a safe, non-production environment before trying it on real equipment.  NetAcad gives you different simulation tools to help build your configuration and troubleshooting skills.  Because these are simulation tools, they typically do not have all the functionality of real equipment.  One such tool is the Syntax Checker. In each Syntax Checker, you are given a set of instructions to enter a specific set of commands.  You cannot progress in Syntax Checker unless the exact and full command is entered as specified.  More advanced simulation tools, such as Packet Tracer, let you enter abbreviated commands, much as you would do on real equipment.

## The Command Structure

### Basic IOS Command Structure

This topic covers the basic structure of commands of the Cisco IOS.  A network administrator must know the basic IOS command structure to be able to use the CLI for devices configuration.

A Cisco IOS device supports many commands.  Each IOS command has a specific format, or syntax, and can only be executed in the appropriate mode.  The general syntax for a command, shown in the figure, is the command followed by any appropriate keywords and arguments.

- Keyword – This is a specific parameter defined in the operating system (in the figure, ip protocols).

- Argument – This is not predefined; it is a value or variable defined by the user (in the figure, 192.168.10.5).

After entering each complete command, including any keywords and arguments, press the Enter key to submit the command to the command interpreter.

### IOS Command Syntax Check

A command might require one or more arguments.  To determine the keywords and arguments required for a command, refer to the command syntax.  The syntax provides the pattern, or format, that must be used when entering a command.

As identified in the table, boldface text indicates commands and keywords that are entered as shown.  Italic indicates an argument for which the user provides the value.

| Boldface | Boldface text indicates commands and keywords that you enter literally as shown. |
| --- | --- |
| Italics | Italic text indicates arguments for which you supply values. |
| [square brackets] | [Square brackets] indicate an optional element (keyword or argument). |
| {braces} | {braces} indicate a required element (keyword or argument). |
| [braces { and ¦ vertical } lines ] | Braces and vertical lines within square brackets indicate a required choice within a optional element. Spaces are used to clearly delineate parts of the command. |

For instance, the syntax for using the description command is description string. The argument is a string value provided by the user.  The description command is typically used to identify the purpose of an interface.  For example, entering the command, description Connects to the main headquarters office switch, describes where the other device is at the end of the connection.

The following examples demonstrate conventions used to document and use IOS commands:

- Ping ip-address – The command is ping and the user-defined argument is the ip-address of the destination device.  For example, ping 10.10.10.5

- Traceroute ip-adress – The command traceroute and the user-defined argument is the ip-address of the destination device.  For example, traceroute 192.168.254.254.

If a command is complex with multiple arguments, you may see it represented like this:

The command will typically be followed with a detailed description of the command and each argument.

The Cisco IOS Command Reference is the ultimate source of information for a particular IOS command.

### IOS Help Features

The IOS has two forms of help available: context-sensitive help and command syntax check.

Context-sensitive help enables you to quickly find answers to these questions:

- Which commands are available in each command mode?

- Which command starts with specific characters or group of characters?

- Which arguments and keywords are available to particular commands?

To access context-sensitive help, simply enter a question mark at the CLI.

Command syntax check verifies that a valid command was entered by the user.  When a command is entered, the command line interpreter evaluates the command from left to right. If the interpret understands the command, the requested action is executed, and the CLI returns to the appropriate prompt.  However, if the interpreter cannot understand the command being entered, it will provide feedback describing what is wrong with the command.

### Hot Keys and Shortcuts

The IOS CLI provides hot keys and shortcuts that make configuring, monitoring, and troubleshooting easier.

Commands and keywords can be shortened to the minimum number of characters that identify a unique selection.  For example, the configure command can be shortened to conf because configure is the only command that begins with conf. An even shorter version, con, will not work because more than one command begins with con.  Keywords can also be shortened.

The table lists keystrokes to enhance command line editing.

| TAB | Completes a partial command name entry |
| --- | --- |
| BACKSPACE | Erases the character to the left of the cursor |
| CTRL + D | Erases the character at the cursor |
| CTRL + K | Erases all character from the cursor to the end of the command line |
| ESC D | Erases all characters from the cursor to the end of the word |
| CTRL + U or CTRL + X | Erases all characters from the cursor back to the beginning of the command line |
| CTRL + W | Erases the word to the left of the cursor |
| CTRL + A | Moves the cursor to the beginning of the line |
| LEFT ARROW or CTRL + B | Moves the cursor one character to the left |
| ESC B | Moves the cursor back one word to the left |
| ESC F | Moves the cursor forward one character to the right |
| UP ARROW or CTRL + P | Recalled the previous command in the history buffer, beginning with the most recent command |
| DOWN ARROW or CTRL + N | Goes to the next line in the history buffer |
| CTRL + R or CTRL + I or CTRL + L | Redisplays the system prompt and command line after a console message is received |

Note: While the DELETE key typically deletes the character to the right of the prompt, the IOS command structure does not recognize the DELETE key.

When a command output produces more text than can be displayed in a terminal window, the IOS will display a “—More—” prompt. The following table describes the keystrokes that can be used when this prompt is displayed.

| ENTER Key | Displays the next line |
| --- | --- |
| SPACE BAR | Displays the next screen |
| Any other key * | Ends the display string, returning to previous prompt. *Except “y”, which answers “yes” to the –More—prompt, and acts like the SPACE BAR |

This table lists commands used to exit out of an operation,

| CTRL + C | When in any configuration mode, end the configuration mode and returns to privileged EXEC mode. When in setup mode, aborts ack to the command prompt. |
| --- | --- |
| CTRL + Z | When in any configuration mode, ends the configuration mode and returns to privileged EXEC mode. |
| CTRL + SHIFT + 6 | All-purpose break sequence used to abort DNS lookups, traceroutes, pings, etc. |

## Basic Device Configuration

### Device Name

You have learned a great deal about the Cisco IOS, navigating the IOS, and the command structure.  Now you’re ready to configure devices!  The first configuration command on any device should be to give it a unique device name or hostname.  By default, all devices are assigned a factory default name.  For example, a Cisco IOS switch is “Switch.”

The problem is, if all switches in a network were let with their default names, it would be difficult to identify a specific device.  For instance, how would you know that you are connected to the right device when accessing it remotely using SSH?  The hostname provides confirmation that you are connected to the correct device.

The default name should be changed to something more descriptive.  By choosing names wisely, it is easier to remember, document and identify network devices.  Here are some important naming guidelines for hosts.

  - Start with a letter

  - Contain no spaces

  - End with a letter or digit

  - Use only letter, digits and dashes

  - Be less than 64 characters in length

An organization must choose a naming convention that makes it easy and intuitive to identify a specific device. The hostnames used in the device IOS preserve capitalization and lowercase characters.  For example, the figure shows that three switches, spanning three different floors, are interconnected together in a network.  The naming convention that was used incorporated the location and the purpose of each device.  Network documentation should explain how these name were chosen so additional devices can be named accordingly.

When the naming convention has been identified, the next step is to use the CLI to apply the names to the devices.  As shown in the example, from the privileged EXEC mode, access the global configuration mode by entering the configure terminal command.

From global configuration mode, enter the command hostname followed by the name of the switch and press Enter.

Note: To return the switch to the default prompt, use the no hostname global config command.

Always make sure the documentation is updated each time a device is added or modified.  Identify devices in the documentation by their location, purpose, and address.

### Password Guidelines

The use of weak or easily guessed passwords continues to be the biggest security concern of organizations.  Network devices, including home wireless routers, should always have passwords configured to limit administrative access.

Cisco IOS can be configured to use hierarchical mode passwords to allow different access privileges to a network device.

All networking devices should limit administrative access by securing privileged EXEC, user EXEC, and remote Telnet access with passwords.  In addition, all passwords should be encrypted and legal notifications provided.

When choosing passwords, use strong passwords that are not easily guessed.  There are some key points to consider when choosing passwords:

  - Use passwords that are more than eight characters in length

  - Use a combination of upper and lowercase letters, numbers, special characters and/or numeric sequences

  - Avoid using the same password for all devices

  - Do not use common words because they are easily guessed

Use an internet search to find a password generator.  Many will allow you to set the length, character set and other parameters.

Note: Most of the labs in this course use simple passwords such as cisco or class.  These passwords are considered weak and easily guessable and should be avoided in production environments.  We only use these passwords for convenience in a classroom setting, or to illustrate configuration examples.

### Configure Passwords

When you initially connect to a device, you are in user EXEC mode.  This mode is secured using the console.

To secure user EXEC mode access, enter line console configuration mode using line console 0 global configuration command, as shown in the example.  The zero is used to represent the first (and in most cases the only) console interface.  Next, specify the user EXEC mode password using the password password command.  Finally, enable user EXEC access using the login command.

Console access will now require a password before allowing access to the user EXEC mode.

To have administrator access to all IOS commands including configuring a device, you must gain privileged EXEC mode access.  It is the most important access method because it provides complete access to the device.

To secure privileged EXEC access, use the enable secret password global config command, as shown in the example.

Virtual terminal (VTY) lines enable remote access using Telnet or SSH to the device.  Many Cisco witches support up to 16 VTY lines that are numbered 0 to 15.

To secure VTY lines, enter the line VTY mode using the line vty 0 15 global config command.  Next, specify the VTY password using the password password command.  Lastly, enable VTY access using login command.

An example of securing the VTY lines on a switch is shown.

### Encrypt Passwords

The startup-config and running-config files display most passwords in plaintext.  This is a security threat because anyone can discover the passwords if they have access to these files.

To encrypt all plaintext passwords, use the service password-encryption global config command as shown in the example.

The command applies weak encryption to all unencrypted passwords.  This encryption applies only to passwords in the configuration file, not to passwords as they are sent over the network.  The purpose of this command is to keep unauthorized individuals from viewing passwords in the configuration file.

Use the show running-config command to verify that passwords are now encrypted.

### Banner Messages

Although requiring passwords is one way to keep unauthorized personnel out of the network, it is vital to provide a method for declaring that only authorized personnel should attempt to access the device.  To do this, add a banner to the device output.  Banners can be an important part of the legal process if someone is prosecuted for breaking into a device.  Some legal systems do not allow prosecution, or even the monitoring of users, unless a notification is visible.

To create a banner message of the day on a network device, use the banner motd # the message of the day # global config command.  The “#” in the command syntax is called the delimiting character.  It is entered before and after the message.  The delimiting character can be any character as long as it does not occur in the message.  For this reason, symbols such as “#” are often used.  After the command is executed, the banner will be displayed on all subsequent attempts to access the device until the banner is removed.

The following example shows the steps to configure the banner on Sw-Floor-1.

## Save Configurations

### Configuration Files

You know how to perform basic configuration on a switch, including passwords and banner messages.  This topic will show you how to save your configurations.

There are two system files that store the device configuration:

  - Startup-config – This is the saved configuration file that is stored in NVRAM.  It contains all the commands that will be use by the device upon start or reboot.  Flash does not lose its contents hen the device is powered off.

  - running-config – This is stored in Random Access Memory (RAM). It reflects the current configuration.  Modifying a running configuration affects the operation of a Cisco device immediately.  RAM is volatile memory.  It loses all of its content when the device is powered off or restarted.

The show running-config privileged EXEC mode command is used to view the running config.  As shown in the example, the command will list the complete configuration currently stored in RAM.

To view the startup configuration file, use the show startup-config privileged EXEC command.

If power to the device is lost, or if the device is restarted, all configuration changes will be lost unless they have been saved.  To save changes made to the running configuration to the startup configuration file, use the copy running-config startup-config privileged EXEC mode command.

### Alter the Running Configuration

If changes made to the running config do not have the desired effect and the running-config has not yet been saved, you can restore the device to its previous configuration.  Remove the changed commands individually or reload the device using the reload privileged EXEC mode command to restore the startup-config.

The downside to using the reload command to remove an unsaved running config is the brief amount of time the device will be offline, causing network downtime.

When a reload is initiated, the IOS will detect that the running config has changed that were not saved to the startup configuration.  A prompt will appear to ask whether to save the changes.  To discard the changes, enter n or no.

Alternatively, if undesired changes were saved to the startup config, it may be necessary to clear all the configuration.  This requires erasing the startup config and restarting the device.  The startup config is removed by using the erase startup-config privilege EXEC mode command.  After the command is issued, the switch will prompt you for confirmation.  Press ENTER to accept.

After removing the startup config from NVRAM, reload the device to remove the current running config file from RAM.  On reload, a switch will load the default startup config that originally shipped with the device.

### Capture Configuration to a Text File

Configuration files can also be saved and archived to a text document.  This sequence of steps ensures that a working copy of the configuration file is available for editing or reuse later

For example, assume that a switch has been configured, and the running config has been saved on the device.

Step 1. Open terminal emulation software, such as PuTTY or Tera Term, that is already connect to a switch.

Step 2. Enable logging in the terminal software and assign a name and file location to save the log file.  The figure displays that All session output will be captured to the file specified (i.e., MySwitchLogs.)

Step 3. Execute the show running-config or show startup-config command at the privileged EXEC prompt.  Text displayed in the terminal window will be placed into the chosen file.

Step 4. Disable logging in the terminal software.  The figure shows how to disable logging by choosing None session logging option.

The text file created can be used as a record of how the device is currently implemented.  The file could require editing before being used to restore as saved configuration to a device.

To restore a configuration file to a device:

Step 1.  Enter global configuration mode on the device.

Step 2. Copy and paste the text file into the terminal window connected to the switch.

The text in the file will be applied as commands in the CLI and become the running configuration on the device.  This is a convenient method of manually configurating a device.

## Ports and Address

### IP Addresses

If you want your end devices to communicate with each other, you must ensure that each of them has an appropriate IP address and is correctly connected.  You will learn about IP addresses, device ports and the media used to connect devices in this topic.

The use of IP addresses is the primary means of enabling devices to locate one another and establish end-to-end communication on the internet.  Each end device on a network must be configured with an IP address.  Examples of end devices include the following:

  - Computers (work stations, laptops, file servers, web servers)

  - Network printers

  - VoIP phones

  - Security cameras

  - Smart phones

  - Mobile handheld devices (such as wireless barcode scanners_

The structure of an IPv4 address is called dotted decimal notation and is represented by four decimal numbers between 0 and 255. IPv4 addresses are assigned to individual devices connected to a network.

Note: IP in this course refers to both IPv4 and IPv6 protocols.  IPv6 is the most recent version of IP and is replacing the more common IPv4.

With the IPv4 address, a subnet mask is also necessary.  An IPv4 subnet mask is a 32-bit value that differentiates the network portion of the address from the host portion.  Coupled with the IPv4 address, the subnet mask determines which subnet the device is a member

The example in the figure displays the IPv4 address (192.168.1.10), subnet mask (255.255.255.0), and default gateway (192.168.1.1) assigned to a host.  The default gateway address is the IP address of the router that the host will use to access remote networks, including the internet.

IPv6 addresses are 128 bits in length and written as a string of hexadecimal values.  Every four bits is represented by a hexadecimal digit; for a total of 32 hexadecimal values  Groups of four hexadecimal digits are separated by a colon.  IPv6 addresses are not case-sensitive and can be written in either lowercase or uppercase.

### Interfaces and Ports

Network communications depend on end user device interfaces, networking device interfaces, and the cables that connect them.  Each physical interface has specifications, or standards, the define it.  A cable connecting to the interface must be designed to match the physical standards of the interface.  Types of network media include twisted-pair copper, fiber-optic cables, coaxial cables, or wireless.

Different types of network media have different features and benefits.  Not all network media have the same characteristics.  Not all media are appropriate for the same purpose.  These are some of the differences between various types of media:

  - Distance the media can successfully carry a signal

  - Environment in which the media is to be installed

  - Amount of data and the speed at which it must be transmitted

  - Cost of the media and installation

Not only does each link on the internet require a specific network media type, but each link also requires a particular network technology.  For example, Ethernet is the most common local-area network (LAN) technology used today.  Ethernet ports are found on end-user devices, switch devices and other networking devices that can physically connect to the network using a cable.

Cisco IOS Layer 2 switches have physical ports for devices to connect.  These ports do not support Layer 3 IP addresses.  Therefore, switches have one or more switch virtual interfaces (SVIs). These are virtual interfaces because there is no physical hardware on the device associated with it.  An SVI is created in software.

The virtual interface lets you remotely manage a switch over a network using IPv4 and IPv6.  Each switch comes with one SVI appearing in the default configuration “out of the box”.  The default SVI is interface VLAN1.

Note: A Layer 2 switch does not need an IP address.  The IP address assigned to the SVI is use to remotely access the switch.  An IP address is not necessary for the switch to perform its operation.

## Configure IP Addressing

### Manual IP Address Configuration for End Devices

Much like you need your friends’ telephone numbers to text or call them, end devices in your network need an IP address so that they can communicate with other devices on your network.  In this topic, you ill implement basic connectivity by configuring IP addressing on switches and PCs.

IPv4 address information can be entered into end devices manually, r automatically using Dynamic Host Configuration Protocol (DHCP).

To manually configure an IPv4 address on a Windows host, open the Control Panel > Network Sharing Center > Change adaptor setting and choose the adaptor.  Next, right-click and select Properties to display the Local Area Connection Properties, as shown in the figure.

Highlight Internet Protocol Version 4 (TCP/IPv4) and click Properties to open the Internet Protocol Version 4 (TCP/IPv4) Properties window, shown in the figure.  Configure the IPv4 address and subnet mask information and default gateway.

Note: IPv6 addressing and configuration are similar to IPv4.

Note: The DNS server addresses are the IPv4 and IPv6 addresses of the Domain Name System (DNS) server, which are use to translate IP addresses to domain names.

### Automatic IP address Configuration for End Devices

End devices typically default to using DHCP for automatic IPv4 address configuration.  DHCP is a technology that is used in almost every network.  The best way to understand why DHCP is so popular is by considering all the extra work that would have to take place without.

In a network, DHCP enable automatic IPv4 address configuration for every end device that is DHCP-enabled. Imagine the amount of time it would take I every time you connected to the network, you had to annually enter the IPv4 address, the subnet mask, the default gateway, and the DNS serv er.  Multiple that by every user and every device in an organization and you see the problem.  Manual configuration  also increases the chance of misconfiguration by duplication another device’s IPv4 address.

As shown in the figure, to configure DHCP on a Windows PC, you only need to select Obtain an IP address automatically and Obtain DNS server address automatically.  You PC will search out a DHCP server and be assigned the address settings necessary to communicate on the network.

Note: IPv6 uses DHCPv6 and SLAAC (Stateless Address Autoconfiguraton) for dynamic address allocation.

### Switch Virtual Interface Configuration

To access the switch remotely, an IP address and a subnet mask must be configured on the SVI.  To configure an SVI on a witch, use the interface vlan 1 global configuration command.  VLAN 1 is not an actual physical interface but a virtual one. Next assign an IPv4 address using the ip address ip-address subnet mask interface configuration command.  Finally, enable the virtual interface using the no shutdown interface configuration command.

After these commands are configured, the switch has all the IPv4 elements ready for communication over the network.

Note: Similar to a Window host, switch configured with IPv4 address will typically also need to have default gateway assigned.  This can be done using the ip default-gateway ip-address parameter would be the IPv4 address of the local router on the network, as shown in the example.  However, in this module you will only be configuring a network with switches and hosts.  Routers will be introduced later.

# Protocols and Models

## The Rules

### Communications Fundamentals

Networks vary in size, shape and function.  They can be as complex as devices connected across the internet, or as simple as two computers directly connected to one another with a single cable an anything in-between.  However, simply having a wired or wireless physical connection between end devices is not enough to enable communication.  For communication to occur, devices must know “how” to communicate.

People exchange ideas using many different communication methods.  However, all communication methods have the following three elements in common:

  - Message source (sender) – Message sources are people, or electronic devices, that need to send a message to other individuals or devices.

  - Message Destination (receiver) – The destination receives the message interprets it.

  - Channel – This consists of the media that provides the pathway over which the message travels from source to destination.

### Communication Protocols

Sending a message, whether by face-to-face communication or over a network, is governed by rules called protocols.  These protocols are specific to the type of communication method being used.  In our day-to-day personal communing, the rules we use to communicate over one medium, like a telephone call, are not necessarily the same as the rules for using another medium, such as sending a letter.

The process of sending a letter is a like communication that occurs in computer networks.

Imagine two people attempting to communicate, face-to-face.

They must agree on how to communicate.  If the communication is to be voice, they must first agree on the language.  Next, they will have to format the message they want to communicate in a way that is understandable.  If the language is English, but poor sentence structure is used, then the message may be misunderstood.

### Rule Establishment

Before communicating with another, individuals must use established rules or agreements to govern the conversation.  Consider this message for example

“humans communication between govern rules. It is verydifficult tounderstand messages that are not correctly formatted and donot follow the established rules and protocols. A estrutura da gramatica, da lingua, da pontuacao e do sentence faz a configuracao humana compreensivel por muitos individuos diferentes.”

It is difficult to read the message because it is not formatted properly.  It should be written using rules (i.e., protocols) that are necessary for effective communication.  The example below shows the message, following proper protocols:

“Rules govern communication between humans. It is very difficult to understand messages that are not correctly formatted and do not follow the established rules and protocols. The structure of the grammar, the language, the punctuation and the sentence make the configuration humanly understandable for many different individuals.”

Protocols must account for the following requirements to successfully deliver a message that is understood by the receiver:

  - An identified sender and receiver

  - Common language and grammar

  - Speed and timing of delivery

  - Confirmation or acknowledgement of requirements.

### Network Protocol Requirements

The protocols that are used in network communications share many of these fundamental traits.  In addition to identifying the source and destination, computer and network protocols define the details of how a message s transmitted across a network.  Common computer protocols include the following requirements:

  - Message encoding

  - Message formatting and encapsulation

  - Message size

  - Message timing

  - Message delivery options

### Message Encoding

One of the first steps to sending a message is encoding.  Encoding Is the process of converting information into another acceptable form, for transmission.  Decoding reverses this process to interpret the information.

Imagine a person calls a friend to discuss the details of a beautiful sunset.  To communicate the message, she converts her thoughts into an agreed upon language.  She then speeds the words using the sounds and inflections of the spoken language that convey the message.  Her friend then listens to the description and decodes the sounds to understand the message he received.

Encoding between hosts must be in an appropriate format for the medium.  Messages sent across the network are first converted into bits by the sending host.  Each bit is encoded into a pattern of voltages on copper wires, infrared light in optical fibers, or microwaves for wireless systems.  The destination host receives an decodes the signals to interpret the message

### Message Formatting and Encapsulation

When a message is sent from the source to the destination, it must use a specific format or structure.  Message formats depend on the type of message and the channel that is used to deliver the message.

A common example of requiring the correct format in human communication is when sending a letter.  An envelope has the address of the sender and receiver; each located at the proper place on the envelope.  If it’s destination address and formatting are not correct, the letter is not delivered.

The process of placing one message format (the letter) inside another message format (the envelope) is called encapsulation. De-encapsulation occurs when the process is reversed by the recipient, and the letter is removed from the envelope.

Similar to sending a letter, a message that is sent over a computer network follows specific format rules for it to be delivered and processed.

Internet Protocol (IP) is a protocol with a similar function to the envelope example.  In the figure, the fields of the Internet Protocol version 6 (IPv6) packet identify the source of the packet and its destination.  IP is responsible for sending a message from the message source to destination over one or more networks.

### Message Size

When people communicate with each other, the messages that they send are usually broken into smaller parts or sentences.  These sentences are limited in size to what the receiving person can process at one time.

Likewise, when a long message is sent from one host to another over a network, it is necessary to break the message into smaller pieces.  The rules that govern the size of the pieces or frames, communicated across the network are very strict.  They can also be different, depending on the channel used.  Frames that are too long or too short are not delivered.

The size restrictions of frames require the source host to break a long message into individual pieces that meet both the minimum and maximum size requirements.  The long message will be sent in separate frames, with each frame containing a piece of the original message.  Each frame will also have its own addressing information at the receiving host, the individual pieces of the message are reconstructed into the original message.

### Message Timing

Message timing is also very important in network communications.  Message timing includes the following:

  - Flow Control – This is the process of managing the rate of data transmission.  Flow control defines how much information can be sent and the speed at which it can be delivered.  For example, if one person speaks too quickly, it may be difficult for the receiver to hear and understand the message.  In network communication, there are network protocols used by the source and destination devices to negotiate and manage the flow of information.

  - Response Timeout – If a person asks a question and does not hear a response within an acceptable amount of time, the person assumes that no answer is coming and reacts accordingly.  The person may repeat the question or instead may go on with the conversation. Hosts on the network use network protocols that specify how long to wait for responses and what action to take if a response timeout occurs.

  - Access method – This determines when someone can send a message. When a device wants to transmit on a wireless LAN, it is necessary for the WLAN network interface card (NIC) to determine whether the wireless medium is available.

### Message Delivery Options

A message can be delivered in different ways

Sometimes, a person wants to communicate information to a single individual.  At other times, the person may need to send information to a group of people at the same time, or even to all people in the same area.

Network communications has similar delivery options to communicate. There are three types of data communication:

  - Unicast – Information is being transmitted to a single end device

  - Multicast – Information is being transmitted to one or more end devices

  - Broadcast – Inormation is being transmitted to all end devices

Note: The multicast animation is indicating the destination devices.  By default, a switch will end multicast packets out all ports except the incoming port. However, only the hosts that are part of the multicast group will process the packet.

## Protocols

### Network Protocols Overview

You know that for end devices to be able to communicate over a network, each device must abide by the same set of rules.  These rules are called protocols, and they have many functions in a network.  This topic gives you an overview of network protocol.

Network protocols define a common format and set of rules for exchanging messages between devices.  Protocols are implemented by end devices and intermediary devices.  Protocols are implemented by end devices and intermediary devices in software, hardware or both.  Each network protocol has its own function, format, and rules for communications.

The table lists the various types of protocols that are needed to enable communications across one or more networks.

| Network Communications Protocols | Protocols enable two or more devices to communicate over one or more networks. The Ethernet family of technologies involves a variety of protocols such as IP, Transmission Control Protocol (TCP), HyperText Transfer Protocol(HTTP) and many more |
| --- | --- |
| Network security Protocols | Protocols secure data to provide authentication, data integrity, and data encryptions. Examples of secure protocols include Secure Shell (SSH), Secure Sockets Layers (SSL) and Transport Layer Security (TLS) |
| Routing Protocols | Protocols enable routers to exchange route information, compare path information and then to select the best ath to the destination network. Examples of routing protocols include Open Shortest Path First (OSPF) and Border Gateway Protocol (BGP) |
| Service Discovery Protocols | Protocols are use for the automatic detection of devices for services. Examples of service discovery protocols include Dynamic Host Configuration Protocol (DHCP) which discovers services for IP address allocation and Domain Name System (DNS) which is used to perform name-to-IP address translation. |

### Network Protocol Functions

Network communication protocols are responsible for a variety of functions necessary for network communications between end devices.

Computers and network devices used agreed-upon protocols to communicate.  The table lists the functions of these protocols.

| Addressing | This identifies the sender and the intended receiver of the message using a defined addressing scheme. Examples of protocols that provide addressing include Ethernet IPv4 and IPv6 |
| --- | --- |
| Reliability | This function provides guaranteed delivery mechanisms in case messages are lost or corrupted in transit. TCP provides guaranteed delivery. |
| Flow control | This function provides guaranteed delivery mechanisms in case messages are lost or corrupted in transit TCP provides guarantees delivery |
| Sequencing | This function uniquely labels each transmitted segment of data/ The receiving device uses the sequencing information to reassemble the information correctly. This is useful if the data segments are lost, delayed or received out-of-order. TCP provides sequencing services. |
| Error Detection | This function is used to determine if data became corrupted during transmission. Various protocols that provide error detection include Ethernet, IPv4, IPv6 and TCP |
| Application Interface | This function contains information used for process-to-process communications between network applications. For example, when accessing a web page, HTTP or HTTPS protocols are use to communicate between the client and server web processes. |

### Protocol Interaction

A  message sent over a computer network typically requires the use of several protocols, each one with its own functions and format.  The figure shows some common network protocols that are used when a device sends a request to a web server for its web page.

## Protocol Suites

### Network Protocol Suites

In many cases, protocols must be able to work with other protocols so that your online experience gives you everything you need for network communication.  Protocols suites are designed to work with each other seamlessly.

A protocol suite is a group of inter-related protocols necessary to perform a communication function.

One of the best ways to visualize the protocols within a suite interact is to view the interaction as a stack.  A protocol stack shows how the individual protocols within a suite are implemented.  The protocols are viewed in terms of layers, with each higher-level service depending on the functionality defined by the protocol shown in the lower levels.  The lower layer of the stack are concerned with moving data over the network and providing services to the upper layers, which are focused on the content of the message being sent.

As illustrated in the figure, we can use layers to describe the activity occurring in face-to-face communication.  At the bottom is the physical layer where we have two people with voices saying words out loud.  In the middle is the rules layer that stipulates the requirements of communication including that a common language must be chosen.  At the top is the content layer and this is where the content of the communication is spoken.

### Evolution of Protocol Suites

A protocol suite is a set of protocols that work together to provide comprehensive network communication services.  Since the 1970s there have been several different protocol suites, some developed by a standards organization and other developed by various venders.

During the evolution of network communications and the internet there were several competing protocol suites, as shown in the figure.

### TCP/IP Protocol Example

TCP/IP Protocols are available for the application, transport and internet layers.  There are no TCP/IP protocols in the network access layer.  The most common network access layer LAN protocols are Ethernet and WLAN (wireless LAN) protocols.  Network access layer protocols are responsible for delivering the IP packet over the physical medium.

The figure shows an example of the three TCP/IP protocols used to send packets between the web browser of a host and the web server.  HTTP, TCP and IP are the TCP/IP protocols used.  At the network access layer, Ethernet is used in the example.  However, this could also be a wireless standard such as a WLAN or cellular service.

### TCP/IP Protocol Suite

Today, the TCP/IP protocol suite includes many protocols and continues to evolve to support new service.  Some of the more popular ones are shown in the figure.

The following is a brief description of protocols at each layer.

Application Layer

Name System

    - DNS – Domain Name System.  Translates domain names such as cisco.com, into IP addresses

Host Configuration

    - DHCPv4 – Dynamic Host Configuration Protocol for IPv4.  A DHCPv4 server dynamically assigns IP4 addressing information to DHCP4 clients at start-up and allows the addresses to be re-use when no longer needed

    - DHCPv6 – Dynamic Host Configuration Protocol for IPv6.  DHCPv6 is similar to DHCPv4. A DHCPv6 server dynamically assigns IPv6 address information to DHCPv6 clients at start-up.

    - SLAAC – Stateles Adress Autoconfiguration. A method that allows a device to obtain its IPv6 addressing information without using a DHCPv6 server

Email

    - SMTP – Simple Mail Transfer Protocol.  Enables clients to send email to a mail server and enables servers to send emails to other servers.

    - POP3 – Post Office Protocol version 3.  Enables clients to retrieve email from a mail server and download the email to the client’s local mail application

    - IMAP – Internet Message Access Protocol.  Enables clients to access email stored on a mail server as well as maintaining email on the server.

File Transfer

    - FTP – File Transfer Protocol. Sets the rules that enable as user on one host to access and transfer files to and from another host over a network.  FTP is a reliable, connection-oriented, and acknowledged file delivery protocol.

    - SFTP – SSH File Transfer Protocol.  As an extension to Secure Shell (SSH) protocol, SFTP can be used to establish a secure file transfer session in which the file transfer is encrypted. SSH is a method for secure remote login that is typically used for accessing the command line of a device.

    - TFTP – Trivial File Transfer Protocol.  A simple, connectionless file transfer protocol with best-effort, unacknowledged file delivery.  IT uses less overhead than FTP.

Web and Web Service

    - HTTP – Hypertext transfer Protocol.  A set of rules for exchanging text, graphic images, sound, video and other multimedia files on the World Wide Web.

    - HTTPS – HTTP Secure. A secure form of HTTP that encrypts the data that is exchanged over the World Wide Web.

    - REST – Representational State Transfer.  A web service that uses application programming interfaces (APIs) and HTTP requests to create web applications.

Transport Layer

Connection-Oriented

    - TCP – Transmission Control Protocol.  Enables reliable communication between processes running on separate hosts and provides reliable, acknowledged transmissions that confirm successful delivery

Connectionless

    - UDP – User Datagram Protocol.  Enables a process running on one host to send packets to a process running on another host.  However, UDP does not confirm successful datagram transmission.

Internet Layer

Internet Protocol

    - IPv4 – Internet Protocol version 4.  Receives message segments from the transport layer, packages messages into packets, and addresses packets for end-to-end delivery over a network.  IPv4 uses a 32-bit address

    - IPv6 – Internet Protocol version 6. Similar to IPv4 but uses a 128-bit address

    - NAT – Network Address Translation.  Translate IPv4 addresses from a private network into globally unique public IPv4 addresses

Messaging

    - ICMPv4 – Internet Control Message Protocol for IPv4.  Provide feedback from a destination host to a source host about errors in packet delivery.

    - ICMPv6 – ICMP for Pv6.  Similar functionality to ICMPv4 but is use for IPv6 packets.

    - ICMPv6 ND – ICMPv6 Neighbour Discovery.  Includes four protocol messages that are used for address resolution and duplicate address detection.

Routing Protocol

    - OSPF – Open Shortest Path First.  Link-state routing protocol that uses a hierarchical design based on areas.  OSPF is an open standard interior routing protocol.

    - EIGRP – Enhanced Interior Gateway Routing Protocol.  An open standard routing protocol developed by cisco that uses a composite metric based on bandwidth, delay, load and reliability.

    - BGP – Border Gateway Protocol.  An open standard exterior gateway routing protocol used between Internet Service Providers (ISPs).  BGP is also commonly used between ISPs and their large private clients to exchange routing information.

Network Access Layer

Address Resolution

    - ARP – Address Resolution Protocol.  Provides dynamic address mapping between an IPv4 address and a hardware address

Note: You may see other documentation state that ARP operates at the Internet Layer (OSI Layer 3).  However, in this course we state that ARP operates at the Network Access Layer (OSI Layer 2) because it’s primary purpose is to discover the MAC address of the destination.  A MAC address is a Layer 2 operation.

Data Link Protocols

    - Ethernet – Defines the rules for wiring nd signaling standards of the network access layer.

    - WLAN – Wireless Local Are Network.  Defines the rules for wireless signaling across the 2.4 GHz and 5 GHz radio frequencies.

## Standard Organizations

### Open Standards

When buying new tires for a car, there are many manufacturers you might choose.  Each of them will have at least one type of tire that fits your car.  This is because the automotive industry uses standards when they make cars. It is the same with protocols.  Because there are many different manufacturers of network components, they must all use the same standards.  In networking, standards are developed by international standard organizations.

Open standards encourage interoperability, competition and innovation.  They also guarantee that the product of no single company can monopolize the market or have an unfair advantage over its competition.

A good example of this is when purchasing a wireless router for the home.  There are many different choices available from a variety of vendor, all of which incorporate standard protocols such as IPv4, IPv6, DHCP, SLAAC Ethernet and 801.11 Wireless LAN.  These open standards also allow a client running the Apple OS X operating system to download a web page from a web server running the Linux operating system.  This is because both operating systems implement the open standard protocols, such as those in the TCP/IP protocol suite.

Standards organizations are usually vendor-neutral, non-profit organizations established to develop and promote the concept of open standards.  These organization are important in maintaining an open internet with freely accessible specifications and protocols that can be implemented by any vendor

A standards organization may draft a set of rules entirely on its own or, in other cases, may select a proprietary protocol as the basis for the standard.  If a proprietary protocol is used, it usually involves the vendor ho created the protocol.

### Internet Standards

Various organizations have different responsibilities for promoting and creating standards for the internet and TCP/IP protocol

- Internet Society (ISOC) – Responsible for promoting the open development and evolution of internet use throughout the world.

- Internet Architecture Board (IAB) - Responsible for the overall management and development of internet standards

- Internet Engineering Task Force (IETF) – Develops, updates and maintains internet and TCP/IP technologies.  This includes the process and documents for developing new protocols and updating existing protocols, which are know as Request for Comments (RfC) documents.

- Internet Research Task Force (IRTF) – Focused on long-term research related to internet and TCP/IP protocols such as Anti-Spam Research Group (ASRG), Cypto Forum Research Group (CFRG) and Peer-to-peer Research Group (P2PRG).

The following are standard organizations involved with the development and support of TCP/IP and include IANA and ICANN.

- Internet Corporation for Assigned Names and Numbers (CANN) – Based in the United States, ICANN coordinates IP address allocation, the management of domain names and assignment of the other information used in TCP/IP protocols

- Internet Assigned Numbers Authority (IANA) – Responsible for overseeing and managing IP address allocation, domain name management, and protocol identifiers for ICANN

### Electronic And Communications Standards

Other standards organizations have responsibilities for promoting and creating the electronic and communication standards used to deliver the IP packets as electronic signals over a wired or wireless medium

These standard organizations include the following:

  - Institute of Electrical and Electronics Engineers (IEEE, pronounced “I-triple-E”) – Organization of electrical engineering and electronic dedicated to advancing technological innovation and creating standards in a wide area of industries including power and energy, healthcare, telecommunication, and networking. Important IEEE networking standards included 802.3 Ethernet and 802.11 WLAN standard.  Search the internet for other IEEE network standards.

  - Electronic Industries Alliance (EIA) – Organization is best known for its standard relating to electrical wiring, connectors and the 19-inch racks used to mount networking equipment.

  - Telecommunications Industry Association (TIA) – Organization responsible for developing communication standards in a variety of areas including radio equipment, cellular towers, Voice over IP (VoIP) devices, satellite communication and more

  - International Telecommunications Union-Telecommunication Standard Sector (ITU-T) – One of the largest and oldest communication standards organization.  The ITU-T defines standards for video compression, Internet Protocols Television (IPTV), and broadband communications, such as a digital subscriber line (DSL)

## Reference Model

### The Benefits of Using a Layered Model

You cannot actually watch real packets travel across a real network, the way you can watch the components of a car being put together on an assembly line.  So, it helps to have a way of thinking about a network so that you can imagine what is happening.  A model is useful in these situations.

Complex concepts such as how a network operates can be difficult to explain and understand.  For this reason, a layered model is used to modularize the operations of a network into manageable layers

These are the benefits of using a layered model to describe network protocols and operations:

  - Assisting in protocol design because protocols that operate at a specific layer have defined information that they act upon and a defined interface to the layers above and below

  - Fostering competition because products from different vendors can work together

  - Preventing technology or capability changes in one layer from affecting other layers above and below

  - Providing a common language to describe networking functions and capabilities.

As shown I the figure, there are to layered models that are used to describe network operations:

### The OSI Reference Model

The OSI Reference Model provides an extensive list of functions and services that can occur at each layer.  This type of model provides consistency within all types of network protocols and services by describing what must be done at a particular layer, but not prescribing how it should be accomplished.

It also describes the interaction of each layer with the layers directly above and below.  The TCP/IP protocols discussed in this course are structured around both the OSI and TCP/IP models.  The table shows details about each layer of the OSI model.  The functionality of each layer and the relationship between layers will become more evident throughout this course as the protocols are discussed in more detail:

| 7 – Application | The application layer contains protocols used for process-to-process communications |
| --- | --- |
| 6 – Presentation | The presentation layer provides for common representation of the data transferred between application layer services |
| 5 – Session | The session layer provides services to the presentation layer to organize its dialogue and to manage data exchange |
| 4 – Transport | The transport layer defines services to segment, transfer and reassemble the data for individual communications between the end devices. |
| 3 – Network | The network layer provides services to exchange the individual pieces of data over the network between identified end devices |
| 2 – Data Link | The data link layer protocol describe methods for exchanging data frames between devices over a common media |
| 1 – Physical | The physical layer protocols describe the mechanical, electrical, functional and procedural means to activate, maintain and de-activate physical connections for a bit transmission to and from a network device |

Note: Whereas the TCP/IP model layers are referred to only by name, the seven OSI model layers are more often referred to by number rather than by name.  For instance, the physical layer is referred to as Layer 1 of the OSI model, data link layer is Layer 2 and so on.

### The TCP/IP Protocol Model

The TCP/IP protocol model for internetwork communications was created in the early 1970s and is sometimes referred to as the internet model.  This type of model closely matches this structure of particular protocol suite.  The TCP/IP model is a protocol model because it describes the functions that occur at each layer of protocol within the TCP/IP suite.

| 4 – Application | Represents data to the user, plus encoding and dialog control |
| --- | --- |
| 3 – Transport | Supports communication between various devices across diverse networks |
| 2 – Internet | Determines the best path through the network |
| 1 – Network Access | Controls the hardware devices and media that make up the network |

The definitions of the standard and the TCP/IP protocols are discussed in a public forum and defined in a publicly available set if IETF RFCs.  An RFC is authored by networking engineers and sent to other IETF members for comments.

### OSI and TCP/IP Comparison

The protocols that made up the TCP/IP protocol suite can also be described in terms of the OSI reference model.  In the OSI model, the network access layer and the application layer of the TCP/IP model are further divided to describe discrete functions that must occur at these layers.

At the network access layer, the TCP/IP protocol suite does not specify which protocols to use when transmitting over a physical medium; it only describes the handoff from the internet layer to the physical network protocols.  OSI layers 1 and 2 discuss the necessary procedures to access the media and the physical means to send data over a network.

The key similarities are in the transport and network layers; however, the two models differ in how they relate to the layers above and below each layer

  - OSI Layer 3, the network layer, maps directly to the TCP/IP internet layer.  This layer is used to describe protocols that address and route messages through an internetwork.

  - OSI Layer 4, the transport layer, maps directly to the TCP/IP transport layer.  This layer describes general services and functions that provide ordered and reliable delivery of data between source and destination hosts.

  - The TCP/IP application layer includes several protocols that provide specific functionality to a variety of end user applications.  The OSI model Layers 5, 6 and 7 are used as references for application software developers and vendor to produce applications that operate on networks.

  - Both the TCP/IP and OSI models are commonly used when referring to protocols at various layers.  Because the OSI model separates the data link layer from the physical layer, it is commonly use when referring to these lower layers.

## Data Encapsulation

### Segmenting Messages

Knowing the OSI reference model and the TCP/IP protocol model will come in handy when you learn about how data is encapsulated as it moves across a network.  It is not as simple as physical letter being sent through the mail system.

In theory, a single communication, such as a video or an email message with many large attachments, could be sent across a network from a source to a destination as one massive, uninterrupted stream of bits.  However, this would create problems for other devices needing to use the same communication channels or links.  These large streams of data would result in significant delays.  Further, if any link in the interconnected network infrastructure failed during the transmission, the complete message would be lost and would have to be retransmitted in full.

A better approach is to divide the data into smaller, more manageable pieces to send over the network.  Segmentation is a process of dividing a stream of data into smaller units for transmissions over the network.  Segmentation is necessary because data networks use the TCP/IP protocol suite send data in individual IP packets.  Each packet is sent separately, similar to sending a long letter as a series of individual postcards. Packets containing segments for the same destination can be sent over different paths.

This leads to segmenting messages having two primary benefits:

  - Increases speed – Because a large data stream is segmented into packets, large amounts of data can be sent over the network without typing up a communications link.  This allows many different conversations to be interleaved on the network called multiplexing.

  - Increase efficiency  - If a single segment fails to reach its destination due to failure in the network or network congestion, only that segment needs to be retransmitted instead of resending the entire data stream.

### Sequencing

The challenge to using segmentation and multiplexing to transmit messages across a network is the level of complexity that is added to the process.  Imagine if you had to send a 100-page letter, but each envelope could only hold one page.  Therefore, 100 envelopes would be required and each envelope would need to be addressed individually.  It is possible that the 100-page letter in 100 different envelopes arrives out-of-order.  Consequently, the information in the envelope would need to include a sequence number to ensure that the receiver could reassemble the pages in the proper order.

In network communications, each segment of the message must go through a similar process to ensure that it gets to the correct destination and can be reassembled into the content of the original message.

### Protocol Data Unites

As application data is passed down the protocol stack on its way to be transmitted across the network media, various protocol information is added at each level.  This is known as the encapsulation process.

Note: Although the UDP PDU is called datagram, IP packets are sometimes also referred to as IP datagrams.

The form that a piece of data takes at a layer is called a protocol data unit (PDU).  During encapsulation, each succeeding layer encapsulates the PDU that it receives from the layer above in accordance with the protocol being used.  At each stage of the process, a PDU has a different name to reflect its new functions.  Although there is no universal naming convention for PDUs, in this course, the PDUs are named according to the protocols of the TCP/IP suite.  The PDUs for each form of data are shown in the figure.

### Encapsulation Example

When messages are being sent on a network, the encapsulation process works for top to bottom.  At each layer, the upper layer information is considered data within the encapsulated protocol.  For example, the TCP segment is considered data within the IP packet.

### De-encapsulation Example

This process is reversed at the receiving host ad is known as de-encapsulation.  De-encapsulation is the processed used by receiving device to remove one or more of the protocol headers.  The data is de-encapsulated as it moves up the stack toward the end-user application.

## Data Access

### Addresses

As you just learned, it is necessary to segment messages in a network.  But those segmented messages will not go anywhere if they are not addressed properly.  This topic gives an overview of network addresses.  You will also get the chance to use the Wireshark Tool, which ill help you to “view” network traffic.

The network and data link layers are responsible for delivering the data from the source device to the destination device.  As shown in the figure, protocols at both layers contain a source and destination address, but their addresses have different purposes:

- Network layer source and destination addresses – Responsible for delivering the IP packet from the original source to the final destination, which may be on the same network or a remote network.

- Data link layer source and the destination addresses  - Responsible for delivering the data link frame from one Network Interface Card (NIC) to another NIC on the same network.

### Layer 3 Logical Address

An IP address is the network layer, or Layer 3, logical address used to deliver the IP packet from the original source to the final destination, as shown in the figure.

The IP packet contains to IP addresses:

  - Source IP Address – The IP address of the sending device, which is the original source of the packet.

  - Destination IP Address – The IP address of the receiving device, which is the final destination of the packet.

The IP addresses indicate the original source IP address and final destination IP address.  This is true whether the source and destination are on the same IP networks or different IP networks.

An IP address contains two parts:

  - Network Porton (IPv4) or Prefix (IPv6) – The left-most part of the address that indicates the network in which the IP address is a member.  All devices on the same network will have the same network portion of the address.

  - Host Portion (IPv4) or Interface ID (IPv6) – The remaining part of the address that identifies a specific device on the network.  This portion is unique for each device or interface on the network.

Note: The subnet mask (IPv4) or prefix-length (IPv6) is used to identify the network portion of an IP address from the host portion.

### Devices on the Same Network

In this example, we have a client computer, PC1, communicating with an FTP server on the same IP network.

  - Source IPv4 Address – The IPv4 address of the sending device, the client computer PC1: 192.168.1.110

  - Destination IPv4 Address – The IPv4 address of the receiving device, FTP server. 192.168.1.9

Notice in the figure that the network portion of the source IPv4 address and the network portion of the destination IPv4 address are the same and therefore; the source and destination are on the same network.

### Role of the Data Link Layer Adress – Same IP Network

When the sender and receiver of the IP packet are on the same network, the data link frame is sent directly to the receiving device.  On an Ethernet network, the data link addresses are known as Ethernet Media Access Control (MAC) addresses, as highlight in the figure.

MAC addresses are physically embedded on the Ethernet NIC

  - Source MAC address – This is the data link address, or the Ethernet MAC address, of the device that sends the data link frame with the encapsulated IP packet.  The MAC address of the Ethernet NIC of PC1 is AA:AA:AA:AA:AA:AA, written in hexadecimal notation.

  - Destination MAC address  - When the receiving device is on the same network as the sending device, this is the data link address of the receiving device.  In this example, the destination MAC address is the MAC address of the TP server. CC:CC:CC:CC:CC:CC, written in hexadecimal notation.

The frame with the encapsulated IP packet can now be transmitted from PC1 directly to the FTP server

### Role of the Network Layer Addresses

When the sender of the packet is on a different network from the receiver, the source and destination IP addresses will represent hosts on different networks.  This will be indicated by the network portion of the IP address of the destination host.

  - Source IPv4 address – The IPv4 address of the sending device, the client computer PC1: 192.1.110

  - Destination IPv4 address – The IPv4 address of the receiving device, the server, Web Server.  172.16.1.99.

Notice in the figure that the network portion of the source IPv4 address and destination IPv4 address are on different networks.

### Role of the Data Link Layer Addresses – Different IP Networks

When the sender and receiver of the IP address are on different networks, the Ethernet data ink frame cannot be sent directly to the destination host because the hose is not directly reachable in the network of the sender  The Ethernet frame must be sent to another device known as the router or default gateway.  In our example, the default gateway is R1.  R1 has an Ethernet data link address that is on the same network a PC1.  Tis allows PC1 to reach the router directly.

  - Source MAC address – The Ethernet MAC address of the sending device, PC1.  The MAC address of the Ethernet interface of PC1 is AA-AA-AA-AA-AA-AA

  - Destination MAC address – When the receiving device, the destination IP address, is on a different network from the sending device uses the Ether MAC address of the default gateway or router.  In this example, the destination MAC address is the MAC address of the R1 Ethernet interface, 11-11-11-11-11-11. This is the interface that is attached to the same network as PC1, as shown in the figure.

The Ethernet frame with the encapsulated IP packet can now be transmitted to R1.  R1 forwards the packet to the destination, Web Server.  This may mean that R1 forwards the packet to another router or directly to Web Server if the destination is on a network connected to R1.

It is important that the IP address of the default gateway be configured on each host on the local network.  All packets to a destination on remote networks are sent to the default gateway.  Ethernet MAC addresses and the default gateway are discussed in more detail in ore modules.

### Data Link Addresses

The data link Layer 2 physical address has a different role.  The purpose of the data link address is to deliver the data link frame from one network interface to another network interface on the same network.

Before an IP packet can be sent over a wired or wireless network, it must be encapsulated in a data link frame, so it can be transmitted over the physical medium.

Host to Router

Router to Router

Router to Server

As the IP packet travels from host-to-router, router-to-router and finally router-to-host, at each point along the way, the IP packet is encapsulated in a new data link frame.  Each data link frame contains the source data link address of the NIC card sending the frame, and the destination data link address of the NIC card receiving the frame.

The Layer 2, data link protocol is only used to deliver the packet from NIC-to-NIC on the same network.  The router removes the Layer 2 information as it is received on one NIC and adds new data link information before forwarding out the exit NIC on its ways towards the final destination.

The IP packet is encapsulated in a data link frame that contains the following data link information:

  - Source data link address – The physical address of the NIC that is sending the data link frame

  - Destination data link address – The physical address of the NIC that is receiving the data link frame  The address s either the next hop router of the address of the final destination device.

## Purpose of the Physical Layer

### The Physical Connection

Whether connecting to a local printer in the home or a website in another country, before any network communication can occur, a physical connection to a local network must be established.  A physical connection can be a wired connection using a cable or  wireless communication using radio waves.

The type of physical connection used depends upon the setup of the network.  For example, in many corporate offices, employees have desktop or laptop computers that are physically connect, via cable, to a shared switch.  This type of setup is wired network.  Dat is transmitted through a physical cable.

In addition to wired connections, many businesses also offer wireless connections for laptops, tablets and smartphones. With wireless devices, data is transmitted using radio waves.  Wireless connectivity is common as individuals and businesses alike discover its advantages.  Devices on a wireless network must be connected to a wireless access point (AP) or wireless router like the one shown in the figure.

These are the components of an access point:

- The wireless antennas (These are embedded inside the router version shown in the figure)

- Several ethernet switchports

- An internet port

Similar to a corporate office, most homes offer both wired and wireless connectivity to the network.  This figure shows a home router and a laptop connecting to the local area network (LAN)

Network Interface Cards

Network Interface Cards (NICs) connect a device to a network.  Ethernet NICs are used for a wired connection, as shown in the figure, whereas wireless local area network (WLAN) NICs are used for wireless. An end-user may include one or both types of NICs.  A network printer, for example, may only have an Ethernet NIC and therefore, must connect to a network using an Ethernet cable.  Other devices, such as tablets and smartphones, might only contain a WLAN NIC and must use a wireless connection.

### The Physical Layer

The OSI physical layer provides the means to transport the bits that make up a data link layer frame across the network media.  This layer accepts a complete frame from the data link layer and encodes it as a series of signals that are transmitted to the local media.  The encoded bits that comprise a frame are received by either an end device or an intermediate device.

The physical layer encodes the frames and creates the electrical, optical or radio wave signals that represent the bits in each frame  These signals are then sent over the media, one at a time.

The destination node physical layer retrieves these individual signals from the media, restores them to their bit representation and passes the bits up to the data link layer as complete frame.

## Physical Layer Characteristics

### Physical Layer Standards

In the previous topic, you gained a high-level overview of the physical layer and its place in a network.  This topic dives a bit deeper into the specifics of the physical layer.  This includes the components ad the media used to build a network, as well as the standards that are required so that everything works together.

The protocols and operations of the upper OSI layers are performed using software designed by software engineers and computer scientists.  The services and protocols in the TCP/IP suite are defined by the Internet Engineering Task Forces (IETF).

The physical layer consists of electronic circuitry, media and connectors developed by engineers.  Therefore, it is appropriate that the standards governing this hardware are defined b the relevant electrical and communications engineering organizations.

There are many different international and national organization, regulatory government organization, and private companies involved in establishing and maintaining physical layer standards. For instance, the physical layer hardware, media, encoding an signaling standards are defined and government by these standard organizations:

- International Organization for Standardization (ISO)

- Telecommunications Industry Association/Electronic Industries Association (TIA/EIA)

- International Telecommunications Union (ITU)

- American National Standards Institute (ANSI)

- Institute of Electrical And Electronics Engineers (IEEE)

- National Telecommunication Regulatory Authorities including the Federal Communication Commission (FOC) in the USA and the European Telecommunication Standards Institute (ETSI)

In addition to these, there are often regional cabling standard groups such as CSA (Canadian Standards Association), CENELEC (European Committee for Electronical Standardization), and JSA/JIS (Japanese Standards Association), which develop local specifications.

### Physical Components

The physical layer standards address three functional areas:

  - Physical Components

  - Encoding

  - Signaling

The physical components are the electronic hardware devices, media and other connectors that transmit the signals that represent the bits.  Hardware components such as NICs, interface and connectors, cable designs are all specified in standards associate with the physical layer.  The various ports and interfaces on a Cisco 1941 router are also examples of physical components with specific connectors and pinouts resulting from standard

### Encoding

Encoding or line encoding is a method of converting a stream of data bits into a predefined “code”.  Codes are groupings of bits used to provide a predictable pattern that can be recognized by both the sender and the receiver.  In other words, encoding is the method or pattern used to represent digital information.  This is similar to how Mores code encodes a message using  series of dots and dashes.

For example, Manchester encoding represents a 0 bit by a high to ow voltage transition, and a 1 bit is represented as a low to high voltage transition.  An example of Manchester encoding I illustrated in the figure.  The transition occurs at the middle of each bit period.  This type of encoding is used in 10 Mbps Ethernet.  Fater data rates require more complex encoding.  Manchester encoding is used in older Ethernet standards such as 10BASE-T.  Ethernet 100BASE-TX use 4B/5B encoding and 1000BASE-T uses 8B/10B encoding.

### Signaling

The physical layer must generate the electrical, optical or wireless signals the represent the “1” and the “0” on the media.  The way that bits are represented is called the signaling method.  The physical layer standards must define what type of signal represents a “1” and what type of signal represents a “0”.  This can be as simple as change in the level of the electrical or optical pulse.  For example, a long pulse might represent a 1 whereas a short pulse might represent a 0.

This is similar to the signaling method used in Morse code, which may use a series of on-off tones, lights or clicks to send text over telephone wires or between ships at sea.

### Encoding

Different physical media support the transfer of bits at different rates.  Data transfer is usually discussed in terms of bandwidth. Bandwidth is the capacity at which a medium can carry data  Digital bandwidth measures the amount of data that can flow from one place to another in a given amount of time.  Bandwidth is typically measured in kilobits per second (kbps), megabits per second (Mbps) or gigabits per second (Gbps).  Bandwidth is sometimes thought of as a speed that bits travel, however this is not accurate  For example, in both 10Mbps and 100Mbps Ethernet, the bits are sent at the speed of electricity.  The difference is the number of bits that are transmitted per second.

A combination of factors determines the practical bandwidth of a network:

- The properties of the physical media

- The technologies chosen for signaling and detecting network signals.

Physical media properties, current technologies and the laws of physics all play a tole in determining the available bandwidth.

The table shows the commonly used units of measure for bandwidth

### Bandwidth Terminology

Terms used to measure the quality of bandwidth include:

  - Latency

  - Throughput

  - Goodput

Latency refers to the amount of time, including delays, for data to travel from one given point to another.

In an internetwork, or a network with multiple segments, throughput cannot be faster than the slowest link in the path from source to destination.  Even if all, or most, of the segments have high bandwidth, it will only take one segment in the path with a low throughput to create a bottleneck in the throughput of the entire network.

Throughput is the measure of the transfer of bits across the media over a given period of time.

Due to a number of factors, throughput usually does not match the specified bandwidth in physical layer implementations.  Throughput is usually lower than the bandwidth  There are many factors that influence throughput:

  - The amount of traffic

  - The type of traffic

  - The latency created by the number of network devices encountered between source and destination

There are many only speed tests that can reveal the throughput of an internet connection.

There is a third measurement to assess the transfer of usable data; it is known as goodput.  Goodput is the measure of usable data transferred over a given period of time.  Goodput is throughput minus traffic overhead for establishing sessions, acknowledgments, encapsulations and retransmitted bits.  Goodput is always lower than throughput, which is generally lower than bandwidth

## Copper Cabling

### Characteristics of Copper Cabling

Copper cabling is the most common type of cabling used in networks today.  In fact, copper cabling is not just one type of cable.  There are three different types of copper cabling that are each used in specific situations.

Networks use copper media because it is inexpensive, easy to install, and has lower resistance to electrical current.  However, copper media is limited by distance and signal interference.

Data is transmitted on copper cables as electrical pulses.  A detector in the network interface of a destination device must receive a signal that can be successfully decoded to match the signal sent.  However, the farther the signal travels, the more it deteriorates.  This is referred to as signal attenuation.  For this reason, all copper media mut follow strict distance limitations as specified by the guiding standards.

The timing and voltage values of the electrical pulses are also susceptible to interference from to sources:

- Electromagnetic Interference (EMI) or radio frequency interference (RFI) – EMI and RFI signals can distort and corrupt the fata signals being carried by copper media.  Potential sources of EMI and RFI include radio wave and electromagnetic devices, such as fluorescent lights or electric motors

- Crosstalk – Crosstalk is disturbance caused by the electric or magnetic fields of a signal on one wire to the signal in an adjacent wire.  In telephone circuits, crosstalk can result in hearing part of another voice conversation from an adjacent circuit.  Specifically, when an electrical current flows through a wire, it creates a small, circular magnetic field around the wire, which can be picked up by an adjacent wire.

The figure shows how data transmission can be affected by interference.

To counter the negative effect of EMI and RFI, some types of copper cables are wrapped in metallic shielding and require proper grounding connections.

To counter the negative effects of crosstalk, some types of copper cables have opposing circuit wire pairs twisted together, which effectively cancels the crosstalk.

The susceptibility of copper cables to electronic noise can also be limited using these recommendations:

- Selecting the cable type or category most suited to a given networking environment

- Designing a cable infrastructure to avoid known and potential sources of interference in the building structure

- Using cabling techniques that include the proper handling and termination of the cables.

### Types of Copper Cabling

There are three main types of copper media used in networking

### Unshielded Twisted-pair (UTP)

Unshielded twisted pair (UT) cabling is the most common networking media.  UTP cabling, terminated with RJ-45 connectors, is used for interconnecting network hosts with intermediary networking devices, such as switches and routers.

In LANs, UTP cable consists of four pairs of color-coded wires that have been twisted together and then encased in a flexible plastic sheath that protects from minor physical damage.  The twisting of wires helps protect against interference from other wires.

As seen in the figure, the color codes identify the individual pair and wires and aid in cable termination.

### Shielded Twisted-pair (STP)

Shielded twisted-pair (STP) provides better noise protection than UTP cabling.  However, compared to UTP cable STP cable is significantly more expensive and difficult to install.  Like UTP cable, STP uses an RJ-45 connector.

STP cables combine the techniques of shielding to counter EMI and RFI, and wire twisting to counter crosstalk.  To gain the full benefit of the shielding, STP cables are terminated with special shielded STP data connectors.  If the cable is improperly grounded, the shield may act as an antenna and pick up unwanted signals.

The STP cable shown uses four main pairs of wires, each wrapped in a foil shield, which are then wrapped in an overall metallic braid or foil

### Coaxial Cable

Coaxial Cable, or “co-ax” for short, gets its name from the fact that there are two conductors that share the same axis.  As shown in the figure, coaxial cable consists of the following:

  - A copper conductor is used to transmit the electronic signals.

  - A layer of flexible plastic insulation surrounds a copper conductor

  - The insulating material is surrounded in a woven copper braid, or metallic foil, that acts as the second wire on the circuit and as a shield for the inner conductor  This second layer, or shield, also reduces the amount of outside electromagnetic interference.

  - The entire cable is covered with a cable jacket to prevent minor physical damage.

There are different types of connectors used with coax cable.  The Bayonet Neill-Concelman (BNC), N type, and F type connectors are shown in the figure.

Although UTP cable has essentially replaced coaxial cable in modern Ethernet installations, the coaxial cable design is used in the following situations:

  - Wireless installations – Coaxial cables attach antennas to wireless devices.  The coaxial cable carries radio frequency (RF) energy between the antennas and the radio equipment.

  - Cable internet installations – Cable service provide internet connectivity to their customers by replacing portions of the coaxial cable and supporting amplification elements with fiber-optic cable.  However, the wiring inside the customer’s premises is still coax cable.

## UTP Cabling

### Properties of UTP Cabling

In the previous topic, you learned a bit about unshielded twisted-pair (UTP) copper cabling.  Because UTP cabling is the standard for use in LANs, this topic goes into detail about its advantages and limitations, and what can be done to avoid problems.

When used as a networking medium, UTP cabling consists of four pairs of color-coded copper wires that have been twisted together and then encased in a flexible plastic sheath.  Its small size can be advantageous during installation.

UTP cable does not use shielding to counter the effects of EMI and RFI.  Instead, cable designers have discovered other ways that they can limit the negative effect of crosstalk:

  - Cancellation – Designers now pair wires in a circuit.  When two wires in an electrical circuit are placed closed together, their magnetic fields are the exact opposite of each other.  Therefore, the two magnetic fields cancel each other and also cancel out any outside EMI and RFI signals.

  - Varying the number of twists per wire pair – To further enhance the cancellation effect of paired circuit wires, designers vary the number of twists of each wire pair in a cable.  UTP cable must follow precise specifications governing how many twists or braids are permitted per meter (3.28 feet) of cable.  Notice in the figure that the orange/orange white pair is twisted less than the blue/blue white pair.  Each coloured pair is twisted a different number of times.

UTP cable relies solely on the cancellation effect produced by the twisted wire pairs to limit signa degradation and effectively provide self-shielding for wire pairs within the network media.

### UTP Cabling Standards and Connectors

UTP cabling conforms to the standards established jointly by the TIA/EIA.  Specifically, TIA/EIA-568 stipulates the commercial cabling standards for LAN installations and is the standard most commonly used in LAN cabling environments. Some of the elements defined are as follows:

  - Cable types

  - Cable lengths

  - Connectors

  - Cable termination

  - Methods of testing cable

The electrical characteristics of copper cabling are defined by the Institute of Electrical and Electronics Engineers (IEEE).  IEEE rates UTP cabling according to its performance.  Cables are placed into categories based on their ability to carry higher bandwidth rates.  For example, Category 5 cable is used commonly in 100BASE-TX Fast Ethernet installations.  Other categories included Enhanced Category 5 cable, Category 6 and Category 6a.

Cables in higher categories are designed and constructed to support higher data rates.  As new gigabit speed Ethernet technologies are being developed and adopted, Category 5e is now the minimally acceptable cable type, with Category 6 being the recommended type for new building installations.

The figure shows three categories of UTP cable:

  - Category 3 was originally used for voice communications over voice lines but later use for data transmission.

  - Category 5 and 5e is used for data transmission.  Category 5 supports 100Mbps and Category 5e supports 1000Mbps

  - Category 6 has an added separator between each wire pair to support higher speeds.  Category 6 supports up to 10Gbps

  - Category 7 also supports 10Gbps

  - Category 8 supports 40Gbps

UTP cable is usually terminated with an RJ-45 connector.  The TIA/EIA-568 standard describes the wire color codes to pin assignments (pinouts) for Ethernet cables.

As shown in the figure, the RJ-45 connector is the male component, crimped at the end of the cable.

The socket, shown in the figure, is the female component of a network device, wall, cubicle partition outlet, or patch panel.  When terminated improperly, each cable is a potential source of physical layer performance degradation.

This figure shows an example of a badly terminated UTP cable.  This bad connector has wires that are exposed, untwisted and not entirely covered by the sheath.

The next figure shows a properly terminated UTP cable. It is a good connector with wires that are untwisted only to the extent necessary to attach the connector

### Straight-through and Crossover UTP

Different situations may require UTP cables to be wired according to different wiring conventions.  This means that the individual wires in he cable have to be connected in different orders to different set of pins in the RJ-45 connectors.

The following are the main cable types that are obtained by using specific wiring conventions:

  - Ethernet Straight-through – The most common type of networking cable.  It is commonly used to interconnect a host to a witch and a switch to a router.

  - Ethernet Crossover – A cable used to interconnect similar devices.  For example, to connect a switch to a switch, a host to a host, or a router to a router.  However, crossover cables are now considered legacy as NICs use medium-dependent interface crossover (auto-MDIX) to automatically detect the cable type and make the internal connection.

Note: Another type of cable is a rollover cable, which is Cisco proprietary.  It is used to connect a workstation to a router or switch console port.

Using a crossover or straight-through cable incorrectly between devices may not damage the device, but connectivity and communication between the devices will not take place.  This is a common error and checking that the device connections are correct should be the first troubleshooting action if connectivity is not achieved.

The figure identifies the individual wire pairs for the T658A and T568B standards.

The table shows the UTP cable type, relate standards and the typical application of these cables.

| Ethernet Straight-through | Both ends T568A or both ends T568B | Connects a network host to a network device such as a switch or hub |
| --- | --- | --- |
| Ethernet Crossover | One end T568A, other end T568B | Connects two network hosts. Connects two network intermediary devices (switch to switch or router to router) |
| Rollover | Cisco proprietary | Connects a workstation serial port to a router console port, using an adapter |

## Fiber-Optic Cabling

### Properties of Fiber-Optic Cabling

As you have learned, fiber-optic cabling is the other type o cabling used in networks.  Because it is expensive, it is not as commonly used at the various types of copper cabling.  But fiber-optic cabling has certain properties that make it the best option in certain situations, which you will discover in this topic.

Optical fiber cable transmits data over longer distances and at higher bandwidths than any other networking media.  Unlike copper wires, fiber-optic cable can transmit signals with less attenuation and is complete immune to EMI and RFI.  Optical fiber is commonly used to interconnect network devices.

Optical fiber is a flexible, but extremely thin, transparent strand of very pure glass, not much bigger than a human hair.  Bits are encoded on the fiber as light impulses.  The fiber-optic cable acts as a waveguide or “light-pipe” to transmit light between the two ends with minimal loss of signal.

As an analogy, consider an empty paper towel roll with the inside coated like a mirror.  It is a thousand meters in length, and a small laser pointer is used to send Morse code signals at the speed o light.  Essentially, that is how a fiber-optic cable operates, except that it is smaller in diameter and uses sophisticated light technologies.

### Types of Fiber Media

Fiber-optic cables are broadly classified into two types:

  - Single-mode fiber (SMF)

  - Multimode fiber (MMF)

Single-Mode Fiber

SMF consists of a very small core and uses expensive laser technology to send a single ray of light, as shown in the figure.  SMF is popular in long-distance situations spanning hundreds of kilometers, such as those required in long haul telephony and cable TV applications.

Multimode Fiber

MMF consists of a larger core and uses LED emitters to send light pulses.  Specifically, light from an LED enters the multimode fiber at different angles, as shown in the figure.  MMFs are popular in LANs because they can be powered by low-cost LEDs.  It provides bandwidth up to 10Gbps over link lengths o up to 550 meters.

One of the highlighted differences between MMF and SMF is the amount of dispersion.  Dispersion refers to the spreading out of a light pulse over time.  Increased dispersion means increased loss of signal strength.  MMF has a greater dispersion than SMF.  That is why MMF can only travel up to 500 meters before signal loss

### Fiber-Optic Cabling Usage

Fiber-optic cabling is now being used in four types of industry:

  - Enterprise Networks – Used for backbone cabling applications and interconnecting infrastructure devices

  - Fiber-to-the-home (FTTH) – Used to provide always-on broadband services to homes and small businesses

  - Long-Haul Networks – Used by service providers to connect countries and cities

  - Submarine Cable Networks – Used to provide reliable high speed, high-capacity solutions capable of surviving in harsh undersea environments at up to transoceanic distances.  Search the internet for “submarine cables telegeography map” to view various maps online.

Our focus in this course is the use of fiber within the enterprise.

### Fiber-Optic Connectors

An optical-fiber connector terminates the end of an optical fiber.  A variety of optical-fiber connectors are available.  The main differences among the types of connectors are dimensions and methods of coupling.  Businesses decide on the type of connectors that will be used, based on their equipment.

Note: Some switches and routers have ports that support fiber-optic connectors through a small form-factor pluggable (SFP) transceiver.  Search the internet for various types of SFPs

Straight Tip (ST) Connectors were one of the first connector types used.  The connector locks securely with a “twist-on/twist-off” bayonet-style mechanism.

Subscriber Connector (SC) connectors are sometimes referred to as “square connectors” or “standard connectors.”  They are widely adopted LAN and WAN connector that uses a push-pull mechanism to ensure positive insertion.  This connector type is used with multimode and single-mode fiber.

Lucent Connector (LC) Simplex Connectors are a smaller version of the SC connector.  These are sometimes called little or local connector and are quickly growing in popularity due to their smaller size.

Duplex Multimode LC Connectors  are similar to an LC simplex connector but uses a duplex connector.

Until recently, light could only travel in one direction over optical fiber.  Two different fibers were required to support the full duplex operation.  Therefore, fiber-optic patch cables bundle together two optical fiber cables and terminate them with a pair of standard, single-fiber connectors. Some fiber connectors accept both the transmitting and receiving fibers in a single connector known as a duplex connector, as shown in the Duplex Multimode LC Connector figure. BX standards such as 100BASE-BX use different wavelengths for sending and receiving over a single fiber.

### Fiber Patch Cords

Fiber patch cords are required for interconnecting infrastructure devices.  The use of color distinguishes between single-mode and multimode cords.  A yellow jacket is for single-mode fiber cables and orange (or aqua) for multimode fiber cables.

### Fiber versus Copper

There are many advantages to using fiber-optic cable compared to copper cables.  The table highlights some of these differences.

At present, in most enterprise environments, optical fiber is primarily used as backbone cabling for high-traffic, point-to-point connections between data distribution facilities.  It is also use for the interconnection of buildings in multi-building campuses.  Because fiber-optic cables do not conduct electricity and have a low signal loss, they are will suited for those uses.

## Wireless Media

### Properties of Wireless Media

You may be taking this course using a tablet or a smart phone.  This is only possible due to wireless media, which is the third way to connect to the physical layer of a network.

Wireless media carry electromagnetic signals that represent the binary digits of data communications using radio or microwave frequencies.

Wireless media provide the greatest mobility options of all media, and the number of wireless-enabled devices continues to increase.  Wireless is now the primary way users connect to home and enterprise networks.

These are some of the imitations of wireless:

  - Coverage Area – Wireless data communication technologies work well in open environments.  However, certain construction materials used in buildings and structures, and the local terrain, will limit effective coverage.

  - Interference – Wireless is susceptible to interference and can be disrupted by such common devices as household cordless phones, some types of fluorescent lights, microwave ovens and other wireless communications.

  - Security – Wireless communications coverage requires no access to a physical strand of media.  Therefore, devices and users, not authorized for access to the network, con gain access to the transmission.  Network security is a major component of wireless network administration.

  - Shared Medium – WLANs operate in half-duplex, which means only one device can send or receive at a time.  The wireless medium is shared amongst all wireless users.  Many users accessing the WLAN simultaneously results in reduced bandwidth for each user

Although wireless is increasing in popularity for desktop connectivity, copper and fiber are the most popular physical layer media for deployment of intermediary network devices such as routers and switches.

### Types of Wireless Media

The IEEE and telecommunications industry standards for wireless data communications cover both the data link and physical layers.  In each of these standards, physical layer specifications are applied to area that include the following:

  - Data to radio signal encoding

  - Frequency and power of transmission

  - Signal reception and decoding requirements

  - Antenna design and construction

These are the wireless standards:

  - Wi-Fi (IEEE 802.11) – Wireless LAN (WLAN) technology, commonly referred to as Wi-Fi.  WLAN uses a contention-base protocol known as carrier sense multiple access/collision avoidance (CSMA/CA).  The wireless NIC must first listen before transmitting to determine if the radio channel is clear.  If another wireless device is transmitting, then the NIC must wait until the channel is clear.  Wi-Fi is a trademark of the Wi-Fi Alliance.  Wi-Fi is used with certified WLAN devices based on the IEEE 802.11 standards.

  - Bluetooth (IEEE 802.15) – This is a wireless personal area network (WPAN) standard, commonly known as “Bluetooth.”  It uses a device pairing process to communicate over distances from 1 – 100 meters.

  - WiMAX (IEEE 802.16) – Commonly known as Worldwide Interoperability for Microwave Access (WiMAX), this wireless standard uses a point-to-multipoint topology to provide wireless broadband access.

  - Zigbee (IEEE 802.15.14) –  Zigbee is a specification used for low-data rate, low-power communications.  It is intended for application that require short-range, low data-rates and long battery life.  Zigbee is typically use for industrial and Internet of Things (IoT) environments such as wireless light switches and medical device data collection

Note: Other wireless technologies such as cellular satellite communications can also provide data network connectivity.  However, these wireless technologies are out of scope for this module.

### Wireless LAN

A common wireless data implementation is enabling devices to connect wirelessly via a LAN.  In general, a WLAN requires the following network devices:

  - Wireless Access Point (AP) – These concentrate the wireless signals from users and connect to the existing copper-based network infrastructure, such as Ethernet.  Home and small business wireless routers integrate the functions of a router, switch and access point into one device, as shown in the figure.

  - Wireless NIC adaptors – These provide wireless communication capability to network hosts

As the technology has developed, a number of WLAN Ethernet-based standards have emerged. When purchasing wireless devices, ensure compatibility and interoperability.

The benefits of a wireless data communications technologies are evident, especially the saving on costly premises wiring and the convenience of host mobility.  Network administrators must develop and apply stringent security policies and processes to protect WLANs from unauthorized access and damage.

## Binary Number Systems

### Binary and IPv4 Addresses

IPv4 addresses begin as binary, a series of only 1s and 0s.  These are difficult to manage, so network administrators must convert them to decimal. This topic shows you a few ways to do this.

Binary is a numbering system that consists of the digits 0 and 1 called bits.  In contrast, the decimal numbering system consists of 10 digits consisting of the digits 0 -9.

Binary is important for us to understand because hosts, servers and network devices use binary addressing.  Specifically, they use binary IPv4 addresses as shown in the figure, to identify each other.

Each address consists of a string of 32 bits, divided into four sections called octets.  Each octet contains 8 bits ( or 1 byte) separated with a dot.  For example, PC1 in the figured is assigned IPv4 address 11000000.10101000.00001010.00000001. Its default gateway address would be that of R! Gigabit Ethernet interface 11000000.10101000.00001010.00000001.

Binary works well with hosts and network devices.  However, it is very challenging for humans to work with.

For each of use by people, IPv4 addresses are commonly expressed in dotted decimal notation.  PC1 is assigned the IPv4 address 192.168.10.10 and its default gateway address is 192.168.10.1, a shown in the figure.

For a solid understanding of networking addressing, it is necessary to know binary addressing and gain practical skills converting between binary and dotted decimal IPv4 addresses.  This section will cover how to convert between base two (binary) and base 10 (decimal) numbering systems.

### Binary Positional Notation

Learning to convert binary to decimal requires an understanding of positional notation.  Positional notation means that a digit represents different values depending on the “position” the digit occupies in the sequence of numbers.  You already know the most common numbering system; the decimal (base 10) notation system.

The decimal positional notation system operates as described in the table.

The following bullets describe each row on the table

  - Row 1 – Radix is the number base.  Decimal notation is based on 10, therefore the radix is 10

  - Row 2 – Position in number considers the position of the decimal number starting with, from right to let, 0 (1st position), 1 (2nd position), 2 (3rd position), 3 (4th position).  These numbers also represent the exponential value used to calculate the positional value in the 4th row.

  - Row 3 – Calculates the positional value y taking the radix and raising it by the exponential value of its position in Row 2

  - Row 4 – Positional value represents units of thousands, hundreds, tens and ones.

The use the positional system, match a given number to its positional value.  The example in the table illustrates how positional notation is used with the decimal number 1234.

In contrast, the binary positional notation operates as described in the table:

The following bullets describe each row of the table

  - Row 1 = Radix is the number base  Binary notation is based on 2, therefore the radix is 2.

  - Row 2 – Position in number considers the position of the binary number starting with, from right to left, 0 (1st position), 1 (2nd position), 2 (3rd position), 3 (4th position).  These numbers also represent the exponential value used to calculate the positional value in the 4th row.

  - Row 3 – Calculates the positional value by taking the radix and raising it by the exponential value of it’s position in Row 2

  - Row 4 – Positional value represents units of ones, twos, fours, eights, etc.

The example in the table illustrates how a binary number 11000000 corresponds to the number 192.  If the binary number had been 10101000, then the corresponding decimal number would be 168.

### Convert Binary to Decimal

To convert a binary IPv4 address to its dotted decimal equivalent, divide the IPv4 address into four 8-bit octets.  Next apply the binary positional value to the first octet binary number are calculate accordingly.

For example, consider that 11000000.10101000.00001011.00001010 is the binary IPv4 address of a host.  To convert the binary address to decimal, start with the first octet, as shown in the table.  Enter the 8bit binary number under the positional value of row 1 and then calculate to produce the decimal umber 192.  This number goes into the first octet of the dotted decimal notation.

Next, convert the second octet of 10101000 as shown in the table.  The resulting decimal value is 168, and it goes into the second octet.

Convert the third octet of 00001011 as shown in the table.

Convert the fourth octet of 00001010 as shown in the table.  This completes the IP address and produces 192.168.11.10

### Decimal to Binary Conversion

It is also necessary to understand how to convert dotted decimal IPv4 address to binary.  A useful tool in the binary positional value table.

Imagine our IPv4 address is 192.168.10.11

Break 192 down into an octet:

| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Check each position starting at 128 and work left to right, to the 1 position.

Starting from 128, is the decimal number of the octet  is <=  or > than 128?

If it is > (greater than) , then enter a 0 in the 128 positional value.

If is <= (Less than or equal to), then enter a 1 in the 128 positional value and then subtract 128 from the decimal value.

Since 128 is less than 192, we can place a 1 in the 128 positional value, like so

| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |

We then subtract 128 from 192 to get us 64.

Then we check if the new decimal number of 64 (192 – 128 ) is <= or > than 64.

Since it is, we will place a “1” under the 64 positional value. Like so:

| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  |  |  |  |  |  |

Then we repeat the steps of subtracting 64 from our decimal number.  64 – 64.  This leaves us with 0.  AS we have no longer anything left in our decimal, we will place 0s in the remaining positional values.  Like so

| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

This means the binary value of our first octet, 192 is 11000000.

We repeat the above until we have converted all 4 octets.

### IPv4 Addresses

As mentioned in the beginning of this topic, routers and computers only understand binary, while humans work in decimal.  It is important for you to gain a thorough understanding of these two numbering systems and how they are used in networking.

## Hexadecimal Number Systems

### Hexadecimal and IPv6 Addresses

Now you know how to convert binary to decimal and decimal to binary.  You need that skill to understand IPv4 addressing in your network.  But you are just as likely to be using IPv6 addresses in your network.  To understand IPv6 addresses, you must be able to convert hexadecimal to decimal and vice versa

Just as decimal is a base ten number system, hexadecimal is a base sixteen system.  The base sixteen number system uses the digits 0 – 9 and the letters A to F.  The figure shows the equivalent decimal and hexadecimal values for binary 0000 to 1111

Binary and hexadecimal work well together because it is easier to express a value as a single hexadecimal digit that as four binary bits.

The hexadecimal numbering system is used in networking to represent IP Version 6 addresses and Ethernet MAC addresses.

IPv6 addresses are 128 bits in length and every 4 bits is represented by a single hexadecimal digit, for a total of 32 hexadecimal values.  IPv6 addresses are not case-sensitive and can be written either in lowercase or uppercase.

As shown in the figure, the preferred format for writing IPv6 address is x:x:x:x:x:x:x:x, with each “x” consisting of four hexadecimal values.  When referring to 8 bits of an IPv4 address we use the term octet.  In IPv6, a hextet is the unofficial term used to refer to a segment o 16 bits or our hexadecimal value.  Each “x” is a single hextet, 16 its or four hexadecimal digits.

The sample topology in the figure displays IPv6 hexadecimal addresses

### Decimal to Hexadecimal Conversions

Converting decimal numbers to hexadecimal values is straightforward.  Follow the steps listed:

- Convert the decimal number to 8-bit binary strings

- Divide the binary strings in groups of four, starting from the rightmost position

- Convert each binary number into the equivalent hexadecimal digit.

The example provides the steps for converting 168 to a hexadecimal.

- 168 is binary 10101000

- 10101000 in two groups of 4 binary digits is 1010 and 1000

- 1010 is hex A and 1000 is hex B

Answer: 168 is AB in hexadecimal.

### Hexadecimal to Decimal Conversions

Converting hexadecimal numbers to decimal values is also straightforward. Follow the steps listed:

- Convert the hexadecimal number to 4-bit binary strings

- Create 80bit binary grouping starting from the rightmost position

- Convert each 80bit binary grouping into the equivalent decimal digit.

This example provides the steps for converting D2 to decimal

- D2 in 4-bit binary strings is 1101 and 0010

- 1101 and 0010 is 11010010 in an 8-bit grouping.

- 11010010 in binary is equivalent to 210 in decimal

Answer: D2 in hexadecimal is 210 in decimal.

# Data Link Layer

## Purpose of the Data Link Layer

The Data Link Layer is responsible for communications between end-device network interface cards (NICs).  It allows upper layer protocols to access the physical layer media and encapsulates Layer 3 packets (Iv4 and IPv6) into Layer 2 Frames.  It also performs error detection and rejects corrupted frames.

IEEE 802 LAN/MAN standards are specific to the type of network (Ethernet, WLAN, WPAN, etc.)

The Data Link Layer consists of two sublayers; the Logical Control (LLC) and the Media Access Control (MAC).

  - The LLC sublayer communicates between the networking software at the upper layers and the device hardware at the lower layers.

  - The MAC sublayer is responsible for data encapsulation and media access control.

Packets exchanged between nodes may experience numerous data link layers and media transitions.  At each “hop along he path,” a router performs our basic Layer 2 functions:

  - Accepts a frame from the network medium

  - De-encapsulates the frame to expose encapsulated packet.

  - Re-encapsulates the packet into a new frame.

  - Forwards the new frame on the medium of the net network segment.

## Topologies

The topology of a network is the arrangement and relationship of the network devices and the interconnections between them.  There are two types of topologies when describing networks:

  - Physical topology – Shows physical connections and how devices are interconnected.

  - Logical topology – Identifies the virtual connections between devices using device interfaces and IP addressing schemes.

There are three types of common physical WAN topologies:

  - Point-to-point – The simplest and most common WAN topology.  Consists of a permanent link between two endpoints.

    - Physical point-to-point topologies directly connect two nodes.

    - The nodes may not share the media with other hosts.

    - Because all frames on the media can only travel to or from the two nodes, point-to-point WAN protocols can be very simple.

  - Hub and spoke – Similar to a “star” topology, where a central site interconnects branch sites through point-to-point links.

  - Mesh – Provides high availability but requires every end system to be connected to every other end system.

End devices on LANs are typically interconnected using a “star” or “extended star” topology.  Star and extended star topologies are easy to install, very scalable and easy to troubleshoot.

Early Ethernet and Legacy Token Ring technologies provide two additional topologies:

  - Bus –  All end systems chained together and terminated at each end.

  - Ring – Each end system s connection to its respective neighbors to form a ring.

### Half-duplex Communications

- Only allows one device to send or receive at a time on a shared medium.

- Used on WLAN and legacy bus topologies with Ethernet hubs

  - Contention-based Access – All nodes operating in half-duplex, competing for use of the medium.

    - Carrier sense multiple access with collision detection (CSMA/CD) as used on legacy bus-topology Ethernet

    - CSMA/CD

      - Used by legacy Ethernet LANs

      - Operates in half-duplex mode where only one device sends or receives at a time.

      - Uses a collision detection process to govern when a device can send and what happens if multiple devices send at the same time.

      - CSMA/CD collision detection process

        - Devices transmitting simultaneously will result in a signal collision on the shared media.

        - Devices detect the collision.

        - Devices wait a random period of time and retransmits data

    - Carrier sense multiple access with collision avoidance (CSMA/CA) as used on Wireless LANs.

    - CSMA/CA

      - Used by IEEE 802.11 WLANs

      - Operates in half-duplex mode where only one device sends or receives at a time.

      - Uses a collision avoidance process to govern when a device can send and what happens if multiple devices send at the same time.

      - CSMA/CA collision avoidance process:

        - When transmitting, devices also include the time duration needed for the transmission.

        - Other devices on the shared medium receive the time duration information and know how long the medium will be unavailable.

### Full-duplex Communications

- Allows both devices to simultaneously transmit and receive on a shared medium.

- Ethernet switches operate in full-duplex mode.

### Controlled access

- Deterministic access where each node has its own time on the medium.

- Used in legacy networks such as Token Ring and ARCNET.

## Data Link Frame

Data is encapsulated by the data link layer with a header and a trailer to form a frame.  A data link frame has three parts:

    - Header

    - Data

    - Trailer

The fields of the header and trailer vary according to data link layer protocol.

The amount of control information carried with in the frame varies according to access control information and logical topology.

### Frame Field

### Layer 2 Addresses

The Layer 2 Address is also referred to as a physical address.  It is contained in the frame header and is used only for local delivery of a frame on the link. It is updated by each device that forwards the frame.

The logical topology and physical media determine the data link protocol used:

- Ethernet

- 802.11 Wireless

- Point-to-Point (PPP)

- High0Level Data Link Control (HDLC)

- Frame-Relay

Each protocol performs media access control for specified logical topologies.

# Ethernet Switching

## Ethernet Frames

### Ethernet Encapsulation

Ethernet is one of two LAN technologies used today, with the other being wireless LANs (WLANs).  Ethernet uses wired communications, including twisted pair, fiber-optic links and coaxial cables.

Ethernet operates in the data link layer and the physical layer.  It is a family of networking technologies defined in the IEEE 802.2 and 802.3 standards.  Ethernet supports data bandwidth of the following:

- 10 Mbps

- 100 Mbps

- 1000 Mbps (1 Gbps)

- 10 000 Mbps (10 Gbps)

- 40 000 Mbps (40 Gbps)

- 100 000 Mbps (100 Gbps)

As shown in the figure, Ethernet standards define both the Layer 2 protocols and the layer 1 technologies.

### Data Link Sublayers

IEEE 802 LAN/MAN protocols, including Ethernet, use the following two separate sublayers of the data link layer to operate.  They are the Logical Link Control (LLC) and the Media Access Control (MAC), as shown in the figure below.

Recall that LLC and MAC have the following roles in the Data Link Layer:

  - LLC Sublayer – This IEEE 802.2 sublayer communicates between the networking software at the upper layers and the device hardware at the lower layers.  It places information in the frame that identifies which network layer protocol is being used for the frame.  This information allows multiple Layer 3 protocols, such as IPv4 and IPv6 to use the same network interface and media.

  - MAC Sublayer – This sublayer (IEEE 802.3, 802.11, or 802.15, for example) is implemented in hardware and is responsible for data encapsulation and media access control.  It provides data link layer addressing and is integrated with various physical layer technologies.

### MAC Sublayer

The MAC sublayer is responsible for data encapsulation and access the media.

IEEE 802.3 data encapsulation includes the following:

  - Ethernet frame -  This is the internal structure of the Ethernet frame

  - Ethernet Addressing – The Ethernet frame includes both a source and destination MAC address to deliver the Ethernet frame from Ethernet NIC to Ethernet NIC on the same LAN

  - Ethernet Error detection – The Ethernet frame includes a frame check sequence (FCS) trailer used for error detection.

As shown in the figure, the IEEE 802.3 MAC sublayer includes the specifications for different Ethernet communications standards over various types of media including copper and fiber.

Recall that legacy Ethernet using a bus topology or hubs, is a shared, half-duplex medium.  Ethernet over a half-duplex medium uses a contention-based access method, carrier sense multiple access/collision detection (CSMA/CS).  This ensures that only one device is transmitting at a time.  CSMA/CD allows multiple devices to share the same half-duplex medium, detecting a collision when more than one device attempts to transmit simultaneously.  It also provides a back-off algorithm for retransmission.

Ethernet LANs of today use switches that operate in full-duplex.  Full-duplex communications with Ethernet switches do not require access control through CSMA/CD.

### Ethernet Frame Fields

The minimum Ethernet frame size is 64 bytes, with an expected maximum of 1518 bytes.  This includes all bytes from the destination MAC address field through the frame check sequence (FCS) field.  The preamble field is not included when describing the size of the frame.

Note: The frame size may be larger if additional requirements are included, such as VLAN tagging.  VLAN tagging is beyond the scope of this course.

Any frame less than 64 bytes in length is considered a “collision fragment” or “runt frame” and is automatically discarded by receiving stations.  Frames more than 1500 bytes of data are considered “jumbo”  or “baby giant frames”.

If the size of a transmitted frame is less than the minimum, or greater than the maximum, the receiving device drops the frame.  Dropped frames are likely to be the result of collisions or other unwanted signals.  They are considered invalid.  Jumbo frames are usually supported by most Fast Ethernet and Gigabit Ethernet switches and NICs.

The figure shows each field in the Ethernet frame.  Refer to the table for more information about the function of each field.

| Preamble and Start Frame Delimiter Fields | The Preamble (7 bytes) and Start Frame Delimiter (SFD), also called the Start of Frame (1 byte), fields are used for synchronization between the sending and receiving devices. These first eight bytes of the frame are used to get the attention of the receiving nodes. Essentially, the first few bytes tell the receivers to get ready to receive a new frame. |
| --- | --- |
| Destination MAC Address Field | This 6-byte field is the identifier for the intended recipient. As you will recall, this address is used by Layer 2 to assist devices in determining if a frame is addressed to them. The address in the frame is compared to the MAC address in the device. If there is a match, the device accepts the frame. Can be a unicast, multicast or broadcast address. |
| Source MAC Address Field | This 6-byte field identifies the originating NIC or interface of the frame. |
| Type / Length | This 2-byte field identifies the upper layer protocol encapsulated in the Ethernet frame Common values are, in hexadecimal, 0x800 for IPv4, 0x86DD for IPv6 and 0x806 or ARP. Note: You may also see this field referred to as EtherType, Type or Length |
| Data Field | This field (46-1500 bytes ) contains the encapsulated data from a higher layer, which is a generic Layer 3 PDU, or more commonly, an IPv4 packet. All frames must be at least 64 bytes long. IF a small packet is encapsulated, additional bits called a pad are used to increase the size of the frame to this minimum size. |
| Frame Check Sequence Field | The Frame Check Sequence (FCS) field (4 bytes) is used to detect errors in a frame. It uses a cyclic redundancy check (CRC). The sending device includes the results of a CRC in the DCS field of the frame. The receiving device receives the frame and generates a CRC to look for errors. If the calculations match, no error occurred. Calculations that do not match and in indication that the data has changed; therefore, the frame is dropped. A change in the data could be the result of disruption of the electrical signals that represent the bits. |

## Ethernet MAC Address

### MAC Address and Hexadecimal

To reiterate, IPv4 addresses are represented using the decimal base 10 number system and the binary base 2 number system.  IPv6 addresses and Ethernet addresses are represented using the hexadecimal base 16 number system.

An Ethernet MAC address consists of a 46-bit binary value.  Hexadecimal is used to identify an Ethernet address because a single hexadecimal digit represents 4 binary bits.  Therefore, a 48-bit Ethernet MAC address can be expressed using only 12 hexadecimal values.

The figure compares the equivalent decimal and hexadecimal values for binary 0000 to 1111

Given that 8 bites (1 byte) is a common binary grouping, binary 00000000 to 11111111 can be represented in hexadecimal as the range 00 to FF.  As shown below:

When using hexadecimal, leading zeros are always displayed to complete the 8-bit representation.  For example, in the table, the binary value 000 1010 is shown in hexadecimal as 0A

Hexadecimal numbers are often represented by the value preceded by 0x (e.g., 0x73) to distinguish between decimal and hexadecimal values in documentation.

Hexadecimal may also be represented by a subscript 16, or the hex number followed by an H (e.g., 73H).

You may have to convert between decimal and hexadecimal values.  If such conversions are require, convert the decimal or hexadecimal value to binary, and then to convert the binary value to either decimal or hexadecimal as appropriate.

### Ethernet MAC Address

In an Ethernet LAN, every network device is connected to the same, shared media.  The MAC address is used to identify the physical source and destination device (NICs) on the local network segment.  MAC addressing provides a method for device identification at the data link layer of the OSI model.

An Ethernet MAC address is a 48-bit address expressed using 12 hexadecimal digits, as shown in the figure.  Because a byte equals 8 bits, we can also say that a MAC address is 6 bytes in length.

All MAC addresses must be unique to the Ethernet device or Ethernet interface.  To ensure this, all vendors that sell Ethernet devices must register with the IEEEE to obtain a unique 6 hexadecimal (i.e., 24-bit or 3-byte) code called the organizationally unique identifier (OUI).

When a vendor assigns a MAC address to a device or Ethernet interface, the vendor must do as follows:

- Use its assigned OUI as the first hexadecimal digits.

- Assign a unique value in the last 6 hexadecimal digits.

Therefore, an Ethernet MAC address consists of a 6-hexadecimal vendor OUI code followed by a 6-hexadecimal vender-assigned value, as shown in the figure.

For example, assume that Cisco needs to assign a unique MAC address to a new device.  The IEEE has assigned the Cisco a OUI of 00-60-2f.  Cisco would then configure the device with a unique vendor code such as 3A-07-BC.  Therefore, the Ethernet MAC address of that device would be 00-60-sF-3A-07-BC.

It is the responsibility of the vendor to ensure that none of its devices be assigned the same MAC address.  However, it is possible for supplicate MAC addresses to exist because of mistakes made during manufacturing, mistakes made in some virtual machine implementation methods, or modifications made using one of several software tools.  In any case, it will be necessary to modify the MAC address with a new NIC or make modifications via software.

### Frame Processing

Sometimes the MAC address is referred to as a burned is address (BIA) because the address is hardcoded into the read-only memory (ROM) on the NIC.  This means that the address is encode into the ROM chip permanently.

Note: On modern PC operating systems and NICs, it is possible to change the MAC address in software.  This is useful when attempting to gain access to a network that filters based on BIA.  Consequently, filtering or controlling traffic base on the MAC address is no longer as secure.

When the computer boots up, the NIC copies its MAC address from ROM into RAM.  When a device is forwarding a message to an Ethernet network, the Ethernet header includes these:

- Source MAC address – This is the MAC address of the source device NIC

- Destination MAC address – This is the MAC address of the destination device NIC

When a NIC received an Ethernet frame, it examines the destination MAC address to see if it matches the physical MAC address that is stores in RAM If there is no match, the device discards the frame. IF there is a match, it passes the frame up to the OSI layers, where the de-encapsulation process takes place.

Note: Ethernet NICs will also accept frames if the destination MAC address isa  broadcast or multicast group o which the host is a member.

Any device that is the source or destination of the Ethernet frame, will have an Ethernet NIC and there, a MAC address.  This includes workstations, servers, printers, mobile devices and routers.

### Unicast MAC address

In Ethernet, different MAC addresses are use for Layer 2 unicast, broadcast and multicast communications.

A unicast MAC address is the unique address that is use when a frame is sent from a single transmitting device to a single destination device.

As an example, a host with IPv4 address 192.168.1.5 (source) requests a web page from the server at IPv4 unicast address 192.168.1.200.  For a unicast packet to be sent and received, a destination IP address must be n the IP packet header.  A corresponding destination MAC address must also be present in the Ethernet frame header.  He IP address and the MAC address combine to deliver data to one specific destination host.

Note: The source MAC address must always be unicast.

### Broadcast MAC Address

An Ethernet broadcast frame is received and processed by every device on the Ethernet LAN.  The features of the Ethernet broadcast are as follows:

- It has a destination MAC address of FF-FF-FF-FF-FF-FF in hexadecimal (48 ones in binary).

- It is flooded out to all Ethernet switch ports except the incoming port.

- It is not forwarded by a router.

If the encapsulated data is in IPv4 broadcast packet, this means the packet contains a destination IPv4 address that has all ones (1s) in the host portion.  This numbering in the address means that all hosts on that local network (broadcast domain) will receive and process the packet.

As an example, the source host sends an IPv4 broadcast packet to all devices on its network.  The IPv4 destination address is a broadcast address, 192.168.1.255.  When the IPv4 broadcast packet is encapsulated in the Ethernet frame, the destination MAC address is the broadcast MAC address of FF-FF-FF-FF-FF-FF in hexadecimal (48 ones in binary).

DHCP for IPv4 is an example of protocol that uses Ethernet and IPv4 broadcast addresses.

However, not all Ethernet broadcasts carry an IPv4 broadcast packet.  For example, ARP Requests do not use IPv4, but the ARP message is sent as an Ethernet broadcast.

### Multicast MAC address

AN Ethernet multicast frame is received and processed by a group of devices on the Ethernet LAN that belong to the same multicast group.  The features of an Ethernet multicast are as follows:

- There is a destination MAC address of 01-00-5E when the encapsulated data is an IPv4 multicast packet and a destination MAC address of 33-33 when the encapsulated data is an IPv6 multicast packet.

- There are other reserved multicast destination MAC address for when the encapsulated data is not an IP, such as Spanning Tree Protocol (STP) and Link Layer Discovery Protocol (LLDP).

- It is flooded out all Ethernet witch port except the incoming port, unless the switch is configured for multicast snooping.

- It is not forwarded by a router, unless the router is configured to route multicast packets.

If the encapsulated data is an IP multicast packet, the devices that belong to a multicast group are assigned a multicast group IP address.  The range of IPv4 multicast address is 144.0.0.0 to 239.255.255.255.  The range of IPv6 multicast addresses begins with ff00::/8.  Because multicast addresses represent a group of addresses (sometimes called a host group), they can only be use as the destination of a packet.  The source will always be a unicast address.

As with the unicast and broadcast addresses, the multicast IP address requires a corresponding multicast MAC address to deliver frames on a local network. The multicast MAC address is associate with, and uses addressing information from, the IPv4 or IPv6 multicast address.

## The MAC Address Table

### Switch Fundamentals

Now that we know about Ethernet MAC addresses, it is time to talk about how a switch uses these addresses to forward (or discard) frames to other devices on a network.  If a switch just forwarded every frame it received out to all ports, your network would be so congested that it would likely just stop working.

At Layer 2 Ethernet switch uses Layer 2 MAC addresses to make forwarding decisions.  It is completely unaware of the data (protocol) being cared in the data portion of the frame, such as an IPv4 packet, an ARP message, or an IPv6 ND packet.  The switch makes its forwarding decisions based solely on the Layer 2 Ethernet MAC addresses.

An Ethernet switch examines its MAC address table to make a forwarding decision for each frame, unlike legacy Ethernet hubs that repeat bits out all ports except the incoming port.  In the figure, the four-port switch was just powered on.  The table shows the MAC address Table which has not yet learned the MAC addresses for the four attached PCs.

Note: MAC addresses are shortened throughout this topic for demonstration purposes.

Note: The MAC address table is sometimes referred to as a content addressable memory (CAM) table.  While the term CAM table is common, for the purposes of tis course, we will refer to it as a MAC address table.

### Switch Learning and Forwarding

The switch dynamically builds the MAC address table by examining the source MAC address of the frames received on a port.  The switch forwards frames by searching for a match between the destination MAC address in the frame and an entry in the MAC address table.

Examine the Source MAC Address

Every frame that enters a switch is checked for new information to learn.  It does this by examining the source MAC address of the frame and the port number where the frame entered the switch.  If the source MAC address does not exist, it is added to the table along with the incoming port number.  If the source MAC address does exist, the switch updates the refresh timer for that entry in the table.  By default, most Ethernet switches keep an entry n the table for 5 minutes.

In the figure, for example, PC-A is sending an Ethernet frame to PC-D.  The table show he switch adds the MAC address for PC-A to the MAC Address table.

Note: If the Source MAC address does exist in the table but on a different port, the switch treats this as a new entry.  The entry is replace using the same MAC address but with the more current port number.

Find the Destination MAC Address

If the destination MAC address is a unicast address, the witch will look for a match between the destination MAC address of the frame and an entry in its MAC address table  If the destination MAC address is in the table, it will forward the frame out the specified port.  If the destination MAC address is not in the able, the switch will forward the frame out all ports except the incoming port.  This is called an unknown unicast.

As shown in the figure, the switch does not have the destination MAC address in its table for PC-D so it sends the frame out all ports except for port 1

Note: If the destination MAC address is a broadcast or a multicast, the frame is also flooded out all ports except the incoming port.

### Filtering Frames

As a switch received frames from different devices, it can populate its MAC address table by examining the source MAC address of every frame.  When the MAC address table of the switch contains the destination MAC address, it can filter the frame and forward out a single port.

PC-D to Switch

In the figure, PC-D is replying to PC-A.  The switch sees the MAC address of PC-D in the incoming frame on port 4.  The switch then puts the MAC address of PC-D into the MAC address Table associated with port 4

Switch to PC-A

Next, because the switch has destination MAC address for PC-A in the MAC Address Table, it wills end the frame only out port 1, as shown in the figure.

PC-A to Switch to PC-D

Next, PC-A sends another frame to PC-D, as shown in the figure.  The MAC address table already contains the MAC address for PC-A; therefore, the five-minute refresh timer for that entry is reset.  Next, because the switch table contains the destination MAC address for PC-D, it sends the frame only out port 4.

## Switch Speeds and Forwarding

### Frame Forwarding Methods on Cisco Switches

As you learned in the previous topic, switches use their MAC address tables to determine which port to use to forward frames.  With Cisco switches, there are two frame forwarding methods and there are good reasons to use one instead of the other, depending on the situation

Switches use one of the following forwarding methods for switching data between network ports:

- Store-and-forward switching – This frame forwarding method receives the entire frame and computes the CRC.  CRC uses a mathematical formula, base on the number of bits (1s) in the frame, to determine whether the receive frame has an error.  If the CRC is valid, the switch looks up the destination address, which determines the outgoing interface.  Then the frame is forwarded out to the correct port.

- Cut-through switching – This frame forwarding method forwards the frame before it is entirely received.  At a minimum, the destination address of the frame must be rea before the frame can be forwarded.

A big advantage of store-and-forward switching is that it determines if a frame has errors before propagating the frame.  When an error is detected in a frame, the switch discards the frame.  Discarding frames with errors reduces the amount o bandwidth consumed by corrupt data.  Store-and-forward witching is required for quality of service (QoS) analysis on converged networks where frame classification for traffic prioritization is necessary.  For example, voice over IP (VoIP) data streams nee to have priority over web-browsing traffic.

### Cut-Through Switching

In cut-through switching, the switch acts upon the data as soon as it is received, even if the transmission is not complete.  The switch buffers just enough of the frame to read the destination MAC address so that it can determine to which port it should forward out the data.  The destination MAC address is located in the first 6 bytes of the frame following the preamble.  The switch looks up the destination MAC address in its switching table, determines the outgoing interface port, and forwards the frame onto its destination through the designate switch port.  The switch does not perform any error checking on the frame.

There are two variants of cut-through switching:

- Fast-forward switching – Fast-forward switching offers the lowest level of latency.  Fast-forward switching immediately forwards a packet after reading the destination address.  Because fast-forward switching starts forwarding before the entire packet has been receive, there ma be times when packets are relayed with errors.  This occurs infrequently, and the destination NIC discards the faulty packet upon receipt.  In fast-forward mode, latency is measured from the first bit receive to the first bit transmitted.  Fast-forward switching is the typical cut-through method of switching.

- Fragment-free switching – In fragment-free switching, the switch stores the first 64 bytes of the frame before forwarding.  Fragment-free switching can be viewed as a compromise between store-and-forward switching and fast-forward switching.  The reason fragment-free switching stores only the first 64 bytes of the frame is that most network errors and collisions occur in the first 64 bytes.  Fragment free switching tries to enhance fast-forward switching by performing a small error check on the first 64 bytes of the frame to ensure that a collision has not occurred before forwarding the frame.  Fragment-free switching is a compromise between the high latency and high integrity of store-and-forward switching, and the low latency and reduced integrity of fast-forward switching.

Some switches are configured to perform cut-through switching on a per-port basis util a user-define error threshold is reached, and then they automatically change to store-and-forward.  When the error rate falls below the threshold, the port automatically changes back to cut-through switching.

### Memory Buffering on Switches

An Ethernet switch may use a buffering technique to store frames before forwarding them.  Buffering may also be use when the destination port is busy because of congestion.  The switch stores the frame until it can be transmitted.

As shown in the table, there are two methods of memory buffering:

Memory Buffering Methods

| Method | Description |
| --- | --- |
| Port-based memory | - Frames are stored in queues that are linked to specific incoming and outgoing ports. - A frame is transmitted to the outgoing port only when all the frames ahead in the queue have been successfully transmitted - It is possible for a single frame to delay the transmission of all the frames in memory. - This delay occurs even if the other frames could be transmitted to open destination ports. |
| Shared memory | - Deposits all frames into a common memory buffer shared by all switch ports and the amount of buffer memory required b a port is dynamically allocated. - The frames in the buffer are dynamically linked o the destination port enabling a packet to be receive on one port and then transmitted on another port, without moving it to a different queue. |

Shared memory buffering also results in the ability to store larger frames with potentially fewer dropped frames.  This is important with asymmetric switching which allows for different data rates on different ports sch as when connecting a server to a 10 Gbps switch port and PCs to 1 Gbps ports.

### Duplex and Speed Settings

Two of the most basic settings on a switch are the bandwidth (sometimes referred to as “speed”) ad duplex settings for each individual switch port.  It is critical that the duplex and bandwidth setting match between the switch port and the connected devices, such as a computer or another switch.

There are to types of duplex settings sued for communications on an Ethernet network:

- Full-duplex – Both ends of the connection can send and receive simultaneously.

- Half-duplex – Only one end o the connection can send at a time.

Autonegotiation is an optional function found on most Ethernet switches and NICs.  It enables two devices to automatically negotiate the best speed and duplex capabilities.  Full duplex is chosen if both devices have the capability along with their highest common bandwidth.

In the figure, the Ethernet NIC for PC-A can operate in full-duplex of half-duplex and in 10 Mbps or 100 Mbps.

Note: Most Cisco switches and Ethernet NICs default to autonegotiation for speed and duplex.  Gigabit Ethernet ports only operate in full-duplex.

Duplex mismatch is one of the most common causes of performance issues on 10/100 Mbps Ethernet links.  It occurs when one port on the link operates at half duplex while the other port operates at full-duplex, as shown in the figure.

Duplex mismatch occurs when one or both ports on a link are reset, and the autonegotiation process does not result in both link partners having the same configuration.  It also can occur when users reconfigure one side of a link and forget to reconfigure the other.  Both sides of a link should have autonegotiation on, or both sides should have it off.  Best practice is to configure both Ethernet switch ports as full-duplex.

### Auto-MDIX

Connections between devices once required the use of either a crossover or straight-through cable.  The type of cable required depended on the type of interconnecting devices.

For example, the figure identifies the correct cable type required to interconnect witch-to-switch, switch-to-router, switch-to-host or router-to-host devices.  A crossover cable is used when connecting like devices, and a straight-through cable is use for connecting unlike devices.

Note: A direct connection between a router and a host requires a cross-over connection.

Most switch devices now support the automatic medium-dependant interface crossover (auto-MDIX) feature.  When enabled, the switch automatically detects the type of cable attached to the port and configures the interfaces accordingly.  Therefore, you can use either a crossover or a straight0through cable for connections to a copper 10/100/1000 port on the switch, regardless of the type of device on the other end of the connection.

The auto-MDIX feature is enabled by default on switching running Cisco IOS release 12.2(18)SE or later.  However, the feature could be disable.  For this reason, you should always use correct cable type and not rely on the auto-MDIX feature.  Auto-MDIX can be re-enable using the mdix auto interface configuration command.

# Network Layer

## Network Layer Characteristics

### The Network Layer

The network layer or OSI Layer 3, provides services to allow end devices to exchange data across networks.  As show in the figure, IP version 4 (IPv4) and IP version 6 (IPv6) are the principle network layer communication protocols.  Other network layer protocols include routing protocols such as Open Shortest Path First (OSPF) and messaging protocols such as Internet Control Message Protocol (ICMP).

To accomplish end-to-end communications across network boundaries, network layer protocols perform four basic operations:

- Addressing end devices – End devices must be configured with a unique IP adder for identification on the network.

- Encapsulation – The network layer encapsulates the protocol data unit (PDU) from the transport layer into a packet.  The encapsulation process adds IP header information, such as the IP address of the source (sending) and the destination (receiving) hosts.  The encapsulation process is performed by the source of the IP packet.

- Routing – The network layer provides services to direct the packets to a destination host on another network.  To travel to other networks, the packet must be processed by a router.  The role of the router is to select the best path and direct packets toward the destination host in the process known as routing.  A packet may cross many routers before reaching the destination host.  Each router a packet crosses to reach the destination hos is called a hop.

- De-encapsulation – When the packet arrives at the network layer of the destination host, the host checks the IP header of the packet.  If the destination IP address within the header matches its own IP address, the IP header is removed rom the packet.  After the packet is de-encapsulated by the network layer, the resulting Layer 4 PDU is passed up to the appropriate service at the transport layer.  The de-encapsulation process is performed by the destination host of the IP packet.

Unlike the transport layer (OSI Layer 4), which manages the data transport between the processes running on each host, network layer communication protocols (i.e, IPv4 and IPv6) specify the packet structure and processing used to carry the data from one host to another host.  Operating without regard to the data carried in each packet allows the network layer to carry packets for multiple types of communications between multiple hosts.

The Network layer:

- Provides services to allow end devices to exchange data

- IP version 4 (IPv4) and IP version 6 (IPv6) are the principle network layer communication protocols.

- The network layer performs four basic operations:

  - Addressing end devices

  - Encapsulation

  - Routing

  - De-encapsulation

### IP Encapsulation

IP encapsulates the transport layer (the layer just above the network layer) segment or other data by adding an IP header.  The IP header is used to deliver the packet to the destination host.

The figure illustrates how the transport layer PFU is encapsulated by the network layer PDU to create an IP packet.

The process of encapsulating data layer by layer enables the services at the different layers to develop a scale without affecting the other layers.  This means the transport layer segments can be readily packages by IPv4 or IPv6 or by any new protocol that might be developed in the future.

The IP header is examined by Layer 3 devices (i.e., routers and Layer 3 switches) as it travels across a network to its destination.  It is important to note, that the IP addressing information remains the same from the time the packet leaves the source host until it arrives at the destination host, except when translated by the device performing Network Address Translations (NAT) for IPv6.

Note: NAT is discussed in later modules

Routers implement routing protocols to route packets between networks.  The routing performed by these intermediary devices examines the network layer addressing in the packet header.  In all cases, the data portion of the packet, that is, the encapsulated transport layer PDU or other data, remained unchanged during the network layer processes.

IP encapsulation:

- IP encapsulates the transport layer segment.

- IP can use either an IPv4 or IPv6 packet and not impact the Layer 4 segment

- IP packet will be examined by all Layer 3 devices as it traverses the network.

- The IP addressing does not change from source to destination

- NAT will change addressing.

### Characteristics of IP

IP was designed as a protocol with low overhead.  It provides only the functions that are necessary to deliver a packet from a source to a destination over an interconnected system of networks  The protocol was not designed to track and manage the flow of packets.  These functions, if requires, are performed by other protocols at other layers, primarily TCP at Layer 4.

These are the basic characteristics of IP:

- Connectionless – There is no connection with the destination established before sending data packets.

- Best efforts – IP is inherently unreliable because packet delivery is not guaranteed.

- Media Independent – Operation is independent of the medium (i.e., copper, fiber-optic, or wireless) carrying the data.

### Connectionless

IP is connectionless, meaning that no dedicated end-to-end connection is created by IP before data is sent.  Connectionless communication is conceptually similar to sending a letter to someone without notifying the recipient in advance.

Connectionless data communications work on the same principle.  As shown in the figure, IP requires o initial exchange of control information to establish an end-to-end connection before packets are forwarded.

IP is connectionless

- IP does not establish a connection with the destination before sending the packet.

- There Is no control information needed (synchronizations, acknowledgements, etc)

- The destination will receive the packet when it arrives, but no pre-notifications are sent by IP

- If there is a need or connection-oriented traffic, then another protocol will handle this (typically TCP at the transport layer)

### Best Effort

IP also does not require additional fields in the header to maintain an established connection.  This process greatly reduces the overhead of IP.  However, with no pre-established end-to-end connection, senders are unaware whether destination devices are present and functional when sending packets, or are they aware if the destination receives the packet, or if the destination deice is able to access and read the packet.

The IP protocol does not guarantee that all packets that are delivered are, in fact, received.  The figure illustrates the unreliable or best-effort delivery characteristics of the IP protocol.

IP is Best Effort:

- IP will not guarantee delivery of the packet.

- IP has reduced overhead since there is no mechanism to resend data that is not received.

- IP does not expect acknowledgments.

- IP does not know I the other device is operational or if it received the packet.

### Media Independent

Unreliable means that IP does not have the capability to manage and recover from undelivered or corrupt packets.  This is because while IP packets are sent with information about the location of delivery, they do not contain information that can be processed to inform the send whether delivery was successful.  Packets may arrive at the destination corrupted, out of sequence, or not at all.  IP provides no capability for packet retransmissions if errors occur.

If out-of-order packets are delivered, or packers are missing then applications using the data, or upper layer services, must resolve these issues.  This allows IP to function very efficiently.  In the TCP/IP protocol suite, reliability is the role o the TCP protocol at the transport layer.

IP operates independently of the media that carry the data at lower layers of the protocol stack.

The OSI data link layer is responsible for taking an IP packet and preparing it for the transmission over the communications medium.  This means that the delivery of IP packets is not limited to any particular medium.

There is, however, one major characteristic of the media that the network layer considers: the maximum size of the PDU that each medium can transport.  This characteristic is referred to as the maximum transmission unit (MTU). Part of the control communication between the data link layer and the network layer is the establishment of a maximum size for the packet.  The data link layer passes the MTU value up to the network layer.  The network layer then determines how large packets can be.

In some cases, an intermediate device, usually a router, must split up an IPv4 packet when forwarding it from one medium to another medium with a smaller MTU.  This process is called fragmenting the packet, or fragmentation.  Fragmentation causes latency.  IPv6 packets cannot be fragmented by the router.

IP is unreliable:

- It cannot manage or fix undelivered or corrupt packets.

- IP cannot retransmit after an error

- IP cannot realign out of sequence packets.

- IP must rely on other protocols for these functions

IP is media independent:

- IP does not concern itself with the type of frame required at the data link layer or the media type at the physical layer.

- IP can be sent over any media type: Copper fiber, or wireless.

The network layer will establish the Maximum Transmission Unit (MTU):

- Network layer receives this from control information sent by the data link layer.

- The network then establishes the MTU size.

Fragmentation is when Layer 3 splits the IPv4 packet into small units.

- Fragmenting causes latency

- IPv6 does not fragment packets.

- Example: Router goes from Ethernet to a slow WAN with a smaller MTU

## IPv4 Packet

### IPv4 Packet Header

IPv4 is one of the primary network layer communication protocols.  The IPv4 packet header is used to ensure that this packet is delivered to its next stop on the way to its destination end device.

An IPv4 packet header consists of fields containing important information about the packet.  These fields contain binary numbers which are examined by the Layer 3 Process

IPv4 Packet Header

  - It ensures the packet is sent in the correct direction (to the destination)

  - It contains information for network layer processing in various fields

  - The information in the header is used by all Layer 3 devices the handle the packet.

### IPv4 Packet Header Fields

The binary values of each field identify various settings of the IP packet.  Protocol header diagrams, which are read left to right, and top down, provide a visual to refer to when discussing protocol fields.  The IP protocol header diagram in the figure identifies the fields of an IPv4.

Significant fields in the IPv4 header include the following:

- Version – Contains a 4-bit binary value set to 0100 that identifies this as in IPv4 packet.

- Differentiated Services of DiffServ (DS) – Formerly called the type of service (ToS) field, the DS field is an 8-bit field used to determine the priority of each packet  The six most significant bits of the DiffServ field are the differentiated services code point (DSCP) bits and the last two bits are the explicit congestion notification (ECN) bits

- Time to Live (TTL) – TTL contains an 8-bit binary value that is used to limit the lifetime of a packet.  The source device of the IPv4 packet sets the initial TTL value.  It is decreased by one each time the packet is processed by a router.  If the TTL field decrements to zero, the router discards the packet and sends and Internet Control Messed Protocol (ICMP) Time Exceeded message to the source IP address.  Because the router decrements the TTL of each packet, the router must also recalculate the Header Checksum.

- Protocol – This field is used to identify the next level protocol.  This 8bt binary value indicates the data payload type that the packet is carrying, which enables the network layer to pass the data to the appropriate upper-layer protocol.  Common values include ICMP (1), TCP (6), and UDP (17).

- Header Checksum – This is used to detect corruption in the IPv4 header.

- Source IPv4 Address – This contains a 32-bit binary value that represents the source IPv4 address of the packet  The source IPv4 address is always a unicast address.

- Destination IPv4 Address – This contains a 32-bit binary value that represents the destination Pv4 address of the packet  The destination IPv4 address is a unicast, multicast or broadcast address.

The two most referenced fields are the source and destination IP addresses.  These fields identify where the packet is coming from and where it is going.  Typically, these addresses do not change while travelling from the source to the destination.

The Internet Header Length (IHL), Total Length, and Header Checksum fields re used to identify and validate the packet.

Ither fields are used to reorder a fragmented packet.  Specifically, the IPv4 packet uses Identification, Flags, and Fragment Offset fields to keep trac of the fragments.  A router may have to fragment an IPv4 packet when forwarding it from one medium to another with a smaller MTU.

The Options and Padding fields are rarely used and are beyond the scope of this module.

The IPv4 network header characteristics:

  - It is in binary.

  - Contains several fields of information

  - Diagram is rea from left to right, 4 bytes per line

  - The two most important fields are the source and destination

## IPv6 Packet

### Limitations of IPv4

IPv4 is still in use today.  This topic is about IPv6, which will eventually replace IPv4.  TO better understand why you need to know the IPv6 protocol, it helps to know the limitations of IPv4 and the advantages of IPv6.

Through the years, additional protocols and processes have been developed to address new challenges.  However, even with changes, IPv4 still has three major issues:

- IPv4 Address Depletion – IPv4 has a limited number of unique public addresses available.  Although there are approximately 4 billion IPv4 addresses, the increasing number of new ip-enabled devices, always-on connections and the potential growth of less-developed regions have increased the nee for more addresses.

- Lack of end-to-end connectivity – Network Address Translation (NAT) is a technology commonly implemented within IPv4 networks.  NAT provides a way for multiple devices to share a single public IPv4 address.  However, because the public IPv4 address is shared, the IPv4 address of an internal network host is hidden.  This can be problematic for technologies that require end-to-end connectivity.

- Increased network complexity – While AT has extended the lifespan if IPv4, it was only meant as a transition mechanism to IPv6.  NAT in its various implementation create additional complexity in the network, creating latency and making troubleshooting more difficult.

### IPv6 Overview

In the early 1990s, the Internet Engineering Task Force (IETF) grew concerned about the issues with IPv4 and began to look for a replacement.  This activity le to the development of IP versions 6 (IPv6). IPv6 overcomes the limitations of IPv4 and is a powerful enhancement with features that better suit current and foreseeable network demands.

Improvements that IPv6 provides include the following:

- Increased address space – IPv6 addresses are based on 128.bit hierarchical addressing as opposed to IPv4 with 32 bits

- Improved packet handling – The IPv6 header has been simplified with fewer fields.

- Eliminates the need for NAT – With such a large number of public IPv6 addresses, NAT between a private IPv4 address and a public IPv4 is not needed. This avoids some of the NAT-induced problems experienced by applications that require end-to-end connectivity.

The 32-bit IPv4 address space provides approximately 4 294 967 296 unique addresses.  IPv6 address space provides 340 282 366 920 938 463 463 374 607 431 768 211 456, or 340 undecillion addresses. This is roughly equivalent to every grain of sand on Earth.

### IPv4 Packet Header Fields in the IPv6 Packet Header

One of the major design improvements of IPv6 over IPv4 is the simplified IPv6 header.

For example, the IPv4 header consists of a variable length header of 20 octets (up to 60 bytes if the Options field is used) and 12 basic header fields, not including the Options field and Padding field.

For IPv6, some fields have remained the same, some fields have changed names and positions, and some IPv4 fields are no longer required, as highlighted in the figure.

In contrast, the simplified IPv6 header shown the next figure consists of a fixed length header for 40 octets (largely due to length of the source and destination IPv6 addresses).

The IPv6 simplified header allows for more efficient processing of IPv6 headers.

### IPv6 Packet Header

The IP protocol header diagram identifies the fields of an IPv6 packet.

- Version – The field contains a 4-bit binary value set to 0110 that identifies this as an IP version 6 packet.

- Traffic Class – This 8-bit field is equivalent to the IPv4 Differentiated Services (DS) field.

- Flow Label – This 20-bit field suggests that all packets with the same flow label receive the same type of handling by routers.

- Payload Length – This 16-bit field indicates the length of the data portion or payload of the IPv6 packet.  This does not include the length of the IPv6 header, which is a fixed 40-byte header.

- Next Header – This 8-bit field is equivalent to the IPv4 Protocol field.  It indicates the data payload type that the packet is carrying, enabling the network layer to pass the data to the appropriate upper-layer protocol.

- Hop Limit – This 8-bit field replaces the IPv4 TTL field.  This value is decremented by a value of 1 by each router that forwards the packet  When the counter reaches 0, the packet is discarded, and an ICMPv6 Time Exceeded message is forward to the sending host.  This indicates that the packet did not reach its destination because the hop limit as exceeded  Unlike IPv4, IPv6 does not include an IPv6 Header Checksum, because this function is performed at both the lower and upper layer.  This means the checksum does not need to be recalculated by each router when it decrements the Hop Limit field, which also improves network performance.

- Source IPv6 Address – This is 128-bit field identifies the IPv6 address of the sending host.

- Destination IPv6 Address – This 128.bit field identifies the IPv6 address of the receiving host.

An IPv6 packet may also contain headers (EH), which provide optional network layer information.  Extension headers are optional and are placed between the IPv6 header and the payload.  EHs are used for fragmentation, security, to support mobility and more.

Unlike IPv4, routers do not fragment routed IPv6 packets.

## How a Host Routes

### Host Forwarding Decision

With both IPv4 and IPv6, packets are always created at the source host. The source host must be able to direct the packet to the destination host.  To do this, host end devices create their own routing table.  This topic discusses how end devices use routing tables.

Another role of the network layer is to direct packets between hosts.  A host can send a packet to the following:

- Itself – A host can ping itself by sending a packet t a special IPv4 address of 127.0.0.1 or an IPv6 address ::1, which is referred to as the loopback interface.  Pinging the loopback interface tests the TCP/IP protocol stack on the host.

- Local host – This is destination host that is on the same local network as the sending host.  The source and destination hosts share the same network address.

- Remote host – This is a destination host on a remote network.  The source and destination hosts do not share the same network address.

In a home or business network, you may have several wired and wireless devices interconnected together using an intermediary device, such as a LAN witch or a wireless access point (WAP).  This intermediary device provides interconnections between local hosts on a local network.  Local hosts can reach each other and share information without the need for any additional devices.  If a host is sending a packet to a device that is configured with the same IP network as the host device, the packet it simply forwarded out of the host interface, through the intermediary device, and to the destination device directly.

Of course, in most situations we want our devices to be able to connect beyond the local network segment, such as out to other homes, businesses, and the internet.  Devices that are beyond the local network segment are known as remote hosts.  When a source device sends a packet to a remote destination device, then the help of routers and routing is needed.  Routing is the process of identifying the best path to a destination.  The router connected to the local network segment is referring to as the default gateway.

### Default Gateway

The default gateway is the network device (i.e., router or Layer 3 switch) that can route traffic to other networks  If you use the analogy that a network is like a room, then the default gateway is like a doorway.  If you want to get to another room or network, you need to find the doorway.

On a network, a default gateway is usually a router with these features:

- It has a local IP address in the same address range as other hosts on the local network.

- It can accept data into the local network and forward data out o the local network.

- It routes traffic to other networks.

A default gateway is required to send traffic outside of the local network.  Traffic connect be forwarded outside the local network if there is no default gateway, the default gateway address is not configured, or the default gateway is down.

### A Host Routes to the Default Gateway

A host routing table will typically include a default gateway.  In IPv4, the host receives the IPv4 address of the default gateway either dynamically from Dynamic Host Configuration Protocol (DHCP) or configured manually.  In IPV6, the router advertises the default gateway address or the host can be configured manually.

Having a default gateway configure creates a default route in a routing table of the PC.  A default route is the route or pathway your computer ill take when it tries to contact a remote network.

Both PC1 and PC2 will have a default route to send all traffic destine to remote networks R1.

### Host Routing Tables

On a Windows host, the route print or netstat -4 command can be used to display the host routing table.  Both commands generate the same output.  The output may seem overwhelming at first but is fairly simple to understand.

Entering the netstat -r command or the equivalent route print command displays here section relate the current TCP/IP network connections:

- Interface List – Lists the Media Access Control (MAC) address and assigned interface number of every network-capable interface on the host, including Ethernet, Wi-Fi and Bluetooth adapters.

- IPv4 Route Table – Lists all known IPv4 routes, including direct connections, local network, and local default routes.

- IP6 Route Table – Lists all known IPv6 routes, including direct connections, local network, and local default routes.

## Introduction to Routing

### Router Packet Forwarding Decision

The previous topic discussed host routing tables.  Most networks also contain routers, which are intermediary devices.  Routers also contain routing tables.  This topic covers router operations at the network layer.  When a host sends a packet to another host, it consults its routing table to determine where to send the packet.  If the destination host is on a remote network, the packet is forwarded to the default gateway, which is usually the local router.

What happens when a packet arrives on a router interface?

The router examines the destination IP address of the packet and searches its routing table to determine where to forward the packet.  The routing table contain a list of all known network addresses (prefixes) and where to forward the packet.  These entries are known as route entries or routes.  The router will forward the packet using the best (longest) matching route entry.

### IP Router Routing Table

The routing table of the router contains network route entries listing all the possible known network destinations.

The routing table stores three types of route entries:

- Directly-connected networks – These network route entries are active router interfaces.  Routers add a directly connected route when an interface is configured with an IP address and is activated.  Each router interface is connected to a different network segment.  In the figure, the directly-connected networks in the R1 IPv4 routing table would be 192.168.10.0/24 and 209.165.200.224/30

- Remote networks – These network route entries are connected to other routers.  Routers learn about remote networks either by being explicitly configure y an administrator or by exchanging route information using a dynamic routing protocol.  In the figure, the remote network in the R1 IPv4 routing table would be 10.1.1.0/24

- Default route – Like a host, most routers also include a default route entry, a gateway of last resort.  The default route is used when there is no better (longer) match in the IP routing table.  In the figure, the R1 IPv4 routing table would most likely include a default route to forward all packets to router R2.

The figure identifies the directly connected and remote networks of router R1.

A router can learn about remote networks in one of two ways:

- Manually – Remote networks are manually entered into the route table using static routes.

- Dynamically – Remote routes are automatically learned using a dynamic routing protocol.

### Static Routing

Static routes are route entries that are manually configured.  The figure shows an example of a static route that was manually configured on router R1.  The static route includes the remote network address and the IP address of the next hop router.

IF there is a change in the network topology, the static route is not automatically updated and must be manually reconfigured.  For example, in the figure R1 has a static route to reach the 10.1.1.0/24 network via R2  If that path is no longer available, R1 would need to be reconfigured with a new static route to the 10.1.1.0/24 network via R3.  Router R3 would therefore need to have a route entry in its routing table to send packets destined for 10.1.1.0/24 to R2

Static routing has the following characteristics:

- A static route must be configured manually.

- The administrator needs to reconfigure a static route if there is a change in the topology and the static route is no longer viable.

- A static route is appropriate for a small network and when there are few or no redundant links.

### Dynamic Routing

A dynamic routing protocol allows the routers to automatically learn about remote networks, including a default route, from other routers.  Routers that use dynamic routing protocols automatically share routing information with other routers and compensate for any topology changes without involving the network administrator. If there is a change in the network topology, routers share this information sing the dynamic routing protocol and automatically update their routing tables.

Dynamic routing protocols include OSPF and Enhanced Interior Gateway Routing Protocol (EIGRP).  The figure shows an example of router R1 and R2 automatically sharing network information using the routing protocol OSPF.

Basic configuration only requires the network administrator to enable the directly connected networks within the dynamic routing protocol.  The dynamic routing protocol will automatically do as follows:

- Discover remote networks

- Maintain up-to-date routing information

- Choose the best path to destination networks

- Attempt to find a new best path if the current path is no longer available

When a router is manually configured with a static route or learns about a remote network dynamically using a dynamic routing protocol, the remote network address and not hop address and entered into the IP routing table.  As shown in the figure if there is a change in the network topology, the routers will automatically adjust and attempt to find a new best path.

Note It is common for some routers to use a combination of both statics routes and a dynamic routing protocol.

### Introduction to an IPv4 Routing Table

Notice in the figure that R2 is connected to the internet.  Therefore, the administrator configured R1 with a default static route sending packets to R2 when there is no specific entry in te routing table that matches the destination IP address.  R1 and R2 are also using OSPF routing to advertise directly connected networks.

The show ip route privileged EXEC mode command is used to view the IPv4 routing table on a Cisco IOS router.  The example shows the IPv4 routing table of router R1  At the beginning of each routing table entry is a code that is used to identify the type of route or ho the route was learned.  Common route sources (codes) include these:

- L – Directly connected local interface IP address

- C – Directly connected network

- S – Static route was manually configured by an administrator

- O – OSPF

- D – EIGRP

The routing displays all the known IPv4 destination routes or R1.

A directly connected route is automatically created when a router interface is configured with IP address information and is activated.  The router adds two route entries with the codes C (i.e., the connected network) and L (i.e., the local interface IP address of the connected network.).  The route entries also identify the exit interface to use to reach the network.  The two directly connected networks in this example are 192.168.10.0/24 and 209.165.200.224/30.

Routers R1 and R2 are also using the OSPF dynamic routing protocol to exchange router information.  In the example routing table, R1 has a route entry for he 10.1.1.0/24 network that is learned dynamically from router R2 via the OSPF routing protocol.

A default route has a network address of all zeroes.  For example, the IPv4 network address is 0.0.0.0.  A static route entry in the routing table begins with a code of S*, as highlighted in the example.

# Address Resolution

## MAC and IP

### Destination on Same Network

Sometimes a host must send a message, but it only knows the IP address of the destination device.  The host needs to know the MAC address of that device, but how can it be discovered?  That is where address resolution becomes critical.

There are two primary addresses assigned to a device on an Ethernet LAN:

- Physical Address (the MAC address) – Used for NIC to NIC communications on the same Ethernet network.

- Logical address (the IP address) – Used to send the packet from the source device to the destination device.  The destination IP address may be on the same IP network as the source, or it may be on a remote network.

Layer 2 physical addresses (i.e., Ethernet MAC addresses) are used to deliver the data link frame with the encapsulated IP packet from one NIC to another NIC that is on the same network.  If the destination IP address is on the same network, the destination MAC address will be that of the destination device.

Consider the following example using simplified MAC address representations:

In this example, PC1 wants to send a packet to PC2  The figure displays the Layer 2 destination and source MAC addresses and the Layer 3 IPv4 addressing that would be included in the packet sent from PC1.

The Layer 2 Ethernet frame contains the following:

- Destination MAC address – This is the simplified MAC address of PC2, 55-55-55.

- Source MAC address – This is the simplified MAC address of the Ethernet NIC on PC1, aa-aa-aa

The Layer 3 IP packet contains the following:

- Source IPv4 address – This is the IPv4 address of PC1, 192.168.10.10

- Destination IPv4 address – This is the IPv4 address of PC2, 192.168.10.11

### Destination on Remote Network

When the destination IP address (IPv4 or IPv6) is on a remote network, the destination MAC address will be the address of the host default gateway (i.e., the router interface).

Consider the following example using a simplified MAC address representation:

In this example, PC1 wants to send a packet to PC2.  PC2 is locate on a remote network.  Because the destination IPv4 address is not on the same local network as PC1, the destination MAC address is that of the local default gateway on the router.

Routers examine the destination IPv4 address to determine the best path to forward the IPv4 packet.  When the router receives the Ethernet frame, it de-encapsulates the Layer 2 information.  Using the destination IPv4 address, it determines the net-hop device and then encapsulates the IPv4 packet in a new data link frame for the outgaining interface.

In our example, R1 would ne encapsulate the packet with new Layer 2 address information as shown in the figure:

The new destination MAC address would be that of the R2 G0/0/1 interface and the new source MAC address be that of the R1 G0/0/1 interface.

Along each link in a path, an IP packet is encapsulate in a frame.  The frame is specific to the data link technology that is associated with that link, such as Ethernet.  If the next-hop device is the final destination, the destination MAC address will be that of the device Ethernet NIC, as shown in the figure:

How are the IP address of the IP packets in a data flow associated with the MAC addresses on each link along the path to the destination?  For IPv4 packets, this is done through a process called Address Resolution Protocol (ARP).  For IPv6 packets, the process is ICMPv6 Neighbor Discovery (ND).

## Address Resolution Protocol (ARP)

### ARP Overview

If your network is using the IPv4 communications protocol, the Address Resolution Protocol, or ARP, is what you need to map IPv4 addresses to MAC addresses.  This topic explains how ARP works.

Every IP device on an Ethernet network has a unique Ethernet MAC address.  When a device sends an Ethernet Layer 2 frame, it contains these two addresses:

- Destinaton MAC address – The Ethernet MAC address of the destination device on the same local network segment.  I the destination host is on another network, then the destination address in the frame would be that of the default gateway (i.e., the router)

- Source MAC address – The MAC address of the Ethernet NIC on the source host.

The figure illustrates the problem when sending a frame to another host on the same segment on an IPv4 network.

To send a packet to another host on the same local IPv4 network, a host must know the IPv4 address and the MAC address of the destination device.  Device destination IPv4 addresses are either known or resolved by device name.  However, MAC addresses must be discovered.

A device uses Address Resolution Protocol (ARP) to determine the destination MAC address of a local device when it knows its IPv4 address

ARP provides two basic functions:

- Resolving IPv4 addresses to MAC addresses

- Maintaining a table of IPv4 address mappings.

### ARP Functions

When a packet is sent to the data link layer to be encapsulated into an Ethernet frame, the device refers to a table in its memory to find the MAC address that is mapped to the IPv4 address.  The table is stored temporarily in RAM memory and called the ARP table or the ARP cache.

The sending device will search its ARP table for a destination IPv4 address and a corresponding MAC address.

- If the packet’s destination IPv4 address is on the same network as the source IPv4 address, the device will search the ARP table for the destination IPv4 address.

- If the destination IPv4 address is on a different network than the source IPv4 address, the device will search the ARP table for the IPv4 address of the default gateway.

In both cases, the search is for an IPv4 address an a corresponding MAC address for the device.

Each entry, or row, of the ARP table binds an IPv4 address with a MAC address.  We call the relationship between the two values, a map.  This simply means that you can locate an IPv4 address in the table and discover the corresponding MAC address  The ARP table temporarily saves (caches) the mapping for the device on the LAN.

If the device locates the IPv4 address its corresponding MAC address is used as the destination MAC address in the frame  If there is no entry found, then the device sends an ARP request.

### ARP Request

An ARP request is sent when a device needs to determine the MAC address that is associated with an IPv4 address, and it does not have an entry or the IPv4 address in its ARP table.

ARP messages are encapsulated directly within an Ethernet frame.  There is no IPv4 header.  The ARP request is encapsulated in an Ethernet frame using the following header information:

- Destination MAC address – This is a broadcast address FF-FF-FF-FF-FF-FF requiring all Ethernet NICs on the LAN to accept and process the ARP request.

- Source MAC address – This is the MAC address of the sender of the ARP request.

- Type – ARP messages have a type field of 0x803.  This informs the receiving NIC that the data portion of the frame needs to be passe to the ARP process.

Because ARP requests are broadcasts, they are flooded out all ports by the switch, except the receiving port.  All Ethernet NICs on the LAN process broadcasts and must deliver the ARP requests to its operating system for processing.  Every device must process the ARP request to see if the target IPv4 address matches its own.  A router will not forward broadcasts out other interfaces.

Only one device on the LAN will have an IPv4 address that matches the target IPv4 address in the ARP request.  All other devices will not reply.

### ARP Operation – ARP Reply

Only the device with the target IPv4 address associated with the ARP request will respond with an ARP reply.  The ARP reply is encapsulated in an Ethernet frame using the following header information:

- Destination MAC address – This is the MAC address of the sender of the ARP request.

- Source MAC address – This is the MAC address of the sender of the ARP reply.

- Type – ARP messages have a type field of 0x806.  This informs the receiving NIC that the data portion of the frame needs to be passed to the ARP process.

Only the device that originally sent the ARP request will receive the unicast ARP reply.  After the reply is received, the device will add the IPv4 address and the corresponding MAC address to the ARP table.  Packets destined for that IPv4 address can now be encapsulated in frames using its corresponding MAC address.

If no device responds to the ARP request, the packet is dropped because a frame cannot be created.

Entries in the ARP table are time stamped.  If a device does not receive a frame from a particular device before the timestamp expires, the entry for this device is removed from the ARP table.

Additionally, static map entries can be entered in an ARP table, but this is rarely done.  Static ARP table entries do not expire over time and must be manually remove.

Note: IPv6 uses a similar process to ARP for IPv4, known as ICMPv6 Neighbor Discovery (ND).  IPv6 uses neighbors’ solicitation and neighbor advertisement messages, similar to IPv4 ARP requests and ARP replies.

### ARP Role in Remote Communications

When the destination IPv4 address is not on the same network as the source IPv4 address, the source device needs to send the frame to its default gateway.  This is the interface of the local router.  Whenever a source device has a packet with an IPv4 address on another network, it will encapsulate that packet in a frame using the destination MAC address of the router.

The IPv4 address of the default gateway is stored in the IPv4 configuration of the hosts.  When a host creates a packet for a destination, it compares the destination IPv4 address and its own IPv4 address to determine if the two IPv4 addresses are located on the same Layer 3 network.  If the destination host is not on its same network, the source checks its ARP tale for an entry with the IPv4 address of the default gateway.  If there is not an entry, it uses the ARP process to determine a MAC address of the default gateway.

### Removing Entries from an ARP Tables

For each device, an ARP cache timer removes ARP entries that have not been use or a specified period.  The times differ depending on the operating system of the device.  For example, newer Windows operating systems store ARP table entries between 15 and 45 seconds, as illustrated in the figure.

Commands may also be used to manually remove some or all of the entries in the ARP table. After an entry has been removed, the process for sending an ARP request and receiving an ARP reply must occur again to enter the map in the ARP table.

### ARP Tables on Networking Devices

On a Cisco router, the show ip arp command is used to display the ARP table, as shown in the figure.

On a Windows 10 PC, the arp -a command is used to display the ARP table, as shown in the figure.

### ARP Issues – ARP Broadcasts and ARP Spoofing

AS a broadcast frame, an ARP request is received and processed by every device on the local network.  On a typical business network, these broadcasts would probably have minimal impact on network performance.  However, if many o devices were to be powered up and all start accessing network services at the same time, there could be some reduction in performance for a short period of time, as shown in the figure.  After the devices send out the initial ARP broadcasts and have learned the necessary MAC addresses, any impact on the network will be minimized.

In some cases, the use of ARP can lead to a potential security risk.  A threat actor can use ARP spoofing to perform an ARP poisoning attack.  This is a technique used b a threat actor to reply to an ARP request for an IPv4 address that belongs to another device, such as the default gateway, as shown in the figure.  The threat actor sends an AP reply with its own MAC address.  The receiver of the ARP reply will add the wrong MAC address to its ARP table and send these packets to the threat actor.

Enterprise level switches include mitigation techniques known as dynamic ARP inspection (DAI). DAI is beyond the scope of this course.

## IPv6 Neighbor Discovery

### IPv6 Neighbor Discovery Messages

IPv6 Neighbor Discovery protocol is sometimes referred to as ND or NDP.  In this course, we will refer to it as ND.  ND provides address resolution, router discovery and redirection services for IPv6 using ICMPv6.  ICMPv6 ND uses five ICMPv6 messages to perform these services.

- Neighbor Solicitation messages

- Neighbor Advertisement messages

- Router Solicitation messages

- Router Advertisement messages

- Redirect message

Neighbor Solicitation and Neighbor Advertisement messages are used for device-to-device messaging such as address resolution (similar to ARP for IPv4).  Devices include both host computers and routers.

Router Solicitation and Router Advertisement messages are for messaging between devices and routers.  Typically router discovery is used for dynamic address allocation and stateless address autoconfiguration (SLAAC)

Note: The fifth ICMPv6 ND message is a redirect message which is use for better next-hop selection.  This is beyond the scope of this course.

IPv6 ND is defined in the IETF RFC 4861

### IPv6 Neighbor Discovery – Address Resolution

Much like ARP for IPv4, IPv6 devices use IPv6 ND to determine the MAC address of a device that has a known IPv6 address.

ICMPv6 Neighbor Solicitation and Neighbor Advertisement messages are used for MAC address resolution.  This is like ARP Request and ARP Replies used by ARP for IPv4.  For example, assume PC1 wants to ping PC2 at IPv6 address 2001:db8:acad::11.  To determine the MAC address for the known IPv6 address, PC1 sends an ICMPv6 Neighbor Solicitation message as illustrated in the figure.

ICMPv6 Neighbor Solicitation messages are sent using special Ethernet and IPv6 multicast addresses.  This allows the Ethernet NIC of the receiving devices to determine whether the Neighbor Solicitation message is for itself without having to send it to the operating system for processing.

PC2 replies to the request with an ICMPv6 Neighbor Advertisement message which includes its MAC address.

# Basic Router Configuration

## Configure Initial Router Settings

### Basic Router Configuration Steps

The following steps should be completed when configuring initial settings on a router

- Configure the device name:

  - Router(config)# hostname <hostname>

- Secure privileged EXEC mode:

  - Router(config)# enable secret <password>

- Secure user EXEC mode:

  - Router(config)# line console 0

  - Router(config-line)# password <password>

  - Router(config-line)# login

- Secure remote Telnet/SSH access:

  - Router(config-line)# line vty 04

  - Router(config-line)# password <password>

  - Router(config-line)# login

  - Router(config-line)# transport input { ssh | telnet }

- Secure all passwords in the config file:

  - Router(config-line)# exit

  - Router(config)# service password-encryption

  - Router(config)# banner motd <message>

- Save the configuration

  - Router(config)# end

  - Router# copy cunning-config startup-config

### Basic Router Configuration Example

In this example, Router R1 in the topology diagram will be configured with initial settings.

To configure the device name for R1, use the following commands.

- Router> enable

- Router# configure terminal

- Enter configuration commands, one per line.

- End with CNTL/Z

- Router(config)# hostname R1

- R1(config)#

Note: Notice how the router prompt will display the router hostname.

All router access should be secured.  Privileged EXEC mode provides the user with complete access to the device and its configuration.  Therefore, it is the most important mode to secure.

The following commands secure privileged EXEC mode and user exec mode enable the Telnet and SSH remote access and encrypt all plaintext (i.e. user EXEC and VTY line) passwords.

- R1(config)# enable secret class

- R1(config)#

- R1(config)# line console 0

- R1(config-line)# password cisco

- R1(config-line)# login

- R1(config-line)# exit

- R1(config)#

- R1(config)# line vty 0 4

- R1(config-line)# password cisco

- R1(config-line)# login

- R1(config-line)# transport input ssh telnet

- R1(config-line)# exit

- R1(config)#

- R1(config)# service password-encryption

- R1(config)#

The legal notification warns users that the device should only be accessed by permitted users.  Legal notification is configured as follows:

- R1 (config)# banner motd #

- Enter TEXT message. End wit a new line and the #

*****************************************

WARNING: Unauthorized access is prohibited!

*****************************************

#

- R1(config)#

If the previous commands were configured and the router accidentally lost power, all configured commands would be lost.  For this reason, it is important to save the configuration when changes are implemented.  The follow commands saves the configuration to NVRAM.

- R1# copy running-config startup-config

- Destination filename [startup-config]?

- Building configuration…

- [OK]

- R1#

## Configure Interfaces

### Configure Router Interfaces

At this point, your router have their basic configurations.  The next step is to configure their interfaces.  This is because routers are not reachable by end devices util the interfaces are configured.  There are many different types of interfaces available on Cisco routers.  For example, the Cisco ISR 4321 router is equipped with two Gigabit Ethernet interfaces:

- GigabitEthernet 0/0/0 (G0/0/0)

- GigabitEthernet 0/0/1 (G0/0/1)

The tasks to configure a router interface is very similar to management SVI o a switch.  Specifically, it includes issuing the following commands.

- Router(config)# interface <type-and-number>

- Router(config-if)# description <description-text>

- Router(config-if)# ip address <ipv4-address subnet-mask>

- Router(config-if)# ipv6 address <ipv6-address/prefix length>

- Router(config-if)# no shutdown

Note: When a router interface is enabled, information messages should be displayed confirming the enabled link

Although the description command is not required to enable an interface, it is good practice to use it.  It can be helpful in troubleshooting on production networks by providing information about the type of network connected. For example, if the interface connects to an ISP or service carrier, the description command would be helpful to enter the third-party connection and contact information.

Note: The description-text is limited to 240 characters.

Using the no shutdown command activate the interface and is similar to powering on the interface.  The interface must also be connected to another device, such as a switch or a router, for the physical layer to be active.

Note: On inter-router connections where there is no Ethernet switch, both interconnecting interfaces must be configured and enabled.

### Configure Router Interfaces Example

In this example, the directly connected interface of R1 in the topology diagram will be enabled.

To configure the interfaces on R1, use the following commands

- R1> enable

- R1# configure terminal

- Enter configuration commands, one per line.

- End with CNTL/Z.

- R1(config)# interface gigabitEthernet 0/0/0

- R1(config-if)# description Link to LAN

- R1(config-if)# ip address 192.168.10.1 255.255.255.0

- R1(config-if)# ipv6 address 2001:db8:acad:10::1/64

- R1(config-if)# no shutdown

- R1(config-if)# exit

- R1(config)#

- *Aug  1 01:43:53.435: %LINK-3-UPDOWN: Interface GigabitEthernet0/0/0, changed state to down

- *Aug  1 01:43:56.447: %LINK-3-UPDOWN: Interface GigabitEthernet0/0/0, changed state to up

- *Aug  1 01:43:57.447: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0, changed state to up

- R1(config)#

- R1(config)#

- R1(config)# interface gigabitEthernet 0/0/1

- R1(config-if)# description Link to R2

- R1(config-if)# ip address 209.165.200.225 255.255.255.252

- R1(config-if)# ipv6 address 2001:db8:feed:224::1/64

- R1(config-if)# no shutdown

- R1(config-if)# exit

- R1(config)#

- *Aug  1 01:46:29.170: %LINK-3-UPDOWN: Interface GigabitEthernet0/0/1, changed state to down

- *Aug  1 01:46:32.171: %LINK-3-UPDOWN: Interface GigabitEthernet0/0/1, changed state to up

- *Aug  1 01:46:33.171: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/1, changed state to up

- R1(config)#

Note: Notice the informational messages informing us that G0/0/0 and G0/0/1 are enabled

### Verify Interface Configuration

There are several commands that can be use to verify interface configuration.  The most useful of these is the show ip interface brief and show ipv6 interface brief commands, as shown in the example:

- R1# show ip interface brief

- Interface              IP-Address      OK? Method Status                Protocol

- GigabitEthernet0/0/0   192.168.10.1    YES manual up                    up

- GigabitEthernet0/0/1   209.165.200.225 YES manual up                    up

- Vlan1                  unassigned      YES unset  administratively down down

- R1# show ipv6 interface brief

- GigabitEthernet0/0/0       [up/up]

- FE80::201:C9FF:FE89:4501

- 2001:DB8:ACAD:10::1

- GigabitEthernet0/0/1       [up/up]

- FE80::201:C9FF:FE89:4502

- 2001:DB8:FEED:224::1

- Vlan1                      [administratively down/down]

- unassigned

- R1#

### Configuration Verification Commands

The table summaries the more popular show commands used to verify interface configuration:

| Show ip interface brief Show ipv6 interface brief | The output displays all interfaces, their ip addresses and their current status. The configured and connected interfaces should display a Status of “up” and Protocol of “up. Anything else with indicate a problem with either the configuration or the cabling |
| --- | --- |
| Show ip route Show ipv6 route | Displays the contents of the IP routing tables stored in RAM |
| Show interfaces | Displays the statistic for all interfaces on the device. However, this command will only display the IPv4 addressing information |
| Show ip interfaces | Displays the IPv4 statistics for all interfaces on a router. |
| Show ipv6 interfaces | Displays the IPv6 statistics for all interfaces on a router |

## Configure the Default Gateway

### Default Gateway on a Host

If your local network has only one router, it will be the gateway router and all hosts and switches on your network must be configured with this information.  If your local network has multiple routers, you must select one of them to be the default gateway router.  This topic explains how to configure the default gateway on hosts and switches.

For an end device to communicate over the network, it must be configured with the correct IP address information, including the default gateway address.  The default gateway is only used when the host wants to send a packet to a device on another network.  The default gateway address is generally the router interface address attached to the local network of the host.  The IP address of the host device and the router interface address must be in the same network.

For example, assume an IPv4 network topology consisting of a router interconnecting to separate LANs. G0/0/0 is connected to network 192.168.10.0, while G0/0/1 is connected to network 192.168.11.0.  Each host device is configured with the appropriate default gateway address.

In this example, if PC1 sends a packet to PC2, then the default gateway is note used.  Instead, PC1 addresses the packet with the IPv4 address of PC2 and forwards the packet directly to PC2 through the switch.

What if  PC1 sent a packet to PC3?  PC1 would address the packet with the IPv4 address of PC3, but would forward the packet to its default gateway, which is the G0/0/0 interface of R1.  The router accepts the packet and accesses its routing table to determine that G0/0/1 is the appropriate exit interface based on the destination address.  R1 then forwards the packet out of the appropriate interface to reach PC3.

The same process would occur on an IPv6 network, although this is not shown in the topology.  Devices would use the IPv6 address of the local router as their default gateway.

### Default Gateway on a Switch

A switch that interconnects client computers is typically a Layer 2 device.  As such, a Layer 2 switch does not require an IP address to function properly.  However, an IP configuration can be configured on a witch to give an administrator remote access to the switch.

To connect to and manage a switch over a local IP network, it must have a switch virtual interface (SVI) configured.  The SVI configured with an IPv4 address and subnet mask on the local LAN.  The switch must also have a default gateway address configured to remotely manage the switch from another network.

The default gateway address is typically configured on all devices that will communicate beyond their local network.

To configure an IPv4 default gateway on a witch, use the ip default-gateway <ip-address> global configuration command.  The ip-address that is configured is the IPv4 address of the local router interface connected to the switch.

The image shows an administrator establishing a remote connection to switch S1 on another network.

In this example, the administrator host would use its default gateway to send the packet to the G0/0/1 interface of R1. R1 would forward the packet to S1 out of its G0/0/0 interface.  Because the packet source IPv4 address came from another network, S1 would require a default gateway to forward the packet to the G0/0/0 interface of R1  Therefore, S1 must be configured with  default gateway to be able to reply and establish an SSH connection with the administrative host

Note: Packets originating from host computers connected to the switch must already have the default gateway address configured on their host computer operating systems.

A workgroup switch can also be configured with an IPv6 address on an SVI.  However, the switch does not require the IPv6 address of the default gateway to be configured manually.  The switch will automatically receive its default gateway from the ICMPv6 Router Advertisement message from the router.

# IPv4 Addressing

## IPv4 Address Structure

### Network and Host Portions

An IPv4 address is a 32-bit hierarchical address that is made up f a network portion and a host portion.  When determining the network portion versus the host portion, you must look at the 32-bit stream, as shown in the figure.

The bits within the network portion of the address must be identical for all devices that reside in the same network.  The bits within the host portion of the address must be unique to identify a specific host within a network.  If two hosts have the same bit-pattern in the specified network portion of the 32-bit stream, those two hosts will reside in the same network.

But how do hosts know which portion of the 32-bits identifies the network and which identifies the host?  This is the role of the subnet mask.

### The Subnet Mask

As show in the figure, assigning an IPv4 address to a host requires the following:

- IPv4 address – This is the unique IPv4 address of the host.

- Subnet Mask – This is use to identify the network/host portion of the IPv4 address.

IPv4 Configuration on a Windows Computer

Note: A default gateway IPv4 address is required to reach remote networks and DNS server IPv4 addresses are required t translate domain names to IPv4 addresses.

The IPv4 subnet mask is use to differentiate the network portion from the host portion of an IPv4 address.  When an IPv4 address is assigned to a device, the subnet mask is used to determine the network address of the device.  The network address represents all the devices on the same network.

The next figure displays the 32-bit subnet mask in dotted decimal and binary formats.

Notice how the subnet mask is a consecutive sequence of 1 bits followed by a consecutive sequence of 0 bits.

To identify the network and host portions of an IPv4 address, the subnet mask is compared to the IPv4 address bit for bit, from left to right as shown in the figure.

Associating a IPv4 Address with its Subnet Mask

Note that the subnet mask does not actually contain the network or host portion of the IPv4 address, it just tells the computer where to look for the part of the IPv4 address that is the network portion, and which part is the host portion.

The actual process used to identify the network portion and host portion is called ANDing.

### The Prefix Length

Expressing network addresses and host addresses with the dotted decimal subnet mask address can become cumbersome.  Fortunately, there is an alternative method of identifying subnet mask, a method called the prefix length.

The prefix length is the number of bit set to 1 in the subnet mask.  It is written in “slash notation”, which is noted by a forward slash (/) followed by the number of bits set to 1.  Therefore, count the number of bits in the subnet mask ad prepend it with a slash.

Refer to the table for examples.  The first column lists various subnet masks that can be used with a host address.  The second column displays the converted 32-bit binary address.  The last column displays the resulting prefix length.

| Subnet Mask | 32-bit Address | Prefix Length |
| --- | --- | --- |
| 255.0.0.0 | 11111111.00000000.00000000.00000000 | /8 |
| 255.255.0.0 | 11111111.11111111.00000000.00000000 | /16 |
| 255.255.255.0 | 11111111.11111111.11111111.00000000 | /24 |
| 255.255.255.128 | 11111111.11111111.11111111.10000000 | /25 |
| 255.255.255.192 | 11111111.11111111.11111111.11000000 | /26 |
| 255.255.255.224 | 11111111.11111111.11111111.11100000 | /27 |
| 255.255.255.240 | 11111111.11111111.11111111.11110000 | /28 |
| 255.255.255.248 | 11111111.11111111.11111111.11111000 | /29 |
| 255.255.255.252 | 11111111.11111111.11111111.11111100 | /30 |

Note: A network address is also required as a prefix or network prefix.  Therefore, the prefix length is the number of 1 bits in the subnet mask.

When representing an IPv4 address using a prefix length, the IPv4 address is written followed by the refix length with no spaces.  For example, 192.168.10.10 255.255.255.0 would be written as 192.168.10.10/24.  Sing various types of prefix lengths will be discussed later.  For ow, focus on the /24 (i.e. 255.255.255.0) prefix.

### Determining the Network – Logical AND

A logical AND is one of three Boolean operations used in Boolean or digital logic.  The other to are OR and NOT.  The AND operation is use to determine the network address.

Logical AND is the comparison of two bits that produce the results show below.  Now how only a 1 AND 1 produces a 1.  Any other combination results in a 0

- 1 AND 1 = 1

- 0 AND 1 = 0

- 1 AND 0 = 0

- 0 AND 0 = 0

Note:  In digital logic, 1 represents True an 0 represents False.  When using an AND operation, both input values must be True (1) for the result to be True (1).

To identify the network address of an IPv4 host, the IPv4 address is logically ANDed, bit by bit, with the subnet mask.  ANDing between the address and the subnet mask yields the network address.

To illustrate how AND is use to discover a network address, consider a host with IPv4 address 192.168.10.10 and subnet mask of 255.255.255.0, as shown in the figure:

- IPv4 host address (192.168.10.10) – The IPv4 address of the host in dotted decimal and binary formats.

- Subnet mask (255.255.255.0) – The subnet mask of the host in dotted decimal and binary formats.

- Network address (192.168.10.0) – The logical AND operation between the IPv4 address and subnet mask results in an IPv4 network address shown in dotted decimal binary formats.

Using the first sequence of bits as an example, notice the AND operation is performed on the 1-bit of the host address with the 1-bit of the subnet mask.  This results in a 1 bit for the network address.  1 AND 1 = 1.

The AND operation between and IPv4 host address and subnet mask results in the IPv4 network address for this host.  In this example, the AND operation between the host address of 192.168.10.10 ad the subnet mask 255.255.255.0 (/24), results in the IPv4 network address of 192.168.10.0/24.  This is an important IPv4 operation, as it tells the host what network it belongs to.

### Network, Host and Broadcast Addresses

Within each network are three types of IP addresses:

- Network address

- Host address

- Broadcast address

Using the topology in the figure, these three types of addresses will be explained.

Network address

A network address is an address that represents a specific network.  A device belongs to this network if it meets three criteria:

  - It has the same subnet mask as the network address

  - It has the same network bits as the network address, as indicated b the subnet mask.

  - It is locate on the same broadcast domain as other hosts with the same network address.

A host determines its network address by performing an AND operation between its IPv4 address and its subnet mask.

As shown in the table, the network address has all 0 bits in the host portion, as determined by the subnet mask.  In this example, the network address is 192.168.10.0/24.  A network address cannot be assigned to a device.

Host addresses

Host addresses are addresses that can be assigned to a device such as a host computer, laptop, smart phone, web camera, printer, router, etc.  The host portion of the address is the bits indicate by the 0 bits in the subnet mask.  Host addresses can have any combination of bits in the host portion except for all 0 bit (this would be a network address) or all 1 bits (this would be a broadcast address).

All devices within the same network, must have the same subnet mask ad the same network bits.  Only the host bits will differ and must be unique.

Notice that in the table, there is a first and last host address:

  - First host address – This first host within a network has all 0 bits with the last (right-most) bit as a 1 bit.  I this example, it is 192.168.10.1/24.

  - Last host address – This last host within a network has all 1 bits with the last (right-most) bit as a 0 bit.  In this example, it is 192.168.10.254/24.

Any addresses between and including, 192.168.10.1/24 through 192.168.10.254/24 can be assigned to a device on the network.

Broadcast address

A broadcast address is an address that is use when it is required to reach all devices on the IPv4 network.  As show in the table, the network broadcast address has all 1 bits in the host portion, as determined b the subnet mask.  In this example, the network address is 192.168.10.255/24.  A broadcast address cannot be assigned to a device.

## IPv4 Unicast, Broadcast and Multicast

### Unicast

In the previous topic you learned about the structure of an IPv4 address; each has a network portion and a host portion.  There are different ways to send a packet from a source device, and these different transmissions affect the destination IPv4 addresses.

Unicast transmission refers to one device sending a message to one other device in one-to-one communications.

A unicast packet has a destination IP address that is a unicast address which goes to a single recipient.  A source IP address can only be a unicast address, because the packet can only originate from a single source.  This is regardless o whether the destination IP address is a unicast, broadcast or multicast.

IPv4 unicast host addresses are in the address range of 1.0.01 to 223.255.255.255.  However, within this range are many addresses that are reserved for special purposes.  These special purpose addresses will be discussed later in this module.

### Broadcast

Broadcast transmission refers to a device sending a message to all the devices on a network in one-to- all communications.

A broadcast packet has a destination IP address with all ones (1s) in the host portion, or 32 one (1) bits).

Note: IPv4 uses broadcast packets.  However, there are no broadcast packets with IPv6.

A broadcast packet must be processed by all devices in the same broadcast domain.  A broadcast domain identifies all hosts on the same network segment.  A broadcast may be directed or limited.  A direct broadcast is sent to all hosts on a specific network.  For example, a host on the 172.16.4.0/24 network sends a packet to 172.16.4.255.  A limited broadcast is sent to 255.255.255.255.  By default, routers do not forward broadcasts.

Broadcast packets use resources on the network and make every receiving host on the network process the packet.  Therefore, broadcast traffic should be limited so that it does not adversely affect the performance of the network or devices.  Because routers separate broadcast domains, subdividing networks can improves performance by eliminating excessive broadcast traffic.

IP Directed Broadcasts

In addition to the 255.255.255.255 broadcast address, there is a broadcast IPv4 address for each network.  Called a directed broadcast, this address uses the highest address I the network, which is the address where all the host bits are 1s.  For example, the directed broadcast address for 192.168.1.0/24 is 192.168.1.255.  This address allows communication to all the hosts in that network.  To send data to all hosts in a network, a host can send a single packet that is addressed to the broadcast address of the network.

A device that is not directly connected to the destination network forwards an IP directed broadcast in the same way it would forward unicast IP packets to a host on that network.  When a directed broadcast packet reaches a router that is directly connected to the destination network, that packet is broadcast on the destination network.

Note: Because of security concerns and prior abuse from malicious users, directed broadcasts are turned off by default with Cisco IOS Release 12.0 with the global configuration command no ip directed-broadcasts

### Multicast

Multicast transmission reduces traffic by allowing a host to send a single packet to a selected set of hosts that subscribe to a multicast group.

A multicast packet is a packet with a destination IP address that is a multicast address.  IPv4 has reserved the 224.0.0.0 to 239.255.255.255 address as a multicast range.

Hosts that receive multicast packets are called multicast clients.  The multicast clients use services requested by a client program t subscribe multicast group.

Each multicast group is represented by a single IPv4 multicast destination address.  When an IPv4 host subscribes to a multicast group, the host processes packets addressed to this multicast address, and packets addressed to its uniquely allocated unicast address.

Routing protocols such as OSPF use multicast transmissions.  For example, routers enabled with OSPF communicate with each other using the reserved OSPF multicast address 224.0.0.5.  Only devices enabled with OSPF will process these packets with 224.0.0.5 as the destination IPv4 address.  All other devices will ignore these packets.

## Types of IPv4 Address

### Public and Private IPv4 Addresses

Just as there are different ways to transmit an IPv4 packet, there are also different types of IPv4 addresses.  Some IPv4 addresses cannot be used to go out to the internet, and other are specifically allocate for routing to the internet.  Some are used to verify a connection and others are self-assigned.  As a network administrator, you will eventually become very familiar with types of IPv4 addresses, but for now, you should at least know what they are and when to use them.

Public IPv4 addresses are addresses which are globally routed between internet service provider (ISP) routers.  However, not all available IPv4 addresses can be used on the internet.  There are blocks of addresses called private addresses that are use by most organization to assign IPv4 addresses to internal hosts.

In the mid-1990s, with the introduction of World Wide Web (WWW), private IPv4 addresses were introduce because of the depletion of IPv4 address space.  Private IPv4 addresses are not unique and can be used internally within any network.

Note: The long-term solution to IPv4 address depletion was IPv6

Note: Private addresses are defined in RFC 1918 and sometimes referred to a RFC 1918 address space.

### Routing to the Internet

Most internal networks, from large enterprises to home networks, use private IPv4 addresses for addressing all internal devices (intranet) including hosts and routers.  However, private address are not globally routable.

In the figure, customer networks 1, 2 and 3 are sending packets outside their internal networks.  These packets have a source IPV4 address that is a private address and a destination IPv4 address that is public (globally routable).  Packets with a private address must be filtered (discarded) or translated to a public address before forwarding the packet to an ISP

Before the ISP can forward this packet, it must translate the source IPv4 address, which is a private address, to a public IPv4 address using Network Adress Translation (NAT).  NAT is used to translate between private IPv4 and public IPv4 addresses.  This is usually done on the router that connects the internal network to the ISP network.  Private IPv4 addresses in the organization’s intranet will be translated to public IPv4 addresses before routing to the internet.

Note: Although, a device with a private IPv4 address is not directly accessible from another device across the internet, the IETF does not consider private IPv4 addresses or NAT as effective security measures.

Organizations that have resources available to the internet, such as a web server, will also have devices that have public IPv4 addresses.  As shown in the figure, this part of the network is known as the DMZ (demilitarized zone).  The router in the figure not only performs routing, it also performs NAT and acts as a firewall for security.

Note: Private IPv4 addresses are commonly use for educational purposes instead of using a public IPV4 address that most likely belongs to an organization.

### Special Use IPV4 Addresses

There are certain addresses, such as the network address and broadcast address, that cannot be assigned to hosts.  There are also special addresses that can be assigned to hosts, but with restrictions on how those hosts can interact with the network.

Loopback addresses

Loopback addresses (127.0.0.0 /8 to 127.255.255.254) are ore commonly identified as only 127.0.01, these are special addresses used by a host to direct traffic to itself.  For example, it can be used o a host to test if the TCP/IP configuration is operational, as shown in the figure.  Notice how the 127.0.0.1 loopback address replies to the ping command.  Also note how any address within this block will loop back to the local host, which is shown with the second ping in the figure.

Pinging the Loopback Interface

Link-Local Addresses

Link-local addresses (169.254.0.0 /16 or 169.254.0.1 to 169.254.255.254) are more commonly known as the Automatic Private IP Addressing (APIPA) addresses or self-assigned addresses.  They are used by a Windows DHCP client to self-configure in the event that there are no DHCP servers available.  Link-local addresses can be used in a peer-to-peer connect but are not commonly use for this purpose.

### Legacy Classful Address

In 1981, IPv4 addresses were assigned using classful addressing as defined in RFC 790, Assigned Numbers. Customers were allocated a network address based on one of three classes, A, B or C.  The RFC divided the unicast ranged into specific classes as follows:

- Class A (0.0.0.0/8 to 127.0.0.0/8) – Designed to support extremely large networks with more than 16 million host addresses.  Class A used a fixed /8 prefix with the first octet to indicate the network address and the remaining three octets for host addresses (more than 16 million host addresses per network).

- Class B (128.0.0.0/16 – 191.255.0.0 /16) – Designed to support the needs of moderate to large size networks with up to approximately 65 000 host addresses. Class B used a fixed /16 prefix with the two high-order octets to indicate the network address and the remaining two octets for host addresses (more than 65 000 host addresses per network).

- Class C (192.0.0.0/24 – 223.255.255.0/24) – Designed to support small network with a maximum of 254 hosts.  Class C used a fixed /24 prefix with the first three octets to indicate the network and the remaining octet for the host addresses (only 254 host addresses per network)

Note: There is also a Class D multicast block consisting of 224.0.0.0 to 239.0.0.0 and Class E experimental address block consisting of 240.0.0.0 – 255.0.0.0.

At the time, with a limited number of computers using the internet, classful addressing with an effective means to allocate addresses.  As shown in the figure, Class A and B networks have very large number of host addresses and Class C has very few.  Class A networks accounted for 50% of the IPv4 networks.  This cause most of the available IPv4 addresses to go unused.

In the mis-1990s, with the introduction of the World Wide Web (WWW), classful addressing was deprecated to more effectively allocate the limited IPv4 address space.  Classful address allocation was replaced with classless addressing, which is used today.  Classless addressing ignores the rules of classes (A, B, C).  Public IPv4 network addresses (network addresses and subnet masks) are allocate based on the number of addresses that can be justified.

### Assignment of IP Addresses

Public IPv4 addresses are addresses which are globally route over the internet.  Public IPv4 addresses must be unique.

Both IPv4 and IPv6 addresses are managed by the Internet Assigned Numbers Authority (IANA).  The IANA manages and allocates blocks of IP addresses to the Regional Internet Registries (RIRs).  The five RIRs are shown in the figure.

RIRs are responsible for allocating IP addresses to ISPs who provide IPv4 address blocks to organization and smaller ISPs.  Organizations can also get their addresses directly from an RIR (subject to the policies of that RIR).

Regional Internet Registries

## Network Segmentation

### Broadcast Domains and Segmentation

Have you ever received an email that was addressed to every person at your work or school?  This was a broadcast email.  Hopefully, it contained information that each of your needed to ko.  But often a broadcast is not pertinent to everyone in the mailing list.  Sometimes, only  a segment of the population needs to read that information.

In an Ethernet LAN, devices use broadcast and the Address Resolution Protocol (ARP) to locate other devices.  ARP sends Layer 2 broadcasts to a known IPv4 address on the local network to discover the associated MAC address.  Devices on Ethernet LANs also locate other devices using services.  A host typically acquires its IPv4 address configuration using the Dynamic Host Configuration Protocol (DHCP) which sends broadcasts on the local network to locate a DHCP server.

Switches propagate broadcasts out all interfaces except the interface on which it as received.  For example, if a switch in the figure were to receive a broadcast, it would forward it to the other switches and other users connected in the network.

Routers Segment Broadcast Domains

Routers do not propagate broadcasts.  When a router receives a broadcast, it does not forward it out other interfaces.  For instance, when R1 receives a broadcast on its Gigabit Ethernet 0/0 interface, it does not forward out another interface.

Therefore, each outer interface connects to a broadcast domain and broadcast are only propagated within that specific broadcast domain.

### Problems with Large Broadcast Domains

A large broadcast domain is a network that connects many hosts.  A problem with a large broadcast domain is that these hosts can generate excessive broadcasts and negatively affect the network.  In the figure, LAN 1 connects 400 users that could generate an excess amount of broadcast traffic.  This results in slow network operations due to the significant amount of traffic it can cause, and slow device operations because a device must accept and process each broadcast packet.

A Large Broadcast Domain

The solution is to reduce the size of the network to create smaller broadcast domains in a process called subnetting.  These smaller network spaces are called subnets.

In the figure, the 400 users in LAN 1 with network address 172.16.0.0 /16 have been divided into two subnets of 200 users each: 172.16.0.0 /24 and 172.16.1.0 /24. Broadcasts are only propagated within the smaller broadcast domains.  Therefore, a broadcast in Lan 1 would not propagate to LAN 2.

Communicating Between Networks

Notice how the prefix length has changed from a single /16 network to two /24 networks.  This is the basis of subnetting: using host bits to create additional subnets.

Note: The terms subnet and network are often used interchangeably.  Most networks are a subnet of some larger address block.

### Reasons for Segmenting Networks

Subnetting reduces overall network traffic and improves network performance.  It also enables an administrator to implement security policies such as which subnets are allowed or not allowed to communicate together.  Another reason is that it reduces the number of devices affected by abnormal broadcast traffic due to misconfigurations, hardware/software problems, or malicious intent.

There are various ways of using subnets to help manage network devices.

## Subnet an IPv4 Network

### Subnet on an Octet Boundary

In the previous topic you learned several good reasons for segmenting a network.  You also learned that segmenting a network is called subnetting.  Subnetting is a critical skill to have when administering an IPv4 network.  It is a bit daunting at first, but it gets much easier with practice.

Ipv4 subnets are created by using one or more of the host bits as network bits.  This is done by extending the subnet mask to borrow some of the bits from the host portion of the address to create additional network bits.  The more host bits that are borrowed, the more subnets that ca be define.  The more bits that are borrowed to increase the number of subnets reduces the number of hosts per subnet.

Networks are most easily subnetted at the octet boundary of /8, /16, and /24.  The table identifies these prefix lengths.  Notice that using longer prefix lengths decreases the number of hosts per subnet.

Subnet Masks on Octet Boundaries

To understand how subnetting on octet boundary can be useful, consider the following example.  Assume an enterprise has chosen the private address 10.0.0.0/8 address at the octet boundary of /16 as shown in the table.   This would provide the enterprise the ability to define up to 256 subnets (i.e., 10.0.0.0/16 – 10.255.0.0/16_ with each subnet capable of connecting 65 534 hosts.  Notice how the first two octets identify the network portion of the address whereas the last two octets are for host IP addresses.

Subnetting Network 10.0.0.0/8 using a /16

Alternatively, the enterprise could choose to subnet the 10.0.0.0/8 network at the /24 octet boundary, as shown in the table.  This would enable the enterprise to define 65 536 subnets each capable of connecting 254 hosts.  The /24 boundary is very popular in subnetting because it accommodates a reasonable number of hosts and conveniently subnets at the octet boundary.

Subnetting Network 10.0.0.0/8 using a /24 Prefix

### Subnet within an Octet Boundary

The examples shown thus far borrowed host bits from the common, /8, /16, and /24 network prefixes.  However, subnets can borrow bits from any host bit position to create other masks.

For instance, a /24 network address is commonly subnetted using longer prefix lengths by borrowing bits from the fourth octet.  This provides the administrator with additional flexibility when assigning network addresses to a smaller number of end devices.

Refer to the table to see six ways to subnet a /24 network.

For each bit borrowed in the fourth octet, the number of subnetworks available is double, while reducing the number of host addresses per subnet:

- /25 row – Borrowing 1 bit from the fourth octet creates 2 subnets supporting 126 hosts each.

- /26 row – Borrowing 2 bits creates 4 subnets supporting 62 hosts each.

- /27 row – Borrowing 3 bits creates 8 subnets supporting 30 hosts each.

- /28 row – Borrowing 4 bits creates 16 subnets supporting 14 hosts each.

- /29 row – Borrowing 5 bits creates 32 subnets supporting 6 hosts each.

- /30 row – Borrowing 6 bits creates 64 subnets supporting 2 hosts each.

## Subnet a Slash 16 and a Slash 8 Prefix

### Create Subnet with a Slash 16 prefix

Some subnetting is easier than other subnetting.  This topic explains how to create subnets that each have the same number of hosts.

In a situation requiring a larger of subnets, an IPv4 network is required that has more host bits available to borrow.  For example, the network address 172.16.0.0 has a default mask of 255.255.0.0, or /16.  This address has 16 bits in the network portion and 16 bits in the host portion.  The 16 bits in the host portion are available to borrow for creating subnets.  The table highlights all the possible scenarios for subnetting a /16 prefix.

Subnet a /16 Network

Although you do not need to memorize this table, you still need a good understanding o how each value in the table is generated.  Do not let the size of the table intimidate you.  The reason it is big is that it has 8 additional bits that can be borrowed, and, therefore, the numbers o subnets and hosts are simply larger.

### Create 100 Subnets with a Slash 16 prefix

Consider a large enterprise that requires at least 100 subnets and has chosen the private address 172.16.0.0/16 as its internal network address.

When borrowing bits from a /16 address, start borrowing bits I the third octet, going from left to right.  Borrow a single bit at a time until the number of bits necessary to create 100 subnets is reached.

The figure displays the number of subnets that can be created when borrowing bits from the third octet and fourth octet.  Notice there are now up to 14 host bits that can borrowed.

Number of Subnets Created

To satisfy the requirements of 100 subnets for the enterprise, 7 bits (i.e., 2^7 = 128 subnets) would need to be borrowed (for a total of 128 subnets), as shown in the figure.

Recall that the subnet mask must change to reflect the borrowed bits.  In this example, when 7 bits are borrowed, the mask is extended 7 bits into the third octet.  In decimal, the mask is represented as 255.255.254.0, or a /23 prefix, because the third octet is 11111110 in binary and the fourth octet is 00000000 in binary.

The figure displays the resulting subnets from 172.16.0.0/23 up to 172.16.254.0 /23

Resulting /23 Subnets

After borrowing 7 bits for the subnet, there is one host bit remaining in the third octet, and 8 host bits remaining in the fourth octet, for a total of 9 bits that were not borrowed. 2^9 results in 512 total host addresses.  The first address is reserved for the network address, and the last address is reserved for the broadcast address, so subtracting for these two addresses (2^9 -2) equals 510 available host addresses for each /23 subnet.

As shown in the figure, the first host address for the first subnet is 172.16.0.1, and the last host address is 172.16.1.254.

Address Range for 172.16.0.0/23 Subnet

### Create 1000 Subnets with a Slash 8 prefix

Some organizations, such as small service providers or large enterprises, may need eve more subnets.  For example, take a small ISP that requires 1000 subnets for its clients.  Each client will need plenty of space in the host portion to create its own subnets.

The ISP has a network address 10.0.0.0 255.0.0.0 or 10.0.0.0/8.  This means there are 8 bits in the network portion and 24 host bit available to borrow towards subnetting.  Therefore, the small ISP will subnet the 10.0.0.0/8 network.

To create subnets, you must borrow bits from the host portion of the IPv4 address of the existing internetwork.  Starting from the left to the right with the first available host bit, borrow a single bit at a time until you reach the number of bits necessary to create 1000 subnets.  As shown in the figure, you need to borrow 10 bits to create 1024 subnets (2^10 = 1024).  This includes 8 bits in the second octet and 2 additional bits from the third octet.

Number of Subnets Created

This figure displays the subnets resulting from borrowing 10 bits, creating subnets from 10.0.0.0/18 to 10.255.192.0/18

Resulting /18 Subnets

Borrowing 10 bits to create the subnets, leaves 14 host bits for each subnet.  Subtracting two hosts per subnet (one for the network address and one for the broadcast address) equates to 2^14 – 2 = 16382 hosts per subnet.  This means that each of the 1000 subnets can support up to 16 382 hosts.

This figure displays the specifics of the first subnet.

Address Range for 10.0.0.0/18 Subnet

## Subnet to Meet Requirements

### Subnet Private versus Public IPv4 Address Space

While it is nice to quickly segment a network into subnets, your organization’s network may use both public and private IP4 addresses.  This affects how you will subnet your network.

The figure shows a typical enterprise network:

- Intranet – This is the internal part of a company’s network, accessible only within the organization.  Devices in the intranet use private IPv4 addresses

- DMZ – This is part of the company’s network containing resources available to the internet such as a web server.  Devices in the DMZ use public IPv4 addresses.

Public and Private IPv4 Address Space

Both the intranet and the DMZ have their own subnetting requirements and challenges.

The intranet uses private IPv4 addressing space.  This allows an organization to use any o the private IPv4 network addresses including the 10.0.0.0/8 prefix with 24 host bits and over 16 million hosts.  Using a network address with 24 hosts bits makes subnetting easier and more flexible.  This includes subnetting on an octet boundary using a /16 or /24.

For example, the private IPv4 network address 10.0.0.0/8 can be subnetted using a /16 mask.  As shown in the table, this results in 256 subnets, with 65 534 hosts per subnet.  If an organization has a need for fewer than 200 subnets, allowing for some growth, this gives each subnet more than enough host addresses.

Subnetting Network 10.0.0.0/8 using a /16

Another option using the 10.0.0.0/8 private IPv4 network address is to subnet using a /24 mask.  As shown in the table, this results in 65 536 subnets, with 254 hosts per subnet.  If an organization needs more than 256 subnets, then using a /24 can be used with 254 hosts per subnet.

Subnetting Network 10.0.0.0/8 using a /24

The 10.0.0.0/8 can also be subnetted using any other number of prefix lengths, such as /12, /18, /20, etc.  This would give the network administrator a wide variety of options.  Using a 10.0.0.0/8 private IPv4 network address makes subnet planning and implementation easy.

What about the DMZ?

Because these devices need to be publicly accessible from the internet, the devices in the DMZ require public IPv4 addresses  The depletion of public IPv4 address space became an issue beginning in the mid-1990s.  Since 2011, IANA and four out of five RIRs have run out of IPv4 address space remains severally limited.  This means an organization must maximize its own limited number of public IPv4 addresses.  This requires the network administrator to subnet their public address space into subnets with different subnet masks, to minimize the umber of unused host addresses per subnet masks, to minimize the number of unused host addresses per subnet.  This is known as Variable Subnet Length Masking (VLSM).

### Minimize Unused Host IPv4 Addresses and Maximize Subnets

To minimize the number of unused host IPv4 addresses and maximize the number of available subnets, there are two considerations when planning subnets: The number of host addresses required for each network and the number of individual subnets needed.

The table displays the specifics for subnetting a /24 network.  Notice how there is an inverse relationship between the number of subnets and the number of hosts.  The more bits that are borrowed to create subnets, the fewer host bits remain available.  If more hosts addresses are needed, more host bits are required, resulting in ewer subnets.

The number of host addresses required in the largest subnet will determine how many bits must be left in the host portion.  Recall that two of the addresses cannot be use, so the usable number of addresses can be calculated as 2n-2

Subnetting a /24 Network

Network administrators must devise the network addressing scheme to accommodate the maximum number of hosts for each network of subnets.  The addressing scheme should allow for growth in both the number of host addresses per subnet and the total number of subnets.

### Efficient IPv4 Subnetting

In this example, corporate headquarters has been allocated a public network address of 172.16.0.0/22 (10 host bit) by its ISP.  As shown in the figure, this will provide 1 022 host addresses.

Note: 172.16.0.0/22 is part of the IPv4 private address space.  We are using this address inside of an actual public IPv4 address

The corporate headquarters has a DMZ and four branch offices, each needing its own public IPv4 address space.  Corporate headquarters needs to make the best use of its limited IPv4 address space.

The topology shown in the figure consists of five sites; a corporate office and four branch sites.  Each site requires internet connectivity and therefore, five internet connections.  This means that the organization requires 10 subnets from the company’s 172.16.0.0/22 public address  The largest subnet requires 40 addresses.

The 172.16.0.0/22 network address has 10 host bits, as shown in the figure  Because the largest subnet requires 40 hosts, a minimum of 6 host bits are needed to provide addressing for 40 hosts.  This is determined by using this formula: 2^6 – 2 = 62 hosts.

Using the formular for determining subnets results in 16 subnets: 2^4 = 16.  Because the example internetwork requires 10 subnets, this will meet the requirement and allow for some additional growth.

Therefor, the first 4 host bits can be use to allocate subnets.  This means two bits from the 3rd octet and two bits from the 4th octet will be borrowed.  When 4 bits are borrowed from the 172.16.0.0/22 network, the new prefix length is /26 with a subnet mask of 255.255.255.192.

As shown in the figure, the subnets can be assigned to each location and router-to-ISO connections

## VLSM

### IPV4 Address Conservation

Because of the depletion of public IPv4 address space, making the most out of the available host addresses is a primary concern when subnetting IPv4 networks.

Note: The larger IPv6 address allows for much easier address planning and allocation than IPv4 allows.  Conserving IPv6 addresses is not an issue.  This is one of the driving forces for transitioning to IPv6

Using traditional subnetting, the same number of addresses is allocated for each subnet.  If all the subnets have the same requirements for the number of hosts, or if conserving IPv4 address space is not an issue, these fixed-sized address blocks would be efficient.  Typically, with public IPv4 addresses, that is not the case.

For example, the topology shown in the figure requires seven subnets, one for each of the four LANs, and one for each of the three connections between the routers.

Using traditional subnetting with the given address of 192.168.20.0/24, three bits can be borrowed from the host portion in the last octet to meet the subnet requirement of seven subnets.  As show in the figure, borrowing 3 bits creates 8 subnets and leaves 5 host bits with 30 usable hosts per subnet.  This scheme creates the needed subnets and meets the host requirements of the largest LAN.

These seven subnets could be assigned to the LAN and WAN networks, as shown in the figure.

Although this traditional subnetting meets the needs of the largest LAN and divides the address space into an adequate number of subnets, it results in significant waste of unuse addresses.

For example, only two addresses are needed in each subnet for the three WAN links.  Because each subnet has 30 usable addresses, there are 28 unused addresses in each of these subnets.  As shown in the figure, this results in 84 unused addresses.

Further, this limits future growth by reducing the total number of subnets available.  This inefficient use of addresses is characteristic of traditional subnetting.  Applying a traditional subnetting scheme to this scenario is not very efficient and is wasteful.

The variable-length subnet mask (VLSM) was develop to avoid wasting addresses by enabling us to subnet a subnet.

### VLSM

In all of the previous subnetting example, the same subnet mask was applied for all the subnets.  This means that each subnet has the same number of available host addresses.  As illustrated in the left side of the figure, traditional subnetting creates subnets of equal size.  Each subnet in a traditional scheme uses the same subnet mask.  As shown on the right side of the figure, VLSM allows a network space to be divided into unequal parts.  With VLSM, the subnet mask will vary depending on how many bits have been borrowed for a particular subnet, this the “variable” part of the VLSM.

VLSM is just subnetting a subnet.  The same topology use previously is shown in the figure.  Again, we will use the 192.168.20.0/24 network and subnet it for seven subnets, one for each of the four LANs and one for each of the three connections between the routers.

The figure shows how network 192.168.20.0/24 subnetted into eight equal-sized subnets with 30 usable host addresses per subnet.  Four subnets are use for the LANs and three subnets are use for the connections between the routers.

However, the connections between the routers requires only two host addresses per subnet (one host address for each router interface).  Currently all subnets have 30 usable host addresses per subnet.  To avoided wasting 28 addresses per subnet, VLSM can be used to create smaller subnets for the inter-router connections.

To create smaller subnets for the inter-router links, one of the subnets will be divided.  In this example, the last subnet 192.168.20.224/27, will be further subnetted.  The figure shows the last subnet has been subnetted further by using the subnet mask 255.255.255.252 or /30.

Why /30?  Recall that when the number of needed host addresses is known, the formular 2n-2 (where n equals the number of host bits remaining) can be used.  To provide two usable addresses, two host bits must be left in the host portion.

Because there are five bits in the subnetted 192.168.20.224/27 address space, three more bits can be borrowed, leaving two bits in the host portion.  The calculations at this point are the same as those use for traditional subnetting.  The bits are borrowed, and the subnet ranged are determine.  The figure shows how the four /27 subnets have been assigned to the LANs and three of /30 subnets have been assigned to the inter-router links.

This VLSM subnetting scheme reduces the number of addresses per subnet to a size appropriate for the networks that require fewer subnets.  Subnetting subnet 7 for the inter-router links, allows subnets 4, 5 and 6 to be available for future networks, as well as five additional subnets available for inter-router connections.

Note: When using VLSM, always begin by satisfying the host requirements of the largest subnet.  Continue subnetting until the host requirements of the smallest subnet are satisfied.

### VLSM Topology Address Assignment

Using the VLSM subnets, the LAN and inter-router networks can be addressed without unnecessary waste.

The figure shows the network address assignments and the IPv4 addresses assigned to each router interface.

Using a common addressing scheme, the first host IPv4 address for each subnet is assigned to the LAN interface of the router.  Hosts on each subnet will have a host IPv4 address from the range of host addresses for that subnet and an appropriate mask.  Hsots will use the address of the attached router LAN interface as the default gateway address

The table shows the network addresses and range of the host addresses for each network.  The default gateway address is displayed for the four LANs.

## Structured Design

### IPv4 Network Address Planning

Before you start subnetting, you should develop an IPv4 addressing scheme for your entire network.  You will need to know how many subnets you nee, how many hosts a particular subnet requires, what devices are part of the subnet, which parts of your network use private addresses, and which is public, and many other determining factors.  A good addressing scheme allows for growth.  A good addressing scheme is also the sign of a good network administrator.

Planning IPv4 network subnets requires you to examine both the needs of an organization’s network usage, and how the subnets will be structures.  Performing a network requirement study is the starting point.  This means looking at the entire network, both the intranet and the DMZ, and determining how each area will be segmented.  The address plan includes determining how each area will be segmented.  The address plan includes determining where address conservation is needed (usually within the DMZ), and where there is more flexibility (usually within the intranet).

Where address conservation is required, the plan should determine how many subnets are needed and how many hosts per subnets.  As discussed earlier, this is usually required for public IPV4 address space within the DMZ.  This will most likely include using VLSM.

Within the corporate intranet, address conservation is usually less of an issue.  This is largely due to using private IPv4 addressing, including 10.0.0.0/8, with over 16 million host IPv4 addresses.

For most organizations, private IPv4 addresses allow for more than enough internal (intranet) addresses.  For many larger organizations and ISPs, even private IPv4 address space is not large enough to accommodate their internal needs.  This is another reason why organizations are transitioning to IPv6.

For intranets that use private IPv4 addresses and DMZs that use public IPv4 addresses, address planning and assignment is important.

Where require, the address plan includes determining the needs of each subnet in terms of size.  How many hosts there will be per subnet?  The address plan also needs to include how host addresses will be assigned, which hosts will require static IPv4 addresses, and which hosts can use DHCP for obtaining their addressing information.  This will also help prevent the duplication of addresses, while allowing for monitoring and managing of addresses for performance and security reasons.

Knowing your IPv4 address requirements will determine the range, or ranges, of host addresses that you implement and help ensure that there are enough addresses to cover your network needs.

### Device Address Assignment

Within a network, there are different types of devices that requires addresses:

- End user clients – Most networks allocate IPv4 addresses to client devices dynamically, using Dynamic Host Configuration Protocol (DHCP).  This reduces the burden on network support staff and virtually eliminates entry errors.  With DHCP, addresses are only lease for a period of time and can be reused when the lease expires.  This is an important feature for networks that support transient users and wireless devices  Changing the subnetting scheme means that the DHCP server needs to be reconfigured, and the clients must renew their IPv4 addresses.  Ipv6 clients can obtain address information using DHCPv6 or SLAAC

- Servers and peripherals – These should have a predictable static IP address.  Use a consistent numbering system for these devices.

- Servers that are accessible from the internet – Servers that need to be publicly available on the internet must have a public IPv4 address, most often accessed using NAT.  In some organizations, internal servers (not publicly available) must be made available to the remote users.  In most cases, these servers are assigned private addresses internally, and the user is required to create a virtual private network (VPN) connection to access the server.  This has the same effect as if the user is accessing the server from a host within the intranet.

- Intermediary devices – These devices are assigned addresses for network management, monitoring and security.  Because we must know how to communicate with intermediary devices, they should have predictable, statically assigned addresses.

- Gateway – Routers and firewall devices have an IP address assigned to each interface which serves as the gateway for the hosts in that network.  Typically, the router interface uses either the lowest or highest address in the network.

When developing an IP addressing scheme, it is generally recommended that you have a set pattern of how addresses are allocated to each type o device.  This benefits administrators when adding and removing devices, filtering traffic based on IP, as well as simplifying documentation.

# IPv6 Addressing

## IPv4 Issues

### Need for IPv6

You already know that IPv4 is running out of addresses.  That is why you need to learn about Ipv6.

IPv6 is designed to be the successor to IPv4.  IPv6 has a larger 128-bit address space, providing 340 undecillion (340 followed by 36 zeroes) possible addresses.  However, IPv6 is more than just larger addresses.

When the IETF began its development of a successor to IPv4, it use this opportunity to fix the limitations of IPv4 and include enhancements.  One example is Internet Control Message Protocol version 6 (ICMPv6), which includes address resolution and address autoconfiguration not found in ICMP for IPV4 (ICMPv4).

The depletion of IPv4 address spaces has been the motivating factor moving to IPv6.  As Africa, Asia and other areas of the world become more connected to the internet, there are not enough IPv4 addresses to accommodate this growth.  As shown in the figure, all five RIRs have run out of IPv4 addresses.

IPv4 has a theoretical maximum of 4.3 billion addresses.  Private addresses in combination with Network Address Translation (NAT) have been instrumental in slowing the depletion of IPv4 address space.  However, NAT is problematic for many applications, creates latency, and has limitations that severely impede peer-to-peer communications.

With the ever-increasing number of mobile devices, mobile providers have been leading the way with the transition to IPv6.  The tope two mobile providers in the United States report that over 0% of their traffic is over IPv6.

Most ISPs and content providers such as YouTube, Facebook and Netflix, have also made the transition.  Many companies like Microsoft, Facebook and LinkedIn are transitioning to IPv6-only internally.  In 2018, broadband ISP Comcast reported a deployment of over 65% and British Sy Broadcasting over 86%.

Internet of Things

The internet of today is significantly different than the internet of past decades.  The internet of today is more than email, web pages, and fire transfers between computers.  The evolving internet is becoming an Internet of Things (IoT).  No longer will the only devices accessing the internet be computers, tablets and smartphones.  The sensor-equipped, internet-ready devices of tomorrow will include everything from automobiles and biomedical devices to household appliances and natural ecosystems.

With an increasing internet population, a limited IPv4 address space, issues with NAT and the IoT, the time has come to begin with the transition of IPv6.

### IPv4 and IPv6 Coexistence

There is no specific date to move to IPv6.  Both IPv4 and IPv6 will coexist in the near future, and the transition will take several years.  The IETF has created various protocols and tools to help network administrators migrate their networks to IPv6.  He migration techniques can be divided into three categories.

Dual Stack

Dual stack allows IPv4 and IPv6 to coexist on the same network segment.  Dual stack devices run both IPv4 and IPv6 protocol stacks simultaneously.  Known as native IPv6, this means the customer network has an IPv6 connection to their ISP and I able to access content found on the internet over IPv6

Tunneling

Tunneling is a method of transporting an IPv6 packet over an IPv4 network.  The IPv6 packet is encapsulated inside an IPv4 packet, like other types of data.

Translation

Network Address Translation 64 (NAT64) allows IPv6-enable devise to communicate with Pv4-enabled devices using a translation technique similar to NAT for IPv4.  An IPv6 packet is translate to an IPv4 packet and IPv4 packet is translated into an IPv6 packet.

Note: Tunnelling and translation are for transitioning to native IPv6 and should only be used where needed.  The goal should be native IPv6 communications from source to destination.

## IPv6 Address Representation

### IPv6 Addressing Formats

The first step to learning about IPv6 in networks is to understand the way an IPv6 address is written and formatted. IPv6 addresses are much larger than IPv4 addresses, which is why we are unlikely to run out of them.

IPv6 addresses are 128 bits in length and written as a string o hexadecimal values.  Every four bits is represented by a single hexadecimal digit; for a total of 32 hexadecimal values, as shown in the figure.  IPv6 addresses are not case-sensitive and can be written in either lowercase or uppercase.

Preferred Format

The previous figure also shows that the preferred format for writing an IPv6 address is x:x:x:x:x:x:x:x, with each “x” consisting of four hexadecimal values.  The term octet refers to the eight bits of an IPv4 address.  In IPv6, a hextet is the unofficial term use to refer to a segment of 16 bits, or four hexadecimal values, each “x” is a single hextet which is 16 bits or four hexadecimal digits.

Preferred format means that you write IPv6 address using all 32 hexadecimal digits.  It does not necessarily mean that it is the ideal method for representing the IPv6 address.  In this module, you will see two rules that help to reduce the number of digits needed to represent an IPv6 address.

These are examples of IPv6 addresses in the preferred format

### Rule 1 – Omit Leading Zeros

The first rule to help reduce the notation of IPv6 addresses is to omit any leading 0s (zeros) in any hextet.  Here are four examples of ways to omit leading zeros:

- 01ab can be written as 1ab

- 09f0 can be written as 9f0

- 0a00 can be written as a00

- 00ab can be written as ab

Note: This rule only applies to leading 0s, NOT to trailing 0s, otherwise the address would be ambiguous.  Or example, the hextet “abc” could be either “0abc” or “abc0”, but these do not represent the same value.

### Rule 2 – Double Colon

The second rule to help reduce the notation of IPv6 addresses is that a double colon (::) can replace any single, contiguous string of one or more 16-bit hextets consisting of all zeros.  For example, 2001:db8:café:1:0:0:0:1 (leading 0s omitted) can be represented as 2001:db8:cafe::1.  The double colon is used in place of the three all-0 hextets (0:0:0).

The double colon can only be use once within an address, otherwise there would be more than one possible resulting addresses.  When use with the omitting leading 0s technique, the notation of IPv6 address can often be greatly reduced.  This is commonly known as “compressed format”.

If an address has more than one contiguous string of all-0 hextets, best practice is to use the double colon (::) on the longest string.  If the strings are equal, the first string should use the double colon (::).

## IPv6 Address Types

### Unicast, Multicast, Anycast

As with Ipv4, there are different types of IPv6 addresses.  In fact, there are three broad categories of IPv6 addresses.

- Unicast – An IPv6 unicast address uniquely identifies an interface on an IPv6-enable device.

- Multicast – An IPv6 multicast address is used to send a single IPv6 packet to multiple destinations.

- Anycast – An IPv6 anycast address is any IPv6 unicast address that can be assigned to multiple devices.  A packet sent to an anycast address is routed to the nearest device having that address.  Anycast addresses are beyond the scope of this course.

Unlike IPv4, IPv6 does not have a broadcast address.  However, there is an IPv6 all-nodes multicast address that essentially gives the same result.

### IPv6 Prefix Length

The prefix, or network portion, of an IPv4 address can be identified by a dotted-decimal subnet mask or prefix length (slash notation).  For example, an IPv4 address of 192.168.1.10 with dotted-decimal subnet mask 255.255.255.0 is equivalent to 192.168.1.10/24.

In IPv6 it is only called the prefix length.  IPv6 does not use the dotted-decimal subnet mask notation.  Lie IPv4, the prefix length I represented in slash notation and is used to indicate the network portion of an IPv6 address.

The prefix length can range from 0 to 128.  The recommended IPv6 prefix length for LANs and most other types of networks is 64, as shown in the figure

It is strongly recommended to use a 64-bit interface ID for most networks.  This is because Stateless Address Autoconfiguration (SLAAC) uses 64 bits for the interface ID.  It also makes subnetting easier to create and manage.

### Types of IPv6 Unicast Addresses

An IPv6 unicast address uniquely identifies an interface on an IPv6-enabled device.  A packet sent to a unicast address is received by the interface, which is assigned that address  Similar to IPv4, a source IPv6 address must be a unicast address.  The destination IPv6 address can be either a unicast or a multicast address.  The figure shows the different types of IPv6 unicast addresses.

Unlike IPv4 devices that only have a single address, IPv6 addresses typically have two unicast addresses:

- Global Unicast Address (GUA) – This is similar to a public IPv4 address.  These are globally unique, internet-routable addresses.  GUAs can be configured statically or assigned dynamically.

- Link-Local Address (LLA) – This is required for every IPv6-enabled device.  LLAs are used to communicate with other devices on the same local link.  With IPv6, the term link refers to a subnet.  LLAs are confined to a single link.  Their uniqueness must only be confirmed on that link because they are not routable beyond the link.  In other words, routers will not forward packets with a link-local source or destination address.

### A Note About the Unique Local Addresses

Unique local addresses (range fc00::/7 to fdff::/7) are not yet commonly implemented.  Therefore, this module only covers GUA and LLA configuration.  However, unique local addresses may eventually be used to address devices that should not be accessible from the outside, such as internal servers and printers.

The IPv6 unique local addresses have some similarity to RFC 1918 private addresses for IPv4, but there are significant differences:

- Unique local addresses are used for local addressing within a site or between a limited number of sites.

- Unique local addresses can be use for devices that will never need to access another network.

- Unique local addresses are not globally routed or translated to a global IPv6 address.

Note: Many sites also use the private nature of RFC 1918 addresses to attempt to secure or hide their network from potential security risks.  However, this was never the intended use of these technologies, and the IETF has always recommended that sites take the proper security precautions on their internet-facing router.

### IPv6 GUA

IPv6 global unicast addresses (GUAs) are globally unique and routable on the IPv6 internet.  These addresses are equivalent to public IPv4 addresses. The Internet Committee for Assigned Names and Numbers (ICANN), the operators for IANA, allocates IPv6 address blocks to the five RIRs. Currently, only GUAs with the first three bits of 001 or 2000::/3 are being assigned, as shown in the figure.

The figure shows the range of values for the first hextet where the first hexadecimal digit for currently available GUAs begin with a 2 or a 3.  This is only 1/8th of the total available IPv6 address space, excluding only a very small portion for other types of unicast and multicast addresses.

Note: The 2001:db8::/32 address has been reserved for documentation purposes, including use in examples.

The next figure shows the structure and range of a GUA.

### IPv6 GUA Structure

Global Routing Prefix

The global routing prefix is the prefix, or network, portion of the address that is assigned by the provider, such as an ISP, to a customer or site.  For example, it is common for ISPs to assign a /48 global routing prefix to its customers.  The global routing prefix will usually vary depending on the policies of the ISP.

The previous figure shows a UA using a /48 global routing prefix. /48 prefixes are a common global routing prefix that is assigned ad will be used I most of the examples throughout this course.

For example, the IPv6 address 2001:db8:acad:/48 has a global routing prefix that indicates that the first 48 bits (3 hextets) (2001:db8:acad) is how the ISP knows of this prefix (network).  The double colon (::) following the /48 prefix length means the rest of the address contains all 0s.  The size of the global routing prefix determines the size of the subnet ID.

Subnet ID

The Subnet ID field is the area between the Global Routing Prefix and the Interface ID.  Unlike IPv4 where you must borrow bits from the host portion to create subnets, IPv6 was designed with subnetting in mind.  The Subnet I is used by an organization to identify subnets within the site.  The larger the subnet ID, the more subnets available.

Note: Many organizations are receiving a /32 global routing prefix.  Using the recommended /64 prefix in order to create a 64-bit Interface ID, leaves a 32-bit Subnet ID.  This means an organization with a /32 global routing prefix and a 32-bit Subnet ID will have 4.3 billion subnets, each with 18 quintillion devices per subnet.  This is as many subnets as there are public IPv4 addresses.

The IPv6 address in the previous figure has a /48 Global Routing Prefix, which is common among many enterprise networks.  This makes it especially easy to examine the different parts of the address.  Using a typical /64 prefix length, the first four hextets are for the network portion of the address, with the fourth hextet indicating the Subnet ID.  The remaining four hextets are for the Interface ID.

Interface ID

The IPv6 interface ID is equivalent to the host portion of an IPv4 address.  The term Interface ID is used because a single host may have multiple interfaces, each having one or mote IPv6 addresses.  The figure shows an example of the structure of an IPv6 GUA.  It is strongly recommended that in most cases /64 subnets should be used, which creates a 64-bit interface ID.  A 64-bit interface ID allows for 18 quintillion devices or hosts per subnet.

A /64 subnet or prefix (Global Routing Prefix + Subnet ID) leaves 64 bits for the interface ID.  This is recommended to allow SLAAC-enabled devices to create their own 64-bit interface ID.  It also makes developing an IPv6 addressing plan simple and effective.

Note: Unlike IPv4, in IPv6, the all-0s and all-1s host addresses can be assigned to a device.  The all-1s address can be use because broadcast addresses are not used within IPv6.  The all-0s address can also be used but is reserved as a Subnet-Router anycast address and should be assigned only to routers.

### IPv6 LLA

An IPv6 Link-Local Address (LLA) enables a device to communicate with other IPv6-enabled devices on the same link and only on that link (subnet).  Packets with a source or destination LLA cannot be routed beyond the link from which the packet originated.

The GUA is not a requirement.  However, every IPv6-enabled network interface must have an LLA.

If an LLA is not configured manually on an interface, the device will automatically create its own without communication with a DHCP server.  IPv6-enabled hosts create a Pv6 LLA even if the device has not been assigned a global unicast IPv6 address.  This allows IPv6-enabled devices to communicate with other IPv6-enabled devices on the same subnet.  This includes communication with the default gateway (router).

IPv6 LLAs are in the fe80::/10 range.  The /10 indicates that the first 10 bits are 1111 1110 10xx xxxx.  The first hextet has a range of 1111 1110 1000 0000 (fe80) to 1111 1110 1011 1111 (febf).

The figure shows an example of communication using IPv6 LLAs.  The PC is able to communicate directly with the printer using the LLAs.

The next figure shows some of the uses for IPv6 LLAs

Note: Typically, it is the LLAS of the router, and not the GUA, that is used as the default gateway for other devices on the link.

There are two ways that a device can obtain an LLA:

- Statically – This means the device has been manually configured.

- Dynamically – This means the device creates its own Interface ID by using randomly generated values or using the Extended Unique Identifier (EUI) method, which uses the client MAC address along with additional bits.

## GUA and LLA Static Configuration

### Static GUA Configuration on a Router

As learned in the previous topic, IPv6 GUAs are the same as public IPv4 addresses.  They are globally unique and routable on the IPv6 internet.  An IPv6 LLA lets two IPv6-enabled devices communicate with each other on the same link (subnet).  It is easy to statistically configure IPv6 Guas and LLAs on routers to help you create an IPv6 network.  This topic teaches you how to do just that.

Most IPv6 configuration and verification commands in the Cisco IOS are similar to the IPv4 counterparts.  In many cases, the only difference is the use of “ipv6” in place of “ip” within the commands.

For example, the Cisco IOS command to configure an IPv4 address on an interface is ip address ip-address subnet-mask. In contrast, the command to configure an IPv6 GUA on an interface is ipv6 address ipv6-address/prefix-length.

Notice that there is no space between ipv6-address and prefix-length.

The example configuration uses the topology shown in the figure and these IPv6 subnets.

The example shows the commands required to configure the IPv6 GUA on GigabitEthernet 0/0/0, GigabitEthernet 0/0/1, and the Serial 0/1/0 interface of R1

IPv6 GUA Configuration on Router R1

### Static GUA Configuration on a Windows Host

Manually configuring the IPv6 address on a host is similar to configuring an IPv5 address.

As show in the figure, the default gateway address configured for PC1 is 2001:db8:acad:1::1.  This is the GUA of the R1 GigabitEthernet interface on the same network.  Alternatively, the default gateway address can be configured to match the LLA of the GigabitEthernet interface.  Using the LLA of the router as the default gateway address is considered best practice.  Either configuration will work.

Just as with IPv4, configuring static addresses on clients does not scale to larger environments.  For this reason, most network administrators in an IPv6 network will enable dynamic assignment of IPv6 addresses.

There are two ways in which a device can obtain an IPv6 GUA automatically:

- Stateless Address Autoconfiguration (SLAAC)

- Stateful DHCPv6

SLAAC and  DHCPv6 are covered in the next topic.

Note: When DHCPv6 or SLAAC is used, the LLA of the router will automatically be specified as the default gateway address.

### Static Configuration of a Link-Local Unicast Address

Configuring the LLA manually lets you create an address that is recognizable and easier to remember  Typically, it is only necessary to create recognizable LLAs on routers.  This is beneficial because router LLAs are used as default gateway addresses and in routing advertisement messages.

LLAs can be configured manually using the ipv6 address ipv6-link-local-adress link-local command  When an address begins with this hextet within the range of fe80 to febf, the link-local parameter must follow the address.

The figure below shows an example topology on each interface.

The example shows the configuration of an LLA on router R1

Statically configured LLAs are used to make them more easily recognizable as belonging to router R1.  In this example, all the interfaces of router R1 have been configured with an LLA that begins with fe80::n:1.

Note: The exact same LLA could be configured on each link as long as it is unique on that link.  This is because LLAs only have to be unique on that link.  However, common practice is to create a different LLA on each interface of the router to make it easy to identify the router and the specific interface.

## Dynamic Addressing for IPv6 GUAs

### RS and RA Messages

If you do not ant to statically configure IPv6 GUAs, no nee to worry.  Most devices obtain their IPv6 GUAs dynamically.  This topic explains how this process works using Router Advertisement (RA) and Router Solicitation (RS) messages.  This topic gets rather technical, but when you understand the difference between the three meths that a router advertisement can use, as well as how the EUI-64 process for creating an interface ID differs from a randomly generated process, you will have made a huge leap in your IPv6 expertise.

For the GUA, a device obtains the address dynamically through Internet Control Message Protocol version 6 (ICMPv6) messages.  IPv6 routers periodically send out ICMPv6 RA messages, every 200 seconds, to all IPv6-enabled devices on the network.  An RA message will also be sent in response to a host sending an ICMPv6 RS message, which is a request for an RA message.  Both messages are shown in the figure.

RA messages are on IPv6 router Ethernet interfaces.  The router must be enabled for IPv6 routing, which is not enabled by default.  To enable a router as an IPv6 router, the ipv6 unicast-routing global configuration command must be use.

The ICMPv6 RA message is a suggestion to a device on ho to obtain an IPv6 GUA.  The ultimate decision is up to the device operation system.  The ICMPv6 RA message includes the following:

- Network prefix and prefix length – This tells the device which network it belongs to.

- Default gateway address – This is an IPv6 LLA, the source IPv6 address of the RA message.

- DNS addresses and domain name – These are the addresses of DNS servers and a domain name.

There are three methods for RA messages:

- Method 1: SLAAC – “I have everything you need, including the prefix, prefix length and default gateway address.”

- Method 2: SLAAC with a stateless DHCPv6 server – “Here is my information, but you need to get other information such as DNS addresses from a stateless DHCPv6 server.”

- Method 3: Stateful DHCPv6 (no SLAAC) – “I can give you your default gateway address.  You need to ask a stateful DHCPv6 server for all your other information.

### Method 1 – SLAAC

SLAAC is a method that allows a device to create its own GUA without the services of DHCPv6.  Using SLAAC, devices rely on the ICMPv6 RA messages of the local router to obtain the necessary information.

By default, the RA messages suggests that the receiving device use the information in the RA message to create its own IPv6 GUA and all other necessary information.  The services of a DHCPv6 server are not required.

SLAAC is stateless, which means there is no central server (for example, a stateful DHCPv6 server) allocating GUAs and keeping a list of devices and their addresses.  With SLAAC, the client device uses the information in the RA message to create its own GUA.  As shown in the figure, the two parts of the address are created as follows:

- Prefix – This is advertised in the RA message.

- Interface ID – This uses the EUI-64 process or by generating a random 64-bit number, depending on the device operating system

- The router sends an RA message with the prefix for the local link

- The PC uses SLAAC to obtain a prefix from the RA message and creates its own interface ID

### Method 2 – SLAAC and Stateless DHCPv6

A router interface can be configured to send a router advertisement using SLAAC and stateless DHCPv6.

As shown in the figure, with this method, the RA message suggests devices use the following:

- SLAAC to create its own IPv6 GUA

- The router LLA, which is the RA source IPv6 address, as the default gateway address.

- A stateless DHCPv6 server to obtain other information such as a DNS server address and a domain name.

Note: A stateless DHCPv6 server distributes DNS server addresses and domain name.  It does not allocate GUAs

### Method 3 – Stateful DHCPv6

A router interface can be configured to send an RA using stateful DHCPv6 only.

Stateful DHCPv6 is similar to DHCP for IPv4.  A device can automatically receive its addressing information including a GUA, prefix length and the addresses of DNS servers from a stateful DHCPv6 server.

As shown in the figure, with this method, the RA message suggests devices use the following:

- The router LLA, which is the RA source Pv6 address, for the default gateway address.

- A stateful DHCPv6 server to obtain a GUA, DNS server address, domain name and other necessary information.

A stateful DHCPv6 server allocates and maintain a list of which device receives which IPv6 address.  DHCP for IPv4 is stateful.

Note: The default gateway address can only be obtained dynamically from the RA message.  The stateless or stateful DHCPv6 server does not provide the default gateway address.

### EUI-64 Process Vs. Randomly Generated

When the RA message is either SLAAC or SLAAC with stateless DHCPv6, the client must generate its own interface ID.  The client knows the prefix portion of the address from the RA message, but must create its own interface ID.  The Interface I can be created using the EUI-64 process or a randomly generated 64-bit number, as shown in the figure.

### EUI-64 Process

IEEE define the Extended Unique Identifier (EUI) or modified EUI-64 process.  This process uses the 48-bit Ethernet MAC address of a client and inserts another 16 bits in the middle of the 48-bit MAC address to create a 64-bit interface ID.

Ethernet MAC addresses are usually represented in hexadecimal and are made up of two parts:

- Organizationally Unique Identifier (OUI) – The OUI is a 24-bit (6 hexadecimal digits) vendor code assigned by IEEE

- Device Identifier – The device identifier is a unique 24-bit (6 hexadecimal digits) value within a common OUI.

An EUI-64 interface ID is represented in binary and is made up of three parts:

- 24-bit OUI from the client MAC address, but the 7th bit (the Universally/locally (U/L)bit) is reversed.  This means that if the 7th bit is a 0, it becomes a 1 and vice versa.

- The inserted 16-bit value fffe (in hexadecimal)

- 24-bit Device Identifier from the client MAC address.

The EUI-64 process is illustrated in the figure, using the R1 Gigabit Ethernet MAC address of fc99:4775:cee0

The example output for the ipconfig commands shows the IPv6 GUA being dynamically create using SLAAC and the EUI-64 process.  AN easy way to identify that an address was probably created using EUI-64 is the fffe located in the middle of the interface ID.

The advantage of EUI-64 is that the Ethernet MAC address can be use to determine the interface ID.  It also allows network administrators to easily track an IPv6 address to an end-device using the unique Mac address.  However, this has caused privacy concerns among many users who worried that their packets could be trace to the actual physical computer.  Due to these concerns, a randomly generated interface ID may be used, instead.

### Randomly Generated Interface IDs

Depending on the operating system, a device may use a randomly generated interface ID instead of using the MAC address and the EUI-64 process. Beginning with Windows Vista.  Windows uses a randomly generated interface ID instead of one created with EUI-64.  Windows XP and previous Windows operating system used EUI-64.

Ater the interface ID is established, either through the EUI-64 process or through random generation, it can be combined with an IPv6 prefix in the RA message to create a GUA, as shown in the figure.

Note: To ensure the uniqueness of any IPv6 unicast address, the client may use a process known as Duplicate Address Detection (DAD).  This is similar to an ARP request for its own address.  If there is no reply, then the address is unique.

## Dynamic Addressing for IPv6 LLAs

### Dynamic LLAs

### Dynamic LLAs on Windows

### Dynamic LLAs on Cisco Routers

### Verify IPv6 Address Configuration

# OSYS1200 Introduction to Windows Administration

## The Evolving Role of System Administrators in the Future of Computing

### Introduction

System administrators have historically been the gatekeepers of IT infrastructure.  Their role has been pivotal in ensuring the smooth operation and security of computer systems.  However, with the rapid advancements in technology, the responsibilities of system administrators are evolving.  This essay will explore how the role of system administrators is changing and what the future holds for this critical profession.

### The Impact of Cloud Computing

Cloud computing has revolutionized the way businesses and organizations utilize IT resources.  The shit towards cloud based infrastructure has significantly altered the traditional responsibilities of system administrators.  Instead of managing physical servers and data centers, system administrators now often oversee cloud environments, ensuring optimal performance, security, and cost-efficiency.  This transition requires new skills and expertise, such as cloud platform knowledge and automation techniques.

### Automation and DevOps

Automation is another major trend reshaping the role of system administrators.  DevOps practices, which emphasize collaboration between development and operations teams, have gained widespread adoption.  Automation tools and scripting languages are being used to automate routine tasks, allowing system administrators to focus on more strategic and complex challenges.  This shift towards automation can improve efficiency, reduce errors and accelerate innovation.

### Security and Risk Management

As cyber threats continue to evolve, the responsibility of system administrators to protect sensitive data has become paramount.  They must stay informed about emerging security threats and implement appropriate measures to safeguard systems and networks.  This includes tasks such as vulnerability assessment, intrusion detection, and incident response.  Additionally, system administrators play a crucial role in managing risks associate with data privacy and compliance regulations.

### Emerging Technologies and Skill Requirements

The future of computing is marked by the emergence of new technologies, such as artificial intelligence, machine learning and the internet of things (IoT).  These technologies present both opportunities and challenges for system administrators. They must adapt their skill sets to understand and manage these emerging technologies effectively.  This may involve learning about AI frameworks, data analytics and IoT security protocols.

### Conclusion

The role of system administrators is undergoing a significant transformation in response to technological advancements.  The shift towards cloud computing, automation and emerging technologies require system administrators to develop new skills and adopt new approaches.  By embracing these changes and staying ahead of the curve, system administrators can continue to play a vital role in shaping the future of computing and ensuring the successful operation of IT infrastructure.

## Windows: Using System and Utilities

### Settings Overview

Most configuration options for Windows 11 can be accessed through “Settings”.

To access “Settings”: Click the Start Button > Click Settings

### Administrative Tools

Administrative tools are a collection of system configuration utilities found in System and Security in Control Panel.

Most tools in this category use the Microsoft Management Console (MMC). Microsoft Management Console is a framework that simplifies the development of administrative tools.

The tools include:

| Component Services | Performance Monitor |
| --- | --- |
| Defragment and Optimize Drives | Print Management |
| Disk Cleanup | Resource Monitor |
| Event Viewer | Service |
| iSCSI Initiator | System Configuration |
| Local Security Policy | Task Scheduler |
| Open Database Connectivity (ODBC) | Windows Firewall with Advanced Security |
| Windows Memory Diagnostics |  |

### Microsoft Management Console

Microsoft Management Console (MMC) is a graphical interface shell that provides a structured environment to build management utilities. Network administrators use MMC consoles with snap-ins to perform management tasks. Each console can host one or more snap-ins.

A snap-in is a component that adds control mechanisms to the MMC console for a specific service or object.

You can create a customized MMC console. You can do this by adding the snap-ins you want to a single console and then saving the console as a .msc file.

You can share .msc files between other users and computers.

The MMC console is divided into three sections:

| System Tools | Storage | Services and Applications |
| --- | --- | --- |
| Task Scheduler | Disk Management | Services |
| Event Viewer |  | WMI Control |
| Shared Folders |  |  |
| Local Users and Groups |  |  |
| Performance |  |  |
| Device Manager |  |  |

### Services

Services is a type of Windows application that runs in the background without user interactions.  Typically performs tasks for other software applications.

Services Administration Tool is used to manage Windows 11 services.

Service information:

- Name

- Description

- Status

- Start-up type

- Log On As

Properties of a service:

- General

- Log On

- Recovery

- Dependencies

### Command-Line Administration Tools

Command-Line Administration tools include Command Prompt and Windows PowerShell. These are “text-based” interfaces that require you to type in all commands.

Command Prompt:

Windows PowerShell:

A command-line interface (CLI) for performing system configuration in Windows 11.

- It is an effective replacement for batch scripts and Visual Basic scripts.

- Some things you can do with Windows PowerShell include:

  - Manage services

  - Manage processes

  - Manage networking

  - Edit the registry

  - Manipulate files and folders

  - Retrieve event log events

## Installing and Configuring Apps

### The Windows App Landscape

Types of Windows Applications

- Packages Apps

  - Modern apps often from Microsoft Store

  - Run in a sandboxed environment for security

  - User experience can scale based on the device

- Progressive Web Apps (PWAs)

  - Websites that can be installed like native apps

  - Work offline

  - Run independently of the browser

- Traditional Desktop Applications

  - Executable files (.exe, .msi)

  - Installed directly onto the system

### Managing Apps and Processes with Task Manager

- Open Task Manager

- Processes Tab

  - Lists running applications, background processes and Windows processes.

  - Displays resource usage (CPU, Memory, Disk, Network)

  - “End Task” option to close unresponsive applications.

  - Grouping by type (default) or a single list (View > Group By Type).

- PWAs manage options identical to Store apps.  Accessing PWSs via edge://apps

### Managing Startup Apps

- Accessing the Startup apps list in Task Manager.

- Viewing the impact of startup apps on system performance

- Disabling unnecessary startup apps to improve boot time

## Managing Local and Cloud Storage/Managing Disk and File Types

### Disk Technology

Can be categorized by

  - How it is connected to the computer

  - How it is presented in Windows 11

When reviewing disk technology for use with Windows 11, consider these technologies:

  - Internal Disk

    - Electromechanical drives

    - Solid-state drives (SSD)

    - Custom embedded chip memory (flash memory)

  - External Disk

    - Used to connect removable disk storage. Avoid using ext. disks as a location for operating systems

  - Virtual Hard Disk (VHD)

    - All versions of Windows 11 support VHD operations.  Files can be stored in a VHD storage location just like any other disk technology.

  - Multiple disks as one logical disk.

    - Logical disks appear to the Windows 11 operating system as if it is one disk drive

    - Can combine multiple disks as one logical disk.

### Disk Partition Styles

Windows 11 can organize data on disk drives using a partition style.  When a blank disk is first configured for use by Windows, you can select one of these styles:

- GUID Partition Table (GPT)

- Master Boot Record (MBR)

In Windows Recovery, use the command-line tool MBR2GPT to upgrade from BIOS.MBR to UEFI/GPT

## Disk Types

Empty space on the drive can be organized using three different methods in Windows 11

- Basic disks

  - This is hard disks initialized to use basic storage technology.  Basic disks can have their space organized into one or more defined areas of storage call partitions.  These are defined by the size and type of data it holds.  The partition’s attributes are stored in a data table on the disk that is part of the MBR or GPT specifications.  This is called the Partition Table.

- Dynamic disks

  - Dynamic disks provide flexibility to logically organize disk space across one or more disk drives.  Blocks of space are call volumes.

- Storage spaces

  - Storage Spaces are an inexpensive way to combine, or pool, storage of multiple physical disk drives.  These can also provide fault tolerance with data redundancy across multiple disks.  Software-based storage technology that doesn’t need specialized storage hardware.

### Disk Management Tools

Windows 11 tools for Disk Management

- Disk Management Console

  - MMC Console Snap-in (diskmgmt.msc)

  - Part of the Computer Management utility

  - Allows a member of the Administrators group to observe and make changes to the computer’s disk configuration.

  - Allows changes to be made interactively

    - Usually takes effect immediately without restart

- DiskPart

  - Command-line tool

    - Allows disk, partition, volume and virtual hard disk management operations to be performed

      - From a text-based screen interactively

      - From a scripted file

  - Operations are driven by a sequence of commands

    - As part of a scheduled task

    - As an automated response on the local computer or remotely on another computer

- Storage Cmdlets in Windows PowerShell

### Adding a New Drive

New drives are typically connected via SATA or via USD

A brand-new drive is typically not initialized by Windows.

### Configuring Storage Spaces Fault Tolerance

A storage space volume can be configured for fault tolerance when it is initially created if the proper starting conditions are met.  Different levels of tolerance are available.

| Resiliency Type | Description |
| --- | --- |
| Simple (no resilience) | - Needs one drive minimum in the pool - Good for temporary data and where performance is more important than resilience. - If a drive in the pool fails and contains part of the this volume, the volume is not available to the end user |
| Two-way Mirror | - Needs two drives minimum - A volume’s data is copied to at least two separate disks in the pool; each disk holds a synchronized copy of the volume’s data - Good performance with disk read and write operations - Can handle single drive failure and still be operational. - Supported volume size is limited to the size of the smallest drive member |
| Three-way Mirror | - Needs five drives minimum - Three copies of a volume’s data are spread across all five drives - Good performance with disk read and write operations. - Can handle multiple drive failures and still be operational. |
| Parity | - Similar to RAID 5 in that parity information is calculated and can determine data that is lost from a drive failure. - Needs three drives minimum to handle a single drive failure. - Needs seven drives minimum to handle a double drive failure - Slow performance during a disk failure while the system calculates missing data; best used for low data that needs fault tolerance but not high performance, such as archive data. |

## User Accounts, Passwords and Credentials and User Management

### User Profiles

A collection of desktop and environment configurations for specific user or group of users.

By default, each user has a separate profile stored in C:\Users

Profile folders and information

| 3D objects | Local Setting | SendTo |
| --- | --- | --- |
| AppData | Music | Start Menu |
| Application Data | My Documents | Templates |
| Contacts | NetHood | Videos |
| Cookies | OneDrive | NTUSER.DAT |
| Desktop | Pictures | NTUSER.DAT.LOG |
| Documents | PrintHood | NTUSER.DAT[guid} TM.bif |
| Downloads | Recent | Ntuser.ini |
| Favorites | Saved Games |  |
| Links | Searches |  |

### The Default Profile

Default profile is created when a new user profile is created.

Windows copies the default user profile to create a profile for the new user.

To configure the default profile:

- Create new local user with admin privileges

- Sign in as the designated local user

- Modify the new user’s profile as desired

- Create an answer file with CopyProfile parameter set to true

- Run Sysprep with the /generalize option

- Image the computer and deploy the image

### Mandatory Profile

This profile cannot be modified.

Users can make changes to their desktop settings while they are logged on but are not saved.

Most mandatory profiles are implemented as roaming user profiles.

To change a profile to a mandatory profile, you rename the file NTUSER.DAT to NTUSER.MAN

### Roaming Profile

Roaming profile

- Stored in a network location rather than on the local hard drive

- Settings move with a user from computer to computer on the network

To configure  roaming profile

- You must edit the user account to point the profile directory at a network location.

Each time a user signs in

- A roaming profile is copied to the local computer

### Public Profile

Public Profile

- Different from other profiles because it is not a complete profile

- Does not include NTUSER.DAT file an consequently does not include any registry settings

Public Profile Folders

- Libraries

- Public Account Pictures

- Publick Desktop

- Public Documents

- Public Downloads

- Public Music

- Public Pictures

- Public Videos

## Windows Security and Privacy

## Managing Hardware Devices/Application Support

### Application Environments

Apps are written to interact with specific application environment subsystems

  - Application environment subsystem provides access to operating system services through an application programming interface (API)

All the application environments in Windows restrict apps to running in user mode

  - Where they are isolated from the core functionality of the OS

  - Ensures a poorly written app does not affect system stability

### The Registry

The Registry is a central store for application and OS configuration information

In most cases, you do not need to manually edit or view the registry.

The Registry is divided into sections and levels of data.

Multiple sections exist to organize data by purpose.

  - Individual sections are called hives

Within a single hive, data is stored in keys and values

### Registry Structure: Registry Hives

| HKEY_CLASSES_ROOT | This hive defines file types (classes) and properties associated with those types. For example, file type associations are stored here. This hive is a combination of HKCU\Software\Classes and HKLM\Software\Classes. If settings for a file type are defined in both locations, the user-specific settings take precedence. |
| --- | --- |
| HKEY_CURRENT_USER | This hive contains the user-specific registry information for ntuser.dat stored in the user profile. Any application or operating system settings that are user specific are stored in this hive. If you are attempting to fix a user-specific application issue, the support document will have you review and modify keys here. |
| HKEY_LOCAL_MACHINE | This hive contains global settings for the entire computer and the applications installed on it. If you are attempting to fix any operating system or application issue that is not user specific, the support document will have you review and modify keys here. Ore details about HKLM are provided later in this module. |
| HKEY_USERS | This hive contains the user-specific settings for the current user and several system services The name of the registry key for the currently signed-in user is the security identifier (SID). The content of the key of the currently signed-in user is also available in HKCU |
| HKEY_CURRENT_CONFIG | This hive contains details about the current hardware profile in use. The details report the differences between the standard configuration defined the HKL\System and HKLM\Software and those in the active hardware profile. This hive is also a mapped view to information stored in HKLM\System\CurrentControlSet\Hardware\Profile\Current. |

### Registry Editing Tools

- Back up the portion of the registry you will be changing before you make any changes

- Restrict the number of changes made at one time to limit and identify which change worked

- When possible use a test system rather than a user’s computer to verify changes to resolve the issue

- If you are adjusting registry entries for services for drivers, ensure that the computer can boot properly after changes have been made.

- Graphical Registry editor

- Can use this tool to:

  - Search the registry and modify permissions

  - Export and import sections of registry

## Performance Power

### Troubleshooting Windows Errors

When using the Windows Troubleshooting tool, the more information you can provide about a problem, the more likely you can find a solution.  Use precise search terms in a search engine to locate additional information.

### Steps Recorder

When you are unable to visit or view the desktop that is experiencing errors, ask the user of the workstation to record steps with Step Recorder.

Step Recorder will take a screenshot each time the user clicks on a screen item.  It is then saved in a report (.mht file) that can be emailed or saved to a shared storage location.

Step Recorder does not capture text that has been typed.  Users will need to add comments to the report for any text typed as part of the troubleshooting.

### Reliability Monitor

The Reliability Monitor rates the system stability and lets you monitor events that contribute to said stability.  It collects the following data:

- Software installs and uninstalls

- Application failures

- Windows failures

- Miscellaneous failures.

Reliability Monitor

### Event Viewer

The Event Viewer is used to browse and manage events stored in system even logs.

You can start the Event Viewer as a stand-alone administrative tool, or as part of Computer Management.

Administrative Event view shows the following events from all logs:

- Critical

- Error

- Warning

Other log nodes include:

- Application

- Security

- System

- Application and Services

- Custom View Node

- Subscription Node

- Forwarded Events

Event Viewer

By default, when you view an event log, you see all of the events listed in it.  It can be useful to limit the visible events to only those you are interested in. You can search for specific keywords by using the find option.  The search applies to all text in the events for a single log.  You can filter individual logs by time, event level, event source, even ID, keyword, user and computer to find the events you want to see.

### System Recovery

If a computer does not boot, you can perform a system recovery.  There are to options for this; Reset this PC and advanced start up

### Reset This PC

If you are unable to repair a computer, Windows gives you the option to Reset this PC.  This will remove all apps and put the settings to their default values.  You will have the option to keep your data files and settings.

This can be done from a local re-install or a cloud download.

### Windows Recovery Environment (WinRE)

The WinRE is a small instance of Windows that includes several options for troubleshooting and repairing Windows.  Options from the Advanced Startup Menu are:

- Use a device

- Reset this PC

- Startup Repair

- Startup Settings

- Uninstalling Updates

- UEFI Firmware Settings

- System Restore

- Command Prompt

- System Image Recovery

## Windows Terminal/PowerShell

## Managing Windows PC in Enterprise

### Active Directory

The Active Directory is a centralized directory service that can be used to control a network of Windows computers.  Computers are one to a domain and a computer account is created in the domain.  User accounts are managed, centrally, in the domain rather than on each computer.

Domain controllers are servers that hold a copy of Active Directory Information. They authenticate users when they sign in to a workstation and respond to requests for other domain information, such as printer information or application configuration.

### Active Directory Structure

The domain is the central security database used by all computers that are members of that domain.  Information about user accounts and computers are stored here.  Active Directory uses the same naming conventions for domains and objects as DNS.

Organizational Units (Ous) help organize domain information.  Each domain can be subdivided into OUs.  They can be use for delegating management permissions and apply Group Policies.

You can create more complex Active Directory structures by combining multiple domains into a tree, and multiple trees into a forward.  When multiple domains exist in a forest, trust relationships are automatically generated among the domains.

Within an Active Directory, servers can either be member servers or domain controllers.  These are Server Roles.  Member servers are integrated into the Active Directory and can participate in the domain by sharing files and printers with domain users.

### Active Directory Partitions

Active Directory is divided into manageable units:

- Domain

- Configuration

- Schema

The global catalog server is a domain controller that holds a subset of the information in all domain partitions.

### Active Directory Sites and Replication

Active Directory uses multimaster replication.  This means that information can be changed on any domain controllers.  The changes are then replicated to other controls.  The Active Directory sites are defined by IP subnets.  Within a site, Active Directory replication is uncontrolled.  While between sites, replication is controlled by site links.

### Active Directory and DNS

One of the more common configuration problems in Active Directory networks is the incorrect configuration of DNS on servers and workstations.  Incorrect DNS configuration can result in slow user sign-ins, inability to apply group policies and failed replication among domain controllers.

### Joining a Domain

When a workstation joins a domain, it is integrated into the security structures for the domain.  Administration can be done centrally, using Group Policy.  Security changes occur when domain admin groups become a member of a local admin group, when domain users become a member of local user groups and when domain guests become a member of local guest groups.  Joining a workstation to a domain creates a computer account.

### Time Synchronization

Time Synchronization is a critical part of Active Directory authentication.  If the clock on a computer running windows is more than 5 minutes different from the domain controller, authentication to the Active Directory will fail.

### Offline Domain Join

This allows Windows computer to be joined to an Active Directory without ever directly communicating with the domain controller.  The high level steps to perform an online domain join are as follows:

- Run djoin.exe on Windows Server with the /provision option to create a computer account and a blob file.  The blob file contains authentication information for the Windows computer.

- Copy the blob file to the Windows computer

- Run djin.exe on the Windows computer with the /requestodj option to import the blob file.

- Reboot the apply the changes.

### Group Policy

Group Policy is integrated with the Active Directory and centrally manages the configuration of the Windows computer.  Settings used for this are contained in a Group Policy Object (GPO).  The GPO is a collection of registry settings applied to the computer.  These settings are applied to any user accounts in OU to which the GPO is linked.  Computer settings are also applied.

### Group Policy Inheritance

### Group Policy Preferences

### Multiple Local Policies

### Controlling Device Installation

# DBAS1007 Data Fundamentals

Data is raw.  It is pure fact.  We require context to turn data into something.

Data = pure fact. Information = interpretation of fact. Knowledge = network of information. Insight = novel crosslinking point between information. Wisdom = connection of information.

## Databases

A Database Management System (DBMS) is a software application that interacts with end-users, application, and the database itself to capture and analyze data.  The primary role of a DBMS is to provide a systematic and efficient way to store, retrieve, and manage data in databases.  It ensures data integrity, security and consistency while supporting multiple users and applications simultaneously.  In IT infrastructure, a DBMS is crucial as it underpin the data architecture, enabling organizations to handle large volumes of data, perform complex queries, and generate insights that drive decision-making and operational efficiency.  By centralizing data management, a DBMS enhances data accessibility and reliability, which are essential for modern business operations and digital services.

SQL (sequel) is a Database Management System (DBMS) that is often used for banking and military purposes for its rigidity.

A database is a place where information is stored.  This allows the information to be easily found, used and updated. They are used in websites, apps, and businesses to manage everything from customer details to class schedules.

Databases come in various types, each suited to different applications.

Relational databases (MySQL, PostgreSQL) use structured tables and are ideal for transactional systems like banking and e-commerce.

NoSQL databases (MongoDB, Cassandra) handle unstructured data and are perfect for big data application and real-time analytics.

In-memory databases (Redis) store data in RAM (Random Access Memory) for fast access, making them suitable for caching and real-time applications.

Object-oriented databases (db40) store data as object, which is useful for applications requiring complex data representations.

Graph databases (Neo4) excel in managing interconnected data, making them ideal for social networks and recommendation systems.

Each type of database is designed to meet specific needs, ensuring efficient data management and retrieval for various use cases.

Database diagrams are visual representations of a database’s structure, showcasing tables, columns, relationships and constraints.  They help in understanding how data is organized and interconnected within the database.

Database objects include tables, views, indexes, stored procedures, and triggers, each serving a specific function in data management. Tables store data in rows and columns, views provide virtual tables for querying, indexes enhance search performance, stored procedures encapsulate business logic, and trigger automate actions based on events.

Together, database diagrams and objects facilitate efficient design, implementation and maintenance of databases, ensuring data integrity and accessibility.

## ACID Properties

Atomicity - All parts of a transaction must succeed or none-at-all. If something fails, the database goes back to its original state.

Consistency - A transaction must bring the database from one valid state to another, following all rules and constraints.

Isolation - Transaction happen independently.  One transaction doesn’t affect another while it’s running.

Durability - Once a transaction is complete, the changes are saved permanently, even if the system crashes.

### Best Practices

Best practices in structuring data components involve organizing data to ensure efficiency, scalability, and maintainability.  This includes normalizing data to eliminate redundancy defining clear primary and foreign keys (Keys are used to uniquely identify data) to establish relationships and using indexes to speed up query performance.

It’s essential to design consistent naming conventions for tables and columns, implement data integrity constraints to maintain accuracy and regularly backup and secure data to prevent loss and unauthorized access.

Additionally, documenting the database schema and changes helps in maintaining clarity and facilitating collaboration among developers and database administrators.  These practices collectively enhance reliability and performance of the database system.

### Normalization

Normalization is a process in database design aimed at organizing data to reduce redundancy and improve data integrity. It involves dividing a database into smaller, well-structured tables and defining relationships between them.

The primary goal I to ensure that each piece of data is stored only once, minimizing duplication and potential inconsistencies.  Normalization typically follows a series of steps, known as normal forms, each addressing specific types of redundancy and dependency.

By adhering to normalization principles, databases become more efficient, easier to maintain, and capable of supporting complex queries and updates without compromising data accuracy.

### Normal Forms

Normal forms are a series of guidelines used in database normalization to reduce redundancy to dependency by organizing fields and table relationships.

Each normal form addresses specific types of anomalies and inefficiencies.

The most commonly used normal forms are the First Normal Form (1NF), which ensures that all columns contain atomic, indivisible values; the Second Normal Form (2NF), which removes partial dependencies on a composite primary key; and the Third Normal Form (3NF), which eliminates transitive dependencies.

Higher normal forms, like Boyce-Codd Normal Form (BCNF), further refine these principles. Adhering to normal forms is crucial for maintaining data integrity, optimizing query performance and simplifying database maintenance.

### Normalization Steps

To go from First Normal Form (1NF) to Third Normal Form (3NF) in normalization, follow these steps:

- First Normal Form (1NF)

  - Ensure that all columns contain atomic, indivisible values

  - Remove any repeating groups of arrays, so each columns contain only one value per row.

- Second Normal Form (2NF)

  - Ensure the table is already in 1NF

  - Remove partial dependencies, where non-key attributes are dependent on pat of a composite primary key. This involves creating separate tables for sets of values that apply to multiple records.

- Third Normal Form (3NF)

  - Ensure the table is already in 2NF

  - Remove transitive dependencies, where non-key attributes depend on other non-key attributes.  This involves creating separate tables to ensure that non-key attributes depend only on the primary key.

By following these steps, you can systematically reduce redundancy and improve data integrity in your database.

The above is an example of an unnormalized table.

The hallmark of an unnormalized table is that there is repeating information as well as multiple data points in a row.  Take “Courses” for example; for Alice, it has “Math, Science” and for Bob it has “English, History, Math.”  Through Normalization we will get this broken down to be more concise,”

The first step to this is to re-write this as First Normal Form (1NF).

To do this, we need to identify the Primary Key.  This is the central data point that we are going to base our tables off.  For this example, we will be using “StudentID.”

Now that we have broken the original table down to First Normal Form (1NF) we can see that there is still some repeating information, however, “Courses” has been re-named to “Course” and no longer has multiple entries per row.

However, we can still break this down further, into Second Normal Form (2NF).

To go to Second Normal Form (2NF) we have split the table into two different tables, to reduce redundancy. Note that the “Students Table” contains unique information and the “Courses Table” lists the courses each student is enrolled in. This structure ensures that each non-key attribute is fully dependent on the Primary Key, satisfying the 2NF.

Now we have entered Third Normal Form (3NF). In this structure, the “StudentCourses Table” acts as a junction table, linking “Student ID” from “Students Table” to “CourseID” from the “Courses Table.” This setup is essential for accurately representing the enrollment of students in multiple courses and maintaining data integrity in a normalized database.

This structure is in Third Normal Form (3NF) because:

- Students Table

  - Contains only atomic values

  - Each non-key attribute (Name) I fully dependent on the Primary Key (StudentID)

- Course Table

  - Contains only atomic values.

  - Each non-key attribute (CourseName) is fully dependent on the Primary Key (CourseID)

- StudentCourses Table

  - Contains only atomic values

  - The composite Primary Key (StudentID, CourseID) ensures that each non-key attribute is fully dependent on the entire Primary Key.

  - There are no transitive dependencies, as the table only links StudentID to CourseID.

### ERDs and ERMs

In the database world, Entity Relationship Modeling (ERM) and Entity Relationship Diagrams (ERD) are closely related but serve different purposes.

### Entity Relationship Modeling (ERM)

In the context of Relational Database Management System (RDBMS), ERM stans for Entity Relationship Model. This model is a fundamental concept used in database design to visually represent the structure of data and the relationships between different data entities.

- Conceptual Framework: ERM is a high-level conceptual framework used to outline the entities relevant to a system and their interrelationships.

- Purpose: It focuses on the overall structure and organization of data, ensuring that all necessary components are identified before diving into specifics.

- Usage: ERM is typically utilize in the initial stages of database design to conceptualize the data structure.

Using an Entity Relationship Model (ERM) in database design offers several advantages.

- Clear Structure

  - ERM provides a clear and organized way to represent data and its relationship, making it easier to understand the database structure

- Improved Communication

  - ER diagrams server as a visual tool that helps stakeholders, including developers and business analysts, communicate effectively about the database design.

- Simplified Maintenance

  - A well-designed ERM makes it easier to maintain and update the database as changes in requirements or data structures occur.

- Enhanced Query Performance

  - Properly structure databases based on ERM can lead to optimized query performance, as relationships and data dependencies are clearly defined.

- Scalability

  - ERM facilitates the design of scalable databases that can grow and adapt to changing business needs without significant restructuring.

An ER model usually results from systematic analysis to define and describe the data created and needed by processes in a business area.  Typically, it represents records of entities and events monitored and directed by business processes, rather than processes themselves.  It is usually drawn in a graphical form as boxes (entities) that are connected by lined (relationships) which express the associations and dependencies between entities.  It can also be expressed I a verbal form, for example: one building may be divided into zero or more apartments, but one apartment can only be located in one building.

Entities may be defined not only by relationships, but also by additional properties (attributes), which include identifiers called “primary keys”.  Diagrams created to represent attributes as well as entities and relationships may be called entity-attribute-relationship diagrams, rather than entity-relationship models.

An ER model is typically implemented as a database.  In a simple relational database implementation, each row of a table requests one instance of an entity type, and each field in a table represents an attribute type. In a relational database, a relationship between entities is implemented by storing the primary key of one entity as a point or “foreign key” in the table of another entity.

There is a tradition for ER/data models to be built at two or three levels of abstraction.  The conceptual-logical-physical hierarchy below is used in other kinds of specification and is different from the three scheme approach to software engineering.

### Entity Relationship Diagrams (ERD)

- Visual Representation: ERD is a graphical depiction of the ERM, illustrating how entities interact with one another through relationships.

- Detail Level: ERDs include specific attribute and cardinalities, providing a detailed view of the data structure

- Usage: ERDs are employed for communication and documentation purposes, helping stakeholders understand the relationship between entities.

### Key Differences

- Purpose: ERM is used for conceptualizing the data structure, while ERD provides a graphical depiction of that structure.

- Detail Level: ERM outlines entities and relationships broadly, whereas ERD includes specific attributes and cardinalities.

- Usage ERM is often utilized in the initial stages of database design, while ERD is employed for communication and documentation

### Components

- Entities: Primary objects or concepts within the system, such as “Customer”, “Order”, or “Product.

- Relationships: Define how entities interact with one another, classified as one-to-one, one-to-many, or many-to-many.\

- Attributes: Properties or details that describe an entity, like “Name”, “Email”, and “Pone Number” for a “Customer” entity.

### Chen Notation

- Chen Notation is a way to visually represent entities and their relationships in an Entity Relationship Diagram (ERD). Named after Peter Chen, who introduced it in 1976, this notation uses specific symbols to illustrate the structures of a database.

- Key Elements of Chen Notation

  - Entities: Represented by rectangles, entities are objects or concepts that store data.

  - Attributes: Represented by ovals, attributes are properties or details about entities.

  - Relationships: Represented by diamonds, relationships show how entities are connected.

  - Primary Keys: Underlined attributes within an entity rectangle indicate primary keys.

- Chen Notation is particularly useful for conceptual database design, helping to clearly define the data structure and relationships before implementation.

## Three Types of Data Models

### Conceptual Data Model

- Purpose: Provides a high-level overview of the system, focusing on the entities and their relationships without detailing attributes or keys.

- Components: Entities and relationships

- Audience: Business stakeholders and analysts

- Example: An ER diagram showing entities like Customer, Order and Product with relationships between them.

### Logical Data Model

- Purpose: Adds more detail so the conceptual model by specifying attributes, primary keys, and foreign keys, but still independent of physical implementation

- Components: Entities, attributes, primary keys, foreign keys and relationships.

- Audience: Database designers and developers

- Example: An ER diagram showing Customer with attributes like CustomerID, Name and Address, Order with attributes like OrderID, OrderDate ad CustomerID

### Physical Data Model

- Purpose: Translates the logical model into a physical structure, detailing how data will be stored in the database, including tables, columns, data types, indexes, and constraints.

- Components: Tables, columns, data types, indexes, constraints and relationships.

- Audience: Database administrator and developers.

- Example: A database scheme showing the Customer table with columns CustomerID (INT), Name (VARCHAR), and Address (VARCHAR) and the Order table with columns OrderID (INT) OrderDate(DATE) and CustomerID (INT)

## SQL

SQL or Structured Query Language, was developed in the early 1970s by IBM researches Raymond Boyce and Donald Chamberlin.

It was initially called SEQUEL (Structured English Query Language) and was designed to manage and retrieve data stored in IBM’s original relational database management system, System R.

Over time, SQL evolved and became the standard language for relational database management systems, with its first commercial implementation appearing in the late 1970s.  The American Standards Institute (ANSI) adopted SQL as the standard in 1986, solidifying its role as a fundamental tool in database management and manipulation.

Today, SQL remains a critical skill for data professionals and is widely used across various industries.

### SQL Statements & Execution

A SQL statement is a command used to interact with a database, performing tasks like querying, updating, and managing data.

In MySQL Workbench, you can execute these statements individually by writing them in the query editor, highlighting the specific statement you want to run, and then clicking the “Execute” button or pressing Ctrl + Enter

This method allows for precise control and testing of each command, ensuring accurate database operations.

### SQL DDL

SQL DDL, or Data Definition Language, is a subset of SQL used to define and manage database structures.

It includes commands such as CREATE, ALTER, DROP and TRUNCATE, which are used to create, modify, delete and truncate database objects like tables, indexes and views.

Essentially, DDL commands help in setting up and organizing the schema of a database, ensuring that the data is stored in a structured and efficient manner.

### Logical Data Model

Data Types in a logical data model:

    - VARCHAR(n) – Variable-length string

    - INT / BIGINT – Whole numbers

    - DECIMAL(p, s) – Precise decimal values

    - DATE, TIME, DATETIME

    - BOOLEAN

    - TEXT, BLOB -  Large text or binary data

# WEBD1000 Website Development

## Web Development History

### Introduction to Web Development

- What is web development?

  - The process of creating websites and web applications for the internet

  - Involves coding, designing and maintaining websites.

- Importance of Understanding History

  - Knowing the evolution of web technologies helps in appreciating current standards and trends.

### The Birth of the Web (1989 – 1991)

- Tim Berners-Lee’s Vision

  - 1989: Proposed a project at CERN for a system of linked information.

  - 1990: Developed the first web browser (WorldWideWeb) and the first website (info.cern.ch

  - 1991: Public release of the World Wide Web, allowing people to create and share documents globally.

### The Rise of Web Browsers (1993 – 1995)

- Mosaic (1993)

  - The first widely used web browser with a graphical interface, making the web accessible to non-technical users.

- Netscape Navigator (1994)

  - Improved user experience with faster loading times and features like cookies.

  - Become the dominant browser, sparking the “Browser Wars.”

### The Browser Wars and Standards (1995 – 1998)

- Internet Explorer Vs. Netscape

  - Microsoft launched Internet Explorer in 1995, integrating it with Windows.

  - The competition le to rapid advancement but also inconsistencies in web standards.

- Formation of W3C (1994)

  - World Wide Web Consortium (W3C) established by Tim Berners-Lee to develop standardized protocols and guidelines.

### The Dot-Com Boom and Bust (1997 – 2001)

- Rapid Growth

  - Late 1990s.  Explosion of web Startups, e-commerce and online services.

  - Introduction of CSS (1996) for better design control and JavaScript (1995) for interactivity.

- The Dot-Com Bust (2000)

  - Overinvestment in internet companies led to a market crash, reshaping the industry towards sustainable business models.

### The Modern Web (2002 – Present)

- Web 2.0 and Beyond

  - Early 2000s: Rise of dynamic content, user-generated content and social media platforms.

  - HTMLS (2014) and CSS3 (2012): Introduced new features for multimedia, responsive design and more

- Mobile Revolution

  - 2010s: Shift towards mobile-first design as smartphones and tablets become prevalent.

  - Emphasis on responsive web design, accessibility and performance optimization.

### Introduction to Usability

- What is Usability?

  - Usability refers to how easy and efficient it is for users to navigate and interact with a website or application

- Importance of Usability

  - Enhances user experience, satisfaction, and accessibility.

  - Reduces errors and increases the effectiveness of the website.

- Core Components

  - Learnability, efficiency, memorability, errors and satisfaction.

### Principles of Good Usability

- Understand your Users

  - Identify target audience needs and preferences.

  - Conduct user research and testing.

- Consistency

  - Maintain consistency in design, navigation and interaction across the website.

- Feedback

  - Provide users with immediate and clear feedback on their actions. (Button clicks, form submissions, etc)

### Principle of Contrast

- What is Contrast?

  - Contrast is the difference in visual properties (color, brighten) that makes an object distinguishable from other objects and the background.

- Importance in Design

  - Helps draw attention to key elements.

  - Improves readability and accessibility.

- Examples

  - Text colour vs background colour (black text on white background)

  - Button colours that stand out from the page (A red “Submit” button on a white form)

### Applying Contrast Effectively

- Best Practices:

  - Use sufficient contrast between test and background to ensure readability.

  - Use contrast to highlight important elements (call-to-action buttons)

  - Avoid too many contrasting elements that can lead to visual clutter

- Tools

  - Contrast check tools (WebAIM Contrast Checker) to ensure compliance with accessibility standards.

### Principle of Repetition

- What is Repetition

  - Repetition involves reusing the same or similar elements throughout the design to create a sense of unity and consistency.

- Importance in Design

  - Reinforces brand identity and improves use recognition.

  - Enhances visual organization and flow

- Examples

  - Consistent use of fonts, colours and button styles across all pages.

  - Repeating logos or icons to maintain brand consistency.

### Applying Repetition Effectively

- Best Practices

  - Repeat navigation menus, headers and footers on every page for a cohesive look

  - Use consistent typography and colour schemes.

  - Ensure that repeated elements maintain the same meaning and function throughout the site.

### Principle of Alignment

- What is Alignment

  - Alignment refers to the placement of elements on a page in relation to each other, creating a visual connection between them

- Importance in Design

  - Creates a structured and organized layout

  - Guides the user’s eye and improves readability

- Examples

  - Text aligned to the let, center or right for clear reading paths.

  - Images and buttons aligned with text to create harmony.

### Applying Alignment Effectively

- Best Practices

  - Align elements to a grid the ensure consistency across the layout

  - Use alignment to create a clear hierarchy and flow of information.

  - Avoid arbitrary or mixed alignment, which can lead to a disorganized appearance.

### Principle of Proximity

- What is Proximity?

  - Proximity involves placing related elements close together to signify their connection and make the layout easier to scan.

- Importance in Design

  - Helps user quickly identify relationships between elements.

  - Reduces cognitive load by grouping similar information.

- Examples

  - Grouping form labels and input fields together

  - Placing related links or buttons near each other.

### Applying Proximity Effectively

- Best Practices

  - Group related items together (Contact information, navigation links)

  - Use white space strategically to separate unrelate elements)

  - Ensure that proximity reflects the logical relationships between elements

### What is Web Accessibility?

Consider out website visitors.  They may have different visual, auditory, physical, speech, cognitive and/or neurological abilities.

Definition

  - Web accessibility ensures that websites, tools and technologies are designed and developed so that people with disabilities can use them.

Scope

  - Includes visual, auditory, physical, speech, cognitive and neurological disabilities.

Core Principles

  - Making the web usable for everyone, regardless of ability.

### Importance of Accessibility

- Legal compliance

  - Adhering to laws such as Americans with Disabilities Act (ADA) and Web Content Accessibility Guideline (WCAG)b

- Ethical Responsibility

  - Ensuring equal access to information and services.

- Business Benefits

  - Reaching a broader audience and improving user experience for all users.

- SEO Benefits

  - Accessible websites often rank better in search engines due to clean, well-structured content.

### Understanding WCAG (Web Content Accessibility Guidelines)

- Introduction to WCAG

  - WCAG provides a set of guidelines to make web content more accessible

- WCAG Principles

  - Perceivable: Information must be presented in ways that all users can perceive.

  - Operable: Interface components and navigation must be operable.

  - Understandable: Information and the operation of the interface must be understandable.

  - Robust: Content must be robust enough to be interpreted by a wide variety of user agents, including assistive technologies.

### Accessible Design Components: Perceivable Content

- Text Alternatives

  - Provide text alternatives (ALT attributes) for non-text content such as images.

- Captions and Transcripts

  - Include captions for videos and transcripts for audio content.

- Colour Contrast

  - Ensure sufficient colour contrast between text and background for readability.

- Text Resizing

  - Allowing users to resize text without losing content of functionality.

### Accessible Design Components: Operable Interface

- Keyboard Accessibility

  - Ensure all functionalities are accessible via keyboards (navigation, forms)

- Focus Indicators

  - Provide visible focus indicators to guide users through interactive elements.

- Navigable Content

  - Use logical and clear navigation structures, such as heading and landmark.

- Time-based Media Controls

  - Provide users with control to pause, stop or adjust time-based media.

### Accessible Design Components: Understandable Content

- Clear and Simple Language

  - Write content in a clear, simple and concise manner.

- Consistent Navigation

  - Ensure navigation is consistent across pages to avoid confusion.

- Error Identification and Suggestion

  - Provide helpful error messages and suggestions for fixing errors in forms or other interactive elements.

- Readable Fonts

  - Use fonts that are easy to read and avoid overly decorative typefaces

### Accessible Design Components: Robust Content

- Semantic HTML

  - Use HTML elements according to their intended purpose (<header>, <nav>, <main>

- ARIA Landmarks

  - Implement ARIA (Accessible Rich Internet Applications) roles to enhance accessibility for assistive technologies.

- Cross-Browser and Cross-Device Compatibility

  - Ensure your website functions correctly across various browsers and devices.

- Support for Assistive Technologies

  - Ensure compatibility with screen readers, magnifiers and other assistive technologies.

### Tools and Resources for Testing Accessibility

- Automated Tools

  - Wave: Web Accessibility Evaluation Tool for checking accessibility issues.

  - AXE: Accessibility testing tool that integrates with browsers

  - Lighthouse: Built-in Chrome tool for auditing accessibility

- Manual Testing:

  - Keyboard navigation testing.

  - Screen reader testing using tools like NVDA or VoiceOver

- Online Resources

  - WCAD 2.1 Guidelines and tutorial available on W3C website.

  - Accessible design examples and case studies.

## Planning and Designing Websites

### Understanding Project Requirements

What are Project Requirements?

  - Specific goals, tasks and functionalities that a website must achieve.

Types of Requirements

  - Functional requirements: What the website should do (e.g., user login, search functionality.

  - Non-functional Requirements: How the website should perform (e.g., speed, scalability, security.)

Importance

  - Clear requirements ensure that the project meets the client’s or stakeholder’s needs and expectations

### Gathering Project Requirements

Stakeholder Interviews

  - Conduct interviews with clients, users and other stakeholders to understand their needs.

User stories

  - Develop user stories to capture what different types of users need from the website

  - Example: “As a [user type], I want to [action] so that [goal]

Surveys and Research

  - Use surveys and research to gather data on user needs and market trends.

### Analyzing Content Requirements

What is contact analysis?

    - The process of identifying and organizing the content that needs to be included on the website

Types of Content

    - Text: Articles, blog posts, product descriptions

    - Media: Images, videos, infographics

    - Interactive Elements:  Forms, calculators, maps

Content Strategy

    - Develop a strategy for organizing and delivering content that aligns with user needs and business goals.

### Designing Based on Criteria

Design Criteria

  - User Experience (UX): How easy and enjoyable it is for users to interact with the site

  - Visual Design: Aesthetic aspects like color schemes, typography and layout

  - Technical Criteria: Performance, compatibility and accessibility

Setting Priorities

  - Rank design criteria based on product goals, user needs and technical constraints.

### Translating Requirements into Design

Wireframes and Mockups

    - Create wireframes to visual the structure of the website

    - Develop mockups to show the visual design and layout

Information Architecture (IA)

    - Organize content into a logical structure that’s easy for users to navigate

Navigation Design

    - Plan the navigation to ensure that users can find what they’re looking for quickly and easily

### Designing for Content First

Content-First Design Approach

    - Start by understanding and organizing content before focusing on design elements

Benefits

    - Ensures that design supports content, rather than content being forced to fit the design

Practical Steps

    - Identify core content elements

    - Design layouts that prioritize and highlight important content.

### Balancing Aesthetics and Functionality

Aesthetic Design

    - Focus on creating  visually appealing website that align with the brand’s identity

Functional Design

    - Ensure that the website is usable, accessible and provides a seamless experience across all devices.

Achieving Balance

    - Use visual design to enhance, not overpower, functionality.

    - Test design to ensure they meet both aesthetics and functional criteria

### Importance of Wireframes in Web Design

Planning and Communications

    - Provides a visual representation of the website’s structure, helping communicate ideas clearly to stakeholders.

User Experience (UX)

    - Ensures that the user journey is intuitive and efficient before moving on to a detailed design.

Cost and Time Efficiency

    - Identifies potential design issues early in the process, saving time and resources.

### Wireframing Process

Step 1 – Understanding Requirements

    - Review project requirements and user needs

Step 2 – Sketching Basic Layouts

    - Start with low-fidelity sketches to outline the general layout and structure

Step 3 – Refining the Wireframe

    - Transition to high-fidelity wireframes, adding more detail and refining the layout

Step 4 – Feedback and Iterations

    - Share wireframes with stakeholders for feedback and make necessary adjustments

### Key Components of a Wireframe

Navigation

    - Placement of menus, links and buttons for easy site navigation

Content Areas

    - Designated spaces for text, images, videos and other content types

Call-to-Action (CTA)

    - Highlighted buttons or links that guide users to take specific actions

Footer

    - Contains secondary navigation, contact information and other utilities.

### Tools for Wireframing

Popular Wireframing Tools

    - Balsamiq: Known for its simplicity and low-fidelity wireframes

    - Figma: Offers both wireframing and design capabilities with collaborative features

    - Adobe XD: Provides high-fidelity wireframing and prototyping

    - Sketch: Favored for high-fidelity designs and a wide range of plugins

Choosing the Right Tool

    - Consider the complexity of the project, team collaboration needs, and integration with other design tools

### Creating Web Structure Diagram

What are Web Structure Diagrams

    - Visual representations of the website’s information architecture, showing how pages and organized and linked together

Purpose

    - Helps in planning the overall structure of the website, ensuring logical flow and easy navigation

Types of Diagrams

    - Site Map: High-level overview showing the main pages and their relationships

    - Flowchart: Detailed diagram showing the user’s journey through different pages

### Steps to Create a Web Structure Diagram

Step 1: Identify Key Pages

    - Determine the main pages of the website, such as Home, About, Services and Contact

Step 2: Define Page Hierarchy

    - Organize pages into a hierarchical structure, showing parent-child relationships

Step 3: Map Out Navigation Paths

    - Illustrate how users will navigate between pages, ensuring logical and intuitive flow

Step 4: Review and Refine

    - Collaborate with stakeholder to review the structure and make necessary adjustments.

### Best Practices or Wireframing and Structure Diagrams

Keep it Simple

    - Focus on layout and functionality, avoiding distractions from design details

User-Centered Design

    - Always consider the user’s needs and how they will interact with the site

Iterate Based on Feedback

    - Regularly review and update wireframes and diagrams based on user testing and stakeholder feedback

Consistency

    - Maintain consistency in layout, navigation, and interaction patterns across the site.

## HTML

### Understanding HTML Tags an Attributes

  - HTML Tags

    - Tags define the structure and content of the web page.

    - Common tags: <h1> to <h6> for headings. <p> for paragraphs. <div> for divisions, and <span> for inline elements

  - Attributes

    - Provide additional information about an element (e.g, class, id, style)

    - Example: <img src=”image.jpg” alt=”A description of the image”>

  - Self-closing Tags

    - Tags that don’t have a closing part (e.g., <img/>, >br />

Introduction to Semantic HTML Tags

  - What are semantic Tags?

    - Semantic tags clearly describe their meaning both to the browser and the developer

  - Importance

    - Improves code readability and accessibility

    - Helps search engines and assistive technologies understand the content structure

  - Common Semantic Tags

    - <header>: Represents the header of a document or section

    - <nav>:  Defines navigation block

    - <main>: Represents the main content of the document

    - <article>: Defines an independent, self-contained content

    - <footer>: Represents the footer of a document or section

### Understanding URLS

- What is a URL?

  - URL stands for Uniform Resource Locator, which is the address used to access resources on the web

- Components of a URL

  - Scheme – The protocol (e.g. http, https)

  - Host – The domain name (e.g, www.example.com)

  - Path  - The specific resource on the server (e.g., /about.html)

  - Query String – Optional parameters for data (e.g., ?id=123).

- Example

  - https://www.example.com/about.html?id-123

### Creating Hyperlinks with HTML

- What is a Hyperlink?

  - A hyperlink is an element in HTML that allows users to click and navigate to another page or resource.

- Basic Syntax

  - Use the <a> (anchor) tag to create hyperlinks.

  - Example

    - <a href=”https://www.example.com”> Visit Example </a?

- Attributes

  - href: Specifies the destination URL

  - target: Defines where to open the linked document (e.g., _blank opens a new tab).

### Absolute Vs. Relative URLS

- Absolute URLS

  - Includes the full path to the resource, starting from the protocol (e.g., https://www.example.com/page.html)

  - Use case: Linking to externa sites or resources

- Relative URLS

  - Path relative to the current document (e.g., about.html, ../images/photo.jpg)

  - Use case: Linking to internal pages within the same website.

- Example Comparisons

  - Absolute:

    - <a href=”https://www.example.com/contact.html”>Contact us</a

  - Relative:

    - <a href=”contact.html”>contact us</a?

### Adding Images to Web Pages

- Basic Syntax

  - Use the <img> tag to include images to HTML

  - Example

    - <img src=”images/logo.png” alt=”Company logo”>

- Attributes

  - src: Specifies the path to the image file

  - alt: Provides alternative text for accessibility and SEO

  - width and height: Specify the dimensions of the image.

- Best Practices

  - Always include the alt attribute for better accessibility.

  - Optimize images for web to reduce load times.

### Using Special Characters in HTML

- What are Special Characters?

  - Special characters are symbols that have specific meaning sin HTML (e.g., <, >, &) and must be encoded to be displayed correctly.

- Common Special Characters

  - &lt – For < (less than sign)

  - &gt – For > (greater than sign)

  - &amp – For & (ampersand)

  - &copy – For copyright symbol

- Example Usage

  - Displaying code:

    - &lt; div&t Hello Worldd&lt;/div&gt

  - Special symbols

    - &copy; 2024 My Website

### Introduction to CSS (Cascading Style Sheets)

- What is CSS?

  - CSS is used to style and layout webpages including changing colours, fonts and spacing.

- CSS Syntax

  - Selector: Defines which HTML elements the styles apply to.

  - Property: The style attribute you want to change

  - Value: The specific setting for the property

## Responsive Web Design

### What is Responsive Web Design?

  - Definition – Responsive Web Design (RWD) is an approach to web design that ensures websites look good and function well on devices of all sizes, from desktop to mobile phones

  - Key Characteristics  - Flexible layouts that adapt to different screen sizes.

  - Goal – To provide an optimal user experience across a wide range of devices

### Why Responsive Design is Important.

  - User Experience

    - Ensures that users have a seamless experience

    - Reduces the need for zooming, panning and scrolling

  - SEO Benefits

    - Google favors mobile-friendly websites, improving search rankings.

  - Cost Efficiency

    - A single responsive website reduces the need for separate mobile and desktop versions

  - Accessibility

    - Makes content accessible to a broader audience including those using assistive technologies.

## Introduction to Media Queries

### What are Media Queries?

  - CSS technique used to apply styles base on the characteristics of the user’s device, such as screen with, height and orientation.

  - Basic Syntax

    - Example:

@media (max-width: 768pcs)

{

Body

{

Font-size: 14px;

}

}

  - Breakpoints

    - Specific screen widths where the layout of the website changes.

      - Example breakpoints; 320px, 768px, 1024px

  - Usage

    - Apply different styles for mobile, tablet and desktop views.

### Creating Fluid Layouts

What are Fluid Layouts?

  - Layouts that use percentage-based widths rather than fixe pixel sizes, allowing elements to resize relative to the screen size.

Benefits

  - More flexible than fixe layouts.

  - Automatically adapts to different screen sizes without the need for horizontal scrolling.

Example of a Fluid Layout:

.container

{

width: 80%;

margin: 0 auto;

}

Img

{

max-width: 100%;

height: auto;

}

Practical Tips:

  - Use percentage-based widths for containers.

  - Ensure images and videos are responsive by setting max-width to 100%

### Using Media Queries for Responsive Design

You can target difference devices based on the sizes

@media (min-width: 1200px)

{

.container

{

width: 1170px;

}

}

@media (max-width: 768px)

{

.container

{

width: 100%

padding: 0 15px;

}

}

Common Breakpoints;

  - Mobile: max-width 480px

  - Tablet: 481px – 768px

  - Desktop: 769px and above

### Responsive Images and Media

Making Images Responsive:

  - Use the max-width: 100%; property to ensure images resize within their containers.

Img

{

max-width: 100%

height: auto

}

Wrap videos in a container and use padding techniques to maintain aspect ratios

  - Example

.video-container

{

position: relative;

padding-bottom: 56.25%; /* 16:9 aspect ratio */

height: 0;

overflow: hidden;

}

.video-container video

{

position: absolute;

top: 0;

left: 0;

width: 100%;

height: 100%;

}

    - The .video-container div is given a padding-bottom of 56.25%, which corresponds to a 16:9 aspect ration (you can adjust this value for different aspect ratios).  The height is set to 0 to ensure the padding creates the height.  The video element inside the container is positioned absolutely to fill the container

## Fluid Typography

- What is Fluid Typography

  - Adjust font sizes relative to the viewport width to ensure readability on all devices

- Using vw Units

  - Vw (viewport width) units allow text to scale based of the width of the viewport

- Example

h1

{

font-size: 5vw;

}

### Responsive Font Sizing

You can combine vw unites with media queries for more control

@media (max-width: 768px)

{

h1

{

font-size: 6vw;

}

}

@media (min-width: 769px)

{

h1

{

font-size: 3vw;

}

}

### Practical Example: Building a Responsive Layout

Creating a simple responsive web page with a fluid layout and media queries.

HTML

<div class=”container”>

<header>

<h1>Responsive Design</h1>

</header>

<section>

<p>This layout adjusts to different screen sizes.</p>

</section>

</div>

CSS

.container

{

width: 80%;

margin: 0 auto;

}

@media (max-width: 768px)

{

.container

{

width: 100%;

padding: 0 10px;

}

}

## Introduction to Flexbox

What is Flexbox?  Flexbox is a one-dimensional layout method that allows you to distribute space along a single axis (horizontal or vertical).  It is ideal for creating flexible, responsive layouts.

- Flex Container

  - The parent element where Flexbox is applied.

- Flex Items

  - The child elements inside the flex container

- Main Axis

  - The primary axis along which the flex items are laid out

- Cross Axis

  - The axis perpendicular to the main axis.

Basic Structure

### Creating Layout with Flexbox

- Use justify-content to align items along the main axis

- Use align-items to align items along the cross axis.

- Use flex-grow, flex-shrink, and flex-basis to control how items grow, shrink or maintain a base size.

### Using Float for Layouts

What is Float? The float property is used to wrap text around an image or to create simple layouts.

How does it work?  Elements can be “floated” to the let or right, allowing text or other elements to flow around them.

## Frameworks

# Summaries

# PROG1700 Logic and Programming

# NETW1027 Introduction to Networking and Security

### Networking Summary 1.0

- Through the use of networks, we are connected link never before.

- All computers that are connected to a network and participate directly in network communications are classified as hosts.

- Diagrams of networks often use symbols to represent the different devices and connections that make up a network.

- A diagram provides an easy way to understand how devices connect in a large network.

- The two types of network infrastructure are Local Area Network (LANs) and Wide Area Networks (WANs).

- SOHO (Small Office, Home Office) internet connections include cable, cellular,  satellite, and dial-up telephone.

- Business internet connections include Dedicated Leased Line, Metro Ethernet, Business DSL and Satellite.

- Network architecture refers to the technologies that support the infrastructure and the programme services and rules, or protocols, that move data across the network.

- There are four basic characteristics of network architecture: Fault tolerance, Scalability,  Quality of Service (QoS) and Security.

- Recent networking trends that affect organizations and consumers: Bring Your Own Device (BYOD), online collaboration, video communications and cloud computing.

- There are several common external and internal threats to networks.

- Larger networks and corporate networks use antivirus, antispyware and firewall filtering, but they also have other security requirements: Dedicated firewall systems, Access control lists (ACL), Intrusion Prevention System (IPS), and Virtual Private Network (VPN)

- The Cisco Certified Network Associate (CCNA) certification demonstrates your knowledge of foundational technologies.

### Networking Summary 2.0

- All end devices and network device require an operating system (OS)

- Cisco IOS software separates management access into the following two command modes: User EXEC Mode and  Privilege EXEC Mode.

- Global configurations mode is access before other specific configuration modes.  Rom global config mode, the user can enter different sub-configuration modes.

- Each IOS command has a specific format or syntax and can only be executed in the appropriate mode.

- Basic device configurations: hostname, password, encrypt passwords and banner.

- There are two system files that store the device configurations: startup-config and running-config

- IP addresses enable devices to locate one another and establish end-to-end communication on the internet.  Each end device on a network must be configured with an IP address.

### Networking Summary 3.0

- Rules

  - Protocols must have a sender and a receiver

  - Common computer protocol include these requirements: message encoding,  formatting and encapsulation, size, timing, and delivery options.

- Protocols

  - To send a message across the network requires the use o several protocols

  - Each network protocol has its own functions, format and rules for communications.

- Protocol Suites

  - A protocol suite is a group of inter-related protocols.

  - TCP/IP protocol suites are protocols use today.

- Standards Organizations

  - Open standards encourage interoperability, competition and innovation.

- Reference models

  - The two models used in networking are the TCP/IP and the OSI model.

  - The TCP/IP model has 4 Layers and the OSI model has 7 Layers

    - TCP/IP

      - Application

      - Transport

      - Internet

      - Network Access

    - OSI

      - Application

      - Presentation

      - Session

      - Transport

      - Network

      - Data Link

      - Physical

- Data Encapsulation

  - The form that a piece of data takes at any layer is called a protocol data unit (PDU).

  - There are five different PDUs used in the data encapsulation process: data, segment, packet, frame, and bits.

- Data Access

  - The Network and Data Link Layers are going to provide addressing to move data through the network

  - Layer 3 will provide IP addressing and Layer 2 will provide MAC addressing

  - The way these layers handle addressing will depend on whether the source and the destination are on the same network or if the destination is on a different network from the source.

### Networking Summary 4.0

- Before any network communications can occur, a physical connection to a local network, either wired or wireless, must be established.

- The Physical Layer consists of electronic circuitry, media and connectors developed by engineers.

- The Physical Layer standards address three function areas: physical components, encoding, and signaling.

- Three types of copper cabling are: UTP, STP, and coaxial cable (coax)

- UTP cabling conforms to the standards established jointly by the TIA/EIA.  The electrical characteristics of copper cabling are defined by the Institute of Electrical and Electronics Engineers (IEEE)

- The main cable types that are obtained by using specific wiring conventions are Ethernet Straight-through and Ethernet Crossover

- Optical fiber cable transmits data over longer distances and at higher bandwidths than any other networking media.

- There are four topes of fiber-optic connectors: ST, SC, LC and Duplex Multimode LC.

- Fiber-optic patch cords include SC-SC multimode, LC-LC single-mode, ST-LC multimode, and SC-ST single-mode

- Wireless media carry electromagnetic signals that represent the binary digits of data communications using radio or microwave frequencies.  Wireless does have some limitations, including coverage area, interference, security, and the problems that occur with any shared media.

- Wireless stands include the following: Wi-Fi (IEEE 802.11), Bluetooth (IEEE 802.15), WiMax (IEEE 802.16) and Zigbee (IEEE 802.15.4)

- Wireless LAN (WLAN)b requires a wireless AP and wireless NIC adapters.

### Number System Summary

- Binary is a base two numbering system that consists of the numbers 0 and 1, called bits.

- Decimal is a base ten numbering system that consists of the numbers 0 through 9

- Binary is what hosts, servers and networking equipment use to identify each other

- Hexadecimal is a base sixteen (16) numbering system that consists of the numbers 0 through 9 and the letters A to

- Hexadecimal is used to represent IPv6 address and MAC addresses.

- IPv6 addresses are 128 bits long and every 4 bits is represented by a hexadecimal digit or a total of 32 hexadecimal digits.

- To convert hexadecimal to decimal, you must first convert the hexadecimal to binary, then convert the binary to decimal.

- To convert decimal to hexadecimal, you must first convert the decimal to binary and then the binary to hexadecimal.

### Data Link Layer Summary

- The data link layer of the OSI Model (Layer 2) prepares network data for the physical network

- The data link layer is responsible for network interface card (NIC) to network interface card (NIC) communications.

- The IEEE 802 LAN/MAN data link layer consists of the following two sublayers: LLC and MAC.

- The two types topologies used in LAN and WAN networks are physical and logical.

- Three common types of physical WAN topologies are: Point-To-Point, hub and spoke and mesh.

- Half-duplex communications exchange data in one direction at a time.  Full-duplex sends and receives data simultaneously.

- In contention-based multi-access networks, all nodes are operating in half-duplex.

- The examples of contention-based access methods include: CSMA/CD for buss-topology Ethernet LANs and CSMA/CA for WLANS.

- The data link frame has three basic parts: header, data and trailer.

- The frame fields include: frame start and stop indicator flags, addressing, type, control, data, and error detection.

- Data link addresses are also know as physical addresses

- Data link address are only used for link local delivery frames.

### Ethernet Switching Summary

- Ethernet operates in the data link layer and the physical layer

- It is a family of networking technologies defined in the IEEE 802.2 and 802.3 standards.

- The 802 LAN/MAN standards, including Ethernet, use two separate sublayers of the data link layer to operate:

  - LLC Sublayer: (IEEE 802.2) Places information in the frame to identify which network layer protocol is used for the frame.

  - MAC Sublayer: (IEEE 802.2 802.11 or 802.15) Responsible for data encapsulation and media access control, and provides a data link layer addressing.

- The MAC sublayer is responsible for data encapsulation and accessing the media.

- IEEE 802.3 data encapsulation includes the following:

  - Ethernet frame – This is the internal structure of the Ethernet frame.

  - Ethernet addressing – The Ethernet frame includes both a source and destination MAC address to deliver the Ethernet frame from Ethernet NIC to Ethernet NIC on the same LAN

  - Ethernet error detection – The Ethernet frame includes a frame check sequence (FCS) trailer use for error detection.

- The IEEE 802.3 MAC sublayer includes the specifications for different Ethernet communication standards over various types of media, including copper an fiber.

- Legacy Ethernet using a bus topology or hubs is a shared, half-duplex medium uses a contention-base access method, carrier sense multiple access/collision detection (CSMA/CD).

- Ethernet LANs of today use switches that operate in full-duplex.  Full-duplex communications with Ethernet switches do not require access control through CSMA/CD.

- The minimum Ethernet frame size is 64 bytes and the maximum is 1518 bytes  The preamble field is not included when describing the size of the frame.

- Any frame less than 64 bytes in length is considered a “collision fragment” or “runt frame” and is  automatically discarded.  Frame with more than 1500 bytes of data are considered “jumbo” or “baby giant frames”.

- If the size of a transmitted frame is less than the minimum, or greater than the maximum, the receiving device drops the frame.  Dropped frames are likely to be the result of collisions or other unwanted signals.  They are considered invalid.  Jumbo frames are usually supported by most Fast Ethernet and Gigabit Ethernet switches and NICS.

- An Ethernet MAC address consists of a 48-bit binary value, expressed using 12 hexadecimal values.

- Given that 8 bits (one byte) is a common binary grouping, binary 00000000 to 11111111 can be represented in hexadecimal as the range 00 to FF.

- When using hexadecimal, leading zeroes are always displayed to complete the 8-bit representation.  For example the binary value 0000 1010 is represented in hexadecimal as 0A.

- Hexadecimal numbers are often represented by the value preceded by 0x (e.g., 0x73) to distinguish between decimal and hexadecimal values in documentation.

- Hexadecimal may also be represented by a subscript 16, or a hex number followed by an H (e.g., 73H)

- In Ethernet LAN, every network device is connected to the same, shared media.  MAC addressing provides a method for device identification at the data link layer of the OSI model.

- An Ethernet MAC address is a 48-bit address expressed using 12 hexadecimal digits.  Because a byte equals 8 bits, we can also say that a MAC address is 6 bytes in length.

- All AMC addresses must be unique to the Ethernet device or Ethernet interface.  TO ensure that, all vendors that sell Ethernet devices must register with the IEEE to obtain a unique 6 hexadecimal (i.e., 24-bit or 3-byte) code called the organizationally unique identifier (OUI).

- An Ethernet MAC address consists of a 6-hexadecimal vendor OUI code followed by a 6-hexadecimal vendor-assigned value.

- When a device is forwarding a message to an Ethernet network, the Ethernet header include a Source MAC address and a Destination MAC address.

- When a NIC receives an Ethernet frame, it examines the destination MAC address to see if it matches the physical MAC address that is stored in RAM.  If there is no match the device discards the frame  If there is a match it passes the frame up the OSI layers, where the de-encapsulation process takes place.

- Any device that is the source or destination of an Ethernet frame, will have an Ethernet NIC and therefore, a MAC address.  This includes workstations, servers, printers, mobile devices an routers.

- A unicast MAC address is the unique address that is used when a frame is sent from a single transmitting device to a single destination device.

- The process that a source host uses to determine the destination MAC address associate with an IPv4 address is known as Address Resolution Protocol (ARP).  The process that a source host uses to determine the destination MAC address associated with an IPv6 address is known as Neighbor Discovery (ND)

- An Ethernet broadcast frame is received and processed by every device on the Ethernet LAN.  The features of an Ethernet broadcast are as follows:

  - It has a destination MAC address of FF-FF-FF-FF-FF-FF in hexadecimal (48 ones in binary)

  - It is flooded out all Ethernet switch ports except the incoming port.  It is not forwarded by a router.

  - If the encapsulated data is in IPv4 broadcast packet, this means the packet contains a destination IPv4 address that has all ones (1s) in the host portion.  This numbering in the address means that all hosts on the local network (broadcast domain) will receive and process the packet.

- AN Ethernet multicast frame is received and processed by a group of devices that belong to the same multicast group

  - There is a destination MAC address of01-00-5E when the encapsulated data is an IPv4 multicast packet and a destination MAC address of 33-33 when the encapsulated data is an IPv6 multicast packet.

  - There are other reserved multicast destination MAC addresses or when the encapsulated data is not IP, such as Spanning Tree Protocol (STP).

  - It is flooded out all Ethernet switch ports except the incoming port, unless the switch is configured for multicast snooping  IT is not forwarded by a router, unless the router is configured to route multicast packets.

  - Because multicast addresses represent a group of addresses (sometimes called a host group), they can only be use as the destination of a packet  the source will always be a unicast address.

  - As with the unicast ad broadcast addresses, the multicast IP address requires a corresponding multicast MAC address.

- A Layer 2 Ethernet switch uses Layer 2 MAC addresses to make forwarding decisions.  It is completely unaware of the data (protocol) being carried in the data portion of the frame, such as an IPv4 packet, an ARP message, or an IPv6 ND packet.  The switch makes its forwarding decisions based solely on the Layer 2 Ethernet MAC addresses.

- An Ethernet switch examines its MAC address table to make forwarding decision for each frame, unlike legacy Ethernet hubs that repeat bits out all ports except the incoming port.

- When a switch is turned on, the MAC address is empty.

- Every frame that enters a switch is checked for new information.  It does this by examining the source MAC address o the frame and the port number where the frame entered the switch.  IF the source MAC address does not exist, it is added to the table along with the incoming port number.  If the source MAC address does exist, the switch updates the refresh timer for that entry.  By default, most Ethernet switches keep an entry in the table for 5 minutes.

- If the destination MAC address is a unicast address, the switch will look for a match between the destination MAC address of the frame and an entry in its MAC address table.  If the destination MAC address is in the table, it will forward the frame out the specified port.  IF the destination MAC address is not in the table, the switch will forward the frame out all ports except the incoming port.  This is called unknown unicast.

- As a switch receives frames from different devices, it can populate its MAC address table by examining the source MAC address of every frame.  When the MAC address table of the switch contains the destination MAC address , it can filter the frame and forward out a single port

- Store-and-forward switching – This frame forwarding method receives the entire frame and computes the CRC.  If the CRC is valid, the switch looks up the destination address, which determines the outgoing interface.  Then the frame is forwarded out of the correct port.

  - An advantage to store-and forward switching is that it determines if a frame has errors before propagating the frame.  When an error is detected in a frame, the switch discards the frame.  Discarding frames with errors reduces the amount of bandwidth consumed by corrupt data.

  - Store-and-forward switching is required for quality of service (QoS) analysis on converged networks where frame classification for traffic prioritization is necessary.  For example; voice over IP (VoIP) data streams need to have priority over web-browsing traffic.

- Cut through switching – This frame forwarding method forwards the frame before it is entirely received.  At a minimum, the destination address of the frame must be read before the frame can be forwarded.

  - In cut-through witching, the switch acts upon the data as soon as it is received, even if the transmission is not complete.  The switch buffers just enough of the frame to read the destination MAC address so that it can determine to which port it should forward out the data.  The switch does not perform any error checking on the frame.

  - There are two variants of cut-through switching:

    - Fast-forward switching – Offers the lowest level of latency by immediately forwarding a packet after reading the destination address. Because fast-forward switching starts forwarding before the entire packet has been received, there may be times when packets are relayed with errors.  The destination NIC discards the faulty packet upon receipt.  Fast-forward switching is the typical cut-through method of switching.

    - Fragment-free switching – A compromise between the high latency and high integrity of store-and-forward switching and the low latency and reduced integrity of fast-forward switching, the switch stores and performs an error check on the first 64 bytes of the frame before forwarding.  Because most network errors and collisions occur during the first 64 bytes, this ensures that a collision has not occurred before forwarding the frame.

- An Ethernet switch may use a buffering technique to store frames before forwarding them or when the destination port is busy because of congestion.

  - Port-based memory

    - Frames are stored in queues that are linked to specific incoming and outgoing ports.

    - A frame is transmitted to the outgoing port only when all the frames ahead in the queue have been successfully transmitted.

    - It is possible for a single frae to delay the transmission of all frames in memory because of a busy destination port.

    - It is possible for a single frame to delay the transmission of all the frames I memory because of a busy destination port.

    - This delay occurs even if the other frames could be transmitted to open destination ports.

  - Shared memory

    - Deposits all frames into a common memory buffer shared by all switch ports and the amount of buffer memory required by a port is dynamically allocated.

    - The frames in the buffer are dynamically linked to destination port enabling a packet to be receive on one port and then transmitted on another port, without moving it to a different queue.

- Shared memory buffering also results in larger frames that can be transmitted with fewer dropped frames.  This is important with asymmetric switching which allows or different data rates on different ports.  Therefore, more bandwidths can be dedicated to certain ports (e.g., server ports.)

- There are two types of duplex settings used for communications on Ethernet network

  - Full duplex – Both ends of the connection can send and receive simultaneously.

  - Half-duplex – only one end of the connection can send at a time.

- Autonegotiation is an optional function found on most Ethernet switches and NICs.  It enables two devices to automatically negotiate the best speed and duplex capabilities.

- Duplex mismatch is one of the most common causes of performance issues on 10/100 Mbps Ethernet links.  It occurs when one port on the link operates at half-duplex while the other port operates at full duplex.

- This can occur when one or both ports on a link are reset, and the autonegotiation process does not result in both link partners having the same configuration.

- It also can occur when users reconfigure one side of a link and forget to reconfigure the other.  Both sides of a link should have autonegotiation on, or both sides should have it off.  Best practice is to configure both Ethernet switch ports as full-duplex.

- Most switching now support the automatic medium-dependant interface crossover (auto-MDIX) feature.  When enabled, the switch automatically detects the type of cable attached to the port and configures the interfaces accordingly.

- The auto-MDIX feature is enabled by default on switches running Cisco IOS Release 12.2.(18) SE or later.  However, the feature could be disabled.

## Network Layer Summary

- Best Effort –  Delivery method that does not guarantee that the packet will be delivered fully without errors.

- Fragmentation – Term for splitting up an IP packet when forwarding it from one medium to another medium with a smaller Maximum Transmission Unit (MTU)

- Data Link Layer – Layer responsible for taking an IP packet ad preparing it for transmission over the communications medium.

- Transport Layer – OSI layer that sends segments to be encapsulated in an IPv4 or IPv6 packet.

- Destination IP address and Source IP address – Are the two most commonly referenced fields in an IPv4 packet header that indicate where the packet is coming from and where it is going.

- Regarding IPv4 packet header fields – The source and destination IPv4 addresses remain the same while travelling from source to destination.

- Header Checksum – Is the field used to detect corruption in the IPv4 header.

- Protocol – The field includes common values such as ICMP (1), TCP (6), and UDP (17).

- IP address depletion, Increased network complexity and internet routing table expansion, lack of end-to-end connectivity – These are the three major issues associate with IPv4.

- Increased the IP address space and it uses a simpler header to provide improved packet handling – These are improvements provided by IPv6 compared to IPv4

- IPv6 headers consist of 40 octets an contains 8 header fields.

- The Hop Limit field replaces the IPv4 Time to Live field.

- Local hosts can reach each other without the need of  router.

- The default gateway address is the IP address of the router on the local network.

- Netstat -r and route print – Commands that can be entered on a Windows host to view its IPv4 and IPv6 table.

- It is true that a router can be configured with a combination o both static routes and a dynamic routing protocol.

- An “O” next to a route in a routing table indicates that the route learned dynamically from OSPF

- Static routes are manually configured

- Default route is known as a “gateway of last resort.”

- Show ip route used on a Cisco IOS router will show the routing table.

## Address Resolution Summary

- There are two primary addresses assigned to a device on an Ethernet LAN: Layer 2 physical address (the MAC address) and Layer 3 logical address (the IP address).

- ARP (Address Resolution Protocol) provides to basic functions: Resolving IPv4 addresses to MAC addresses and maintaining an ARP table of IPv4 to MAC address mappings.

- To send a frame, a device will search its ARP (Address Resolution Protocol) table for a destination IPv4 address and a corresponding MAC address

  - If the packet’s destination IPv4 address is on the same network, the device will search the ARP (Address Resolution Protocol) table for the destination IPv4 address

  - If the destination IPv4 address is on a different network, the device will search the ARP (Address Resolution Protocol) table for the IPv4 address o the default gateway.

  - If the device locates the IPv4 address, its corresponding MAC address is used as the destination MAC address in the frame.

  - If there is no ARP (Address Resolution Protocol) table entry found, then the device sends an ARP request.

- Entries in the ARP (Address Resolution Protocol) table are not permanent and are removed when an ARP cache timer expires.

- The duration of the ARP (Address Resolution Protocol) cache timer differs depending on the operating system.

- ARP (Address Resolution Protocol) tables entries can also be removed manually by the administrator.

- The show ip arp command displays the ARP table on a Cisco router.

- The arp -a command displays the ARP table on a Windows PC.

- ARP (Address Resolution Protocol) requests are received and processed by every device on the local network.

- Excessive ARP (Address Resolution Protocol) broadcasts can cause some reduction in performance.

- ARP (Address Resolution Protocol) replies can be spoofed by a threat actor to perform an ARP poisoning attack.

- Enterprise level switches include mitigation techniques to protect against ARP attacks.

- IPV6 Neighbor Discovery (ND) protocol provides:

  - Address resolution

  - Router discovery

  - Redirection services

  - ICMPv6 Router Solicitation (RS) and Router Advertisement (RA) messages are used for messaging between devices and routers for router discovery.

  - ICMPv6 redirect messages are used by routers for better next-hop selection.

- IPv6 devices use ND (Neighbor Discovery) to resolve the MAC address of a known IPv6 address.

- ICMPv6 Neighbor Solicitation messages are sent using special Ethernet and IPv6 multicast addresses.

- Layer 2 physical addresses are used to delivery the data link frame with the encapsulated IP packet from one NIC to another NIC on the same network.

- If the destination IP address is on the same network, the destination MAC address will be that of the destination device.

- When the destination IP address (IPv4 or IPv6) is on a remote network, the destination MAC address will be the address o the host default gateway

- An IPv4 device uses ARP to determine the destination MAC address of a loal device when it knows its OPv4 address.

## Basic Router Configuration Summary

- These tasks should be completed with configuring initial settings on a router

  - Configure the device name

    - (config)#Hostname <hostname>

  - Secure privileged EXEC mode

    - (config)#Enable secret <password>

  - Secure user EXEC mode

    - (config)#Line console 0

    - (config)#Password <password>

    - (config)#login

  - Secure remote Telnet/SSH access

    - (config-line)#Line vty 0 4

    - (config-line)#Password <password>

    - (config-line)#Login

    - (config-line)#Transport input { ssh | telnet }

  - Secure all passwords in the config file.

    - (config)#Service password-encryption

  - Provide legal notification

    - (config)#Banner motd <message>

  - Save the configuration

    - Router# copy running-config startup-config

- For routers to be reachable, the router interfaces must be configured

  - Using the no shutdown command activates the interface.  The interface must also be connected to another device, such as a switch or a router, for the physical layer to be active.  There are several commands that can be used to verify interface configuration:

    - show ip interface brief

    - show ipv6 interface brief

    - show ip route

    - show ipv6 route

    - show interfaces

    - show ip interface

    - show ipv6 interface

- For an end device to reach other networks, a default gateway must be configured.

  - The IP address of the host device and the router interface address must be in the same network

- A switch must have a default gateway address configured to remotely manage the switch fro another network.

  - To configure an IPv4 default gateway on a switch, use the ip default-gateway ip-address global configuration command.

# OSYS1200 Introduction to Windows Administration

# DBAS1007 Data Fundamentals

# WEBD1000 Website Development

# Glossary

# PROG1700 Logic and Programming

# NETW1027 Introduction to Networking and Security

# OSYS1200 Introduction to Windows Administration

# DBAS1007 Data Fundamentals

# WEBD1000 Website Development